SPY.DOC

SPY VS02.0 10/23/79

Aeronautical Research Associates of Princeton (A.R.A.P.)

Box 2229, Princeton, NJ 08540 (609) 452-2950

Author: J.D.  Leonard

SPY is an interactive display program  written  in  the

spirit of DEMO and similar to the display's available on the

VAX.  SPY will dynamically display timesharing task activity

on  an  IAS  system.  It was developed on IAS VS3.0, so some

changes may need to be made for IAS VS2.0.  SPY will run  on

terminals  which support direct cursor addressing.  Refer to

SPYBLD.DOC and SPYIN.FTN to build SPY for  various  terminal

types.   The default is VT52.  SPY uses about 5700 words for

a 32 terminal 8 batch stream system and about 5200 words for

a  16  terminal 4 batch stream system.  CPU requirements are

on the order of DEMO for similar update intervals.  SPY will

display  itself while running so you can see how much of the

system it is using.

SPY currently offers a choice of  2  displays.   Future

versions  will hopefully display information on disk activi-

ty.

Display #1 shows the  percent  of  CPU  time  spent  in

Batch,  Interactive  or terminals, Swap time, Null time, and

Real time or System time.  Percent, # of  ticks  accumulated

over  last  interval, and bar graphs showing the percentages

are updated at the specified interval.  The percentages  are

the same as those displayed in the new DECUS DEMO program.

Display  #2  shows  timesharing  job  activity   on   a

per-terminal  basis.  Each line shows the TI, username, jobs

running (up to 3), total ticks accumulated for  the  job(s),

ticks used since last display update, percent utilization of

elaplsed time, and a bar graph of the percentage  used.   Up

to 70% is shown on the graph.  The display is updated at the

specified interval, a'la DEMO.

The command line to initiate $$$SPY is as follows -

PDS> SPY[display #]  [terminal type]

where -

display # is either 1 or 2.  If not  supplied  the

default display (task build parm) will be run.

terminal type is a 4 character mneumonic  to  des-

cribe the type of terminal SPY will be running on.

This is to set the proper escape sequences if they

differ  from the default.  See SPYIN.FTN to define

terminal types other than the VT52.

e.g.  PDS> SPX1

PDS> SPX BEEHIVE

PDS> SPX2 ADM

SPY.DOC                           -2-

SPY VS02.0 10/23/79

The following commands are available while SPY is  run-

ning.  Input is processed as it is in DEMO.

Dn <cr>

where n = 1 changes to display 1 and

n = 2 changes to display 2

In <cr>

where n is the time  between  display  updates  in

seconds, like DEMO.

C <cr>          Clear screen and refresh display.

X <cr> or ^Z    Cause SPY to exit.

The following command descriptions pertain only  to  display

number 2.

Ln <cr>

where n is the number of lines to display, normal-

ly 24.  e.g.  may set to 14 if on VT100 in 132 col

mode.  The value of L is shown at  the  bottom  of

display 2 (L=n).

R range1,range2, ....rangen <cr>

Set the range of terminals to display where range1

through rangen can be any one of the following -

DEF - set range to default  setting  specified  at

task build time.

ALL - set range for all possible terminals to dis-

play.   Will  set  to  display MAXTT terminals and

MAXBA batch streams.  See SPYBLD.DOC and SPY.CMD.

TTn or BAn - for a single terminal where n is  the

octal unit number.

TTn-m or BAn-m - for an inclusive range of  termi-

nals where n,m are the octal unit numbers.  m must

be .GE.  to n.

Any combination of the above is  valid,  presuming

the  unit  numbers are within the upper limits set

by MAXTT and MAXBA.

e.g.  R TT1-6,BA0-2,TT15,TT20

R ALL

R DEF,BA5

SPY.DOC                      -3-

SPY VS02.0 10/23/79

** Notes on display 2 **

If the selected range encompasses more  terminals  than

there is room to display them, SPY will -

1) Try to display terminals with only users logged on,  if

this exceeds the display area then -

2) SPY will select those logged on with  jobs  running  or

suspended.   If  this  number  is less than the display

area, some terminals only logged on will also be shown,

to  fill  out  the display.  If the number of terminals

with jobs running is greater than the display area then

-

3) SPY will select the jobs with the largest percent  use-

age  and display only those jobs.  Setting L to a small

number will force this deduction to take place.

Also shown on display 2 is 'SF', the  swap  flag  indicator.

If this is '*' then one of the jobs running from that termi-

nal was swapped out when SPY 'looked' at it.   Time  of  day

and ticks elapsed since last update are displayed on the top

line.  On the bottom line is the sum of all timesharing jobs

CPU  useage divided by elapsed time to give percent utiliza-

tion.  UNS is the number of users (logged on) not  shown  if

they exceed the number of lines to display, and JNS which is

jobs not shown if there are some of those not displayed.