From the Wizzards Book of RSX MAGIC

D. B. Curtis

SCIENCE APPLICATIONS, INC.

Oak Brook, IL

This  is  the  second  RSX  MAGIC  article.   Due  to  my   time

limitations,  it  may  be the last unless the  wizards out there

pitch in and help.

This article describes processes that, if misused, may cause the

destruction  of your operating system.  These methods should not

be lightly used, they are provided to  help  solve  applications

where  normal  RSX  methods  will  not work.  As with all magic,

mistakes are dreadful!  (Remember the sorcerer's apprentice.)

The last article described a method of sending ASTs to specified

tasks.  This method was initially used in an error logging task.

That error logger task is presented in this article.  I have not

had  time  to  rework the code that sends ASTs so you get to see

the original item.

This task provides a  very  efficient,  general  purpose,  error

display  task.  There is only a six word memory overhead at each

error occurrence.   Essentially  this  is  done  by  dynamically

adding  an  error  directive  to  the  executive.  The technique

indicates  how  to  expand  the  functionality  of  the  RSX-11M

executive without modification to the executive source.

This particular version of the error task  was  written  for  an

RSX-11S  system.   Therefore,  I did not use disk resident error

messages.  However, it would be simple to add that capability.

An example of the use of the error processor is shown below:

MRKT$S  #1,#1,#1

BCC     1$

ERROR   MKTERR,-1,0,0

1$:

In this example you see  an  attempt  to  execute  a  mark  time

directive.   If  the  directive fails, you want an error message

and some attempt to recover from the error.  The recovery may be

as  simple  as  aborting  the  task.  The error macro definition

follows:

Page 2

.MACRO  ERROR,P1,P2,P3,P4

;

; MACRO TO SET UP INSTRUCTION BLOCK FOR ERROR MESSAGES

;

;       ERROR   KEY,TYPE,PRAM1,PRAM2

;

;               KEY <= 6 RAD50 CHARACTERS

;               TYPE = -1 FATAL ERROR

;                    = -2 FOR DELAYED RECOVERY

;                    = -3 FOR JUMP RECOVERY

;                    = 0 FOR MESSAGE ONLY

;       SEE DESCRIPTION IN ERRDIS MODULE

;

.WORD   210             ; ERROR HANDLING INSTRUCTION

.NCHR   TEMPN,P1

.IIF LT 6-TEMPN .ERROR ; RAD50 NAME > 6 CHARACTERS

.IF  LT 3-TEMPN

.RAD50  /P1/

.IFF

.RAD50  /P1/

.RAD50  /   /

.ENDC

.WORD   P2,P3,P4

.ENDM   ERROR

.MACRO  SAVRG

JSR     R5,$SAVRG

.ENDM

The error control block looks like:

-----------------------------------------

|     OP CODE (210)                     |

|---------------------------------------|

|     FIRST 3 CHAR. OF RAD50 KEY        |

|---------------------------------------|

|     SECOND 3 CHAR. OF RAD50 KEY       |

|---------------------------------------|

|     FIRST PARAMETER                   |

|---------------------------------------|

|     SECOND PARAMETER                  |

|---------------------------------------|

|     THIRD PARAMETER                   |

-----------------------------------------

where:

1.  The two word key is used to both identify the error, and  as

a  key  into an expanded message table.  If a match is found

in the message table, the message is printed in addition  to

the normal information.

Page 3

2.  The first parameter specifies the type of error recovery  to

perform.  This parameter takes the following values:

1.  0 -- Message only.  The task continues  execution  after

the error control block.  The second and third parameter

contain data to be printed.  If the second parameter  is

0,  R0 will be printed.  If the third parameter is 0, R5

will be printed.

2.  -1 -- Fatal error.  The  message  is  printed,  and  the

erroring   task   is  aborted.   The  second  and  third

parameters are treated as in the message only case.

3.  -2 -- Recoverable error.  The message is printed and the

task  continues  processing at the location specified in

the second parameter.   The  first  byte  of  the  third

parameter  is  the  number  of  seconds  to  wait before

continuing, and the last byte of the third parameter  is

the event flag to use while waiting.  If the seconds are

zero, it will default to 1, if the event flag  is  zero,

it will default to 1.

4.  -3 -- Recoverable error.  The message is printed and the

task  continues  processing at the location specified in

the second parameter.  If parameter 3 is 0, R5  will  be

printed.

5.  If some other value than  the  ones  listed  above,  the

error is printed, and the offending task is aborted.

For example, you  would  use  -2  for  an  error  that  required

operator intervention and -3 for an error exit from the routine.

The error printout has the following form:

'TSKNAM' PS = XXXXXX, PC = XXXXXX, DSW = XXXXXX

ERROR 'KEYNAM', PRAMS = XXXXXX XXXXXX

Where the XXXXXXs are  octal  numbers  for  the  values  of  the

erroring  task.   KEYNAM is the keyword for the error and TSKNAM

is the name of the task where the error occurred.

If a message is associated with the error, the message  line  is

printed  before  the  standard  display.   Again,  this could be

easily modified for your special needs.

1.0  DESCRIPTION OF THE CODE

This section provides an overview of the operation of the  error

logging task.

Page 4

The basic concept of the error logger is  to  allow  any  active

task  to  display  an error without extra code in the task.  The

error task is invoked by the  execution  of  a  PDP-11  reserved

instruction.   The  error  logging  task  intercepts  the system

illegal instruction processing and checks if the instruction  is

the   one   specific  to  the  error  logging  system.   If  the

instruction  is  not  the   error   logging   instruction,   the

instruction  is  handled  in  the  normal manner, otherwise, the

error logger is invoked.

To install the illegal instruction  intercept,  the  error  task

does  a  Connect-To-Interrupt directive to an unused vector.  At

this time, some PIC code is inserted into the system  pool,  and

the illegal instruction vector is pointed to the new code.  This

code  examines  the  illegal  instruction  and  if  it  is   the

instruction  specific to the error logger, will cause a transfer

to the error logger via the vector specified by the  connect  to

interrupt.   The code in the error logging task is simplified as

the task is built as /PR:5.

Information about the erroring task is gathered and  a  call  to

$FORK2  transfers the task from interrupt level to system state.

Here, information about the error is  placed  in  an  AST  block

which is sent to the error logging task to be processed.

2.0  DISCUSSION OF THE CODE

The  complete  listing  of  the  error  routine   follows   this

description.  Here I describe sections of the code.

2.1  Initialization And Self Test

The following is the initialization and self  test  sections  of

the  error  logging  task.  The task initializes itself and then

suspends.  If the task is resumed, it generates  a  sequence  of

test error messages.

Page 5

CINT:   CINT$   ERRVEC,120000,ERRINT,ERRINI,PR7,ASTAD

START:

DIR$    #CINT           ; CONNECT TO THE VECTOR,

BCC     10$             ; AND INIT IT, IF ERROR

IOT                     ; JUST CRASH

10$:    SPND$S                  ; ELSE SUSPEND FOREVER

;

; IF THIS TASK IS RESUMED, IT WILL SELF TEST

;

ERROR   AAAAA,0,1,2     ; PRINT ERROR MESSAGE WITH

;  DEFAULT PARAMETERS

MOV     #3,R0

MOV     #4,R5

ERROR   BBBBB,0,0,0     ; PRINT ERROR MESSAGE

BR      30$             ; NEXT MESSAGE SHOULD BE DDDDD

20$:    ERROR   CCCCC,-3,40$,6  ; NEXT MESSAGE WILL BE

;    A DELAY MESSAGE

30$:    ERROR   DDDDD,-3,20$,5  ; CCCCC IS NEXT MESSAGE

40$:    ERROR   EEEEE,-2,50$,<<15*400>+5> ; CHECK FOR 5 SECOND DELAY

50$:    ERROR   FFFFF,-2,60$,<<0*400>+0>  ; CHECK DEFAULT DELAY

;   AND EVENT FLAG

60$:    ERROR   DONE,-3,10$,0   ; ALL DONE

Note that the interrupt priority is 7 and that the base  address

of the connect code is at the start of the task (120000).

2.2  AST Handler

The next section of code is the AST routine  that  displays  the

errors.  This routine is easily modified for your special needs.

One nice enhancement would be to  send  the  erroring  tasks  TI

terminal  number to the AST routine so that the error would also

be displayed on that terminal.

Page 6

;

; INPUTS:

;       INFORMATION IS ON THE STACK, ALL REGISTERS MUST BE SAVED

;       THE INFORMATION  ON THE STACK IS:

;         2 WORDS OF RAD50 TASK NAME ISSUING THE INSTRUCTION

;         PS OF THAT TASK

;         PC OF THAT TASK

;         DSW OF THAT TASK

;         2 WORDS OF RAD50 ERROR KEYWORD

;         FIRST PARAMETER

;         SECOND PARAMETER

;

;

; EQUATED SYMBOLS

;

BEL = 7         ; ASCII CODE FOR BELL

.ASECT

. = 0

; STACK PARAMETERS

REGS:   .BLKW   6       ; R5-R0 R5 IS FIRST

ERTNM:  .BLKW   2       ; 2 WORDS OF RAD50 TASK NAME FROM ERROR

ETPS:   .BLKW   1       ; ERRORING TASKS PS

ETPC:   .BLKW   1       ; ERRORING TASKS PC

ETDSW:  .BLKW   1       ; ERRORING TASKS DSW

EKEY:   .BLKW   2       ; 2 RAD50 WORDS WITH KEY NAME

ETP1:   .BLKW   1       ; FIRST PARAMETER

ETP2:   .BLKW   1       ; SECOND PARAMETER

.PSECT

.ENABL  LSB

ASTAD:

.IRPC   REG,<012345>    ; SAVE THE REGISTERS

MOV     R'REG,-(SP)

.ENDM

;

; GET THE INFORMATION FROM THE INTERRUPT PART AND DISPLAY IT

;

MOV     SP,R0           ; GET THE ADDR OF THE ERROR DATA BLOCK

MOV     EKEY(R0),R1     ; GET THE FIRST WORD OF THE KEY

MOV     #MSGKYS,R2      ; AND THE ADDRESS OF THE KEYS

10$:    CMP     R1,(R2)         ; IS THE FIRST PART OF THE KEY THE SAME?

BNE     20$

; HERE THE FIRST PART OF THE KEY MATCHES

CMP     EKEY+2(R0),2(R2) ; DOES THE SECOND PART MATCH?

BNE     20$             ; IF NE NO, SO CONTINUE

BR      50$

20$:    ADD     #6,R2           ; ADVANCE TO NEXT KEY

TST     (R2)            ; CHECK NOT END OF KEYS

BNE     10$             ; KEY NOT FOUND, => NO MESSAGE OTHER

Page 7

;   OTHER THEN KEY ITSELF

;

; HERE, CONVERT KEY TO ASCII, AND PRINT IT AND OTHER VARIABLES

;

.PSECT  MESFMT,RO,D

.NLIST  BEX

MG1:    .ASCII  <BEL><BEL>/'%2R' PS = %P, PC = %P, DSW = %P%N/

.ASCIZ  /ERROR '%2R', PRAMS = %2P%N/

.LIST   BEX

.PSECT

30$:    MOV     SP,R2           ; SAVE POINTER TO PARAMETERS

ADD     #ERTNM,R2

SUB     #200.,SP        ; SAVE SPACE FOR LINE ON STACK

MOV     SP,R0

MOV     SP,R5

MOV     #MG1,R1         ; POINT TO FORMAT INFORMATION

CALL    $EDMSG          ; FORMAT IT

CMP     #200.,R1        ; CHECK STACK DIDN'T GET WIPED

BGE     40$

IOT

40$:    QIOW$S  #IO.WBT,#TTLUN,#TTEV,,,,<R5,R1,#40>

ADD     #200.,SP        ; CLEAN UP STACK

BR      60$

;

; HERE PRINT MESSAGE, AND VARIABLES

;

50$:

MOV     4(R2),R1        ; GET ADDRESS OF MESSAGE

MOVB    (R1)+,R2        ; GET SIZE OF MESSAGE

QIO$S   #IO.WBT,#TTLUN,,,,,<R1,R2,#40>

BR      30$

60$:    .IRPC   REG,<543210>    ; RESTORE THE REGISTERS

MOV     (SP)+,R'REG

.ENDM

ADD     #2*NUMPRM,SP    ; CLEAN OFF PARAMETERS

ASTX$S

.DSABL  LSB

There isn't much magical about this routine, it is just a normal

AST  receiver.   Well, almost normal as the number of parameters

on the stack isn't a standard number, but if you read  the  last

article you will not be surprised by that.

Page 8

2.3  Intercepting The Illegal Instruction Trap

The next  routine  is  called  when  the  connect  to  interrupt

directive   is   executed.   This  routine  is  responsible  for

inserting some code into the system pool space and  intercepting

the  illegal instruction trap.  The format of the intercept code

in the pool follows the form that was described by  a  paper  in

the  DECUS  proceedings<1>.  The error logger could just as well

be inserted into the system with the ISF task  as  described  in

that  paper.   However this task was developed for a stand alone

RSX-11S task and needed to insert the code itself.

------------------------------------

<1>  Extending RSX-11M by Use of the Illegal Instruction Trap DECUS

_________ _______ __ ___ __ ___ _______ ___________ ____

PROCEEDINGS  Vol 6,  No. 2,  Pg. 467.   This paper describes a

means of inserting code into the  system  pool.   It  is  good

background reading for parts of this article

Page 9

;+

;  **-ERRINI-CONNECT TO INTERRUPT ENABLE/DISABLE ROUTINE

;

; THIS ROUTINE ENABLES/DISABLES THE ILLEGAL INSTRUCTION INTERCEPT

; IF CARRY IS CLEAR, IT ENABLES, IF SET, IT DISABLES

; THIS MODULE ALLOCATES SOME POOL MEMORY, PLACES THE ILLEGAL INSTRUCTION

; INTERCEPT CODE INTO THE POOL AREA, MODIFIES XDT TO NOT PLAY WITH THE

; ILLEGAL INSTRUCTION TRAP VECTOR (ONLY IF XDT IS PRESENT) AND RETURNS.

;

; THE DISABLE OF THIS OPERATION UNDOES ALL OF THE ABOVE

;

; INPUTS:

;       CARRY SET TO DETACH FROM VECTOR, CLEAR TO ATTACH TO VECTOR

;

; OUTPUTS:

;       NONE

;

; SIDE EFFECTS:

;

;    MODIFIED EXTERNALS

;

;    OTHER SIDE EFFECTS

;       POLADD,POLSIZ ARE SET TO THE ADDRESS OF THE POOL AREA, AND ITS SIZE

;       THE ILLEGAL INSTRUCTION TRAP VECTOR IS INTERCEPTED.

;

; REGISTER USAGE:

;       R1,R4,R5 ARE SAVED

;

;-

ERRINI:

BCC     10$             ;; CONNECT TO ILLEGAL INSTRUCTION VEC

JMP     ABOPRO          ;; IF CARRY SET, DISCONNECT

10$:    MOV     R5,-(SP)        ;; SAVE REGISTERS

MOV     R4,-(SP)

MOV     R1,-(SP)

MOV     #EPOLCD-POOLCD,R1 ;; NEED TO ALLOCATE POOL SPACE TO PUT

CALL    $ALOCB          ;; TRAP INTERCEPT CODE

BCC     20$

IOT                     ;; IF NO POOL SPACE, CRASH SYSTEM

20$:    MOV     R0,POLADD       ;; SAVE ADDRESS OF ALLOCATED BLOCK

MOV     R1,POLSIZ       ;; AND SIZE OF ALLOCATED BLOCK

MOV     $TKTCB,MYTCBA   ;; AND WHILE AT IT, SET MY TCB ADDRESS

MOV     #<EPOLCD-POOLCD>/2,R2   ;; GET THE NUMBER OF BYTES TO COPY

MOV     #POOLCD,R3      ;; AND THE ADDRESS OF CODE TO MOVE

;; TOO THE POOL

30$:    MOV     (R3)+,(R0)+     ;; DO THE MOVE

SOB     R2,30$          ;; UNTIL FULLY COPIED

MOV     POLADD,R0       ;; GET THE START ADDRESS OF POOL CODE

ADD     #2,R0           ;; BUMP PAST THE "ILL"

MOV     POLSIZ,(R0)+    ;; SAVE THE NUMBER OF BYTES IN CODE

Page 10

MOV     @#ILLVEC,(R0)+  ;; SAVE THE ORIGINAL VECTOR

MOV     #NOP,(R0)       ;; TURN ON THE CODE

MOV     #$XDT,R2        ;; CHECK IF XDT IS AVAILABLE

BEQ     40$             ;; IF EQ NO

SUB     #XDTFIX,R2      ;; CHECK IF XDT IS CONSISTENT

CMP     @#ILLVEC,(R2)

BNE     40$

MOV     R0,(R2)         ;; ADJUST XDT

40$:    MOV     R0,@#ILLVEC     ;; POINT ILLEGAL INSTRUCTION TRAP TO

;; THIS NEW CODE

MOV     (SP)+,R1        ;; RESTORE REGISTERS

MOV     (SP)+,R4

MOV     (SP)+,R5

RETURN                  ;; AND RETURN TO TASK

This code inserts the PIC code that starts at  POOLCD  into  the

system  pool.  It modifies XDT to allow executive debugging with

this code inserted<2>.  The  code  that  is  inserted  into  the

system  pool  examines  the  instruction that caused the illegal

instruction trap.  If the instruction is recognised by the error

system,   this  code  simulates  an  interrupt  via  the  unused

interrupt vector that the error task connected  to.   Otherwise,

the  illegal  instruction  is  passed  to  the  original illegal

instruction trap handler.

------------------------------------

<2>  XDT resets the illegal instruction vector whenever it proceeds

from   a  breakpoint.   Thus  resetting  our  intercept.   The

modification  causes  XDT  to  bypass  resetting  the  illegal

instruction vector.

Page 11

;

; CODE THAT GETS LOADED INTO THE POOL

;  >>>>>> THIS CODE MUST BE PIC <<<<<

;

POOLCD:

.RAD50  /ILL/           ;;; SHOW THIS CODE IS ATTACHED TO THE

.WORD   0,0             ;;; ILLEGAL INSTRUCTION VECTOR

RTI                     ;;; INITIALLY, TURN OFF CODE

EP:     MOV     R5,-(SP)        ;;; SAVE R5

MOV     2(SP),R5        ;;; AND GET TASK PC FROM STACK

MFPI    -(R5)           ;;; GET THE INSTRUCTION THAT CAUSED THE TRAP

CMP     (SP),#210       ;;; IS IT OUR INSTRUCTION?

BEQ     100$

ADD     #2,SP           ;;; NO! SO CLEAR THE STACK

MOV     (SP)+,R5        ;;; RESTORE R5

JMP     @EP-4           ;;; AND PROCESS NORMALLY

;

; HERE IT IS OUR ILLEGAL INSTRUCTION

;

100$:   ADD     #2,SP           ;;; CLEAN THE STACK

MOV     (SP)+,R5        ;;; RESTORE R5

MOV     @#ERRVEC,-(SP)  ;;; AND JUMP VIA THE VECTOR WE ARE CONNECTED TO

JMP     @(SP)+          ;;; POSITION INDEPENDENT INDIRECT JUMP

EPOLCD:                         ;;; END OF THE CODE INSERTED INTO THE POOL

The illegal  instruction  trap  vector  points  to  EP-2.   EP-4

contains the address of the original service routine.

If the error logging task exits, the modifications to  XDT  must

be removed along with the intercept code from the system pool.

Page 12

ABOPRO:

MOV     R1,-(SP)        ;; SAVE ITB POINTER

MOV     @#ILLVEC,R0     ;; GET CURRENT VECTOR TO FIND OUR CODE

MOV     -2(R0),@#ILLVEC ;; RESET THE ILLEGAL INSTRUCTION VEC

MOV     #$XDT,R2        ;; RESET XDT

BEQ     1$

SUB     #XDTFIX,R2

MOV     -2(R0),(R2)

1$:     MOV     POLADD,R0       ;; GET THE ADDRESS OF THE POOL CODE

MOV     POLSIZ,R1       ;; AND THE SIZE

CALL    $DEACB

MOV     (SP)+,R1        ;; RESTORE ITB POINTER

RETURN

The next routine contains a large amount of  magic.   It  copies

the  error  parameter block from the user task and, if required,

adjusts the  issuing  tasks  program  counter.   After  entering

system  state  by a call to $FORK2, other actions on the issuing

task are performed.  These  actions  are  aborting  or  delaying

execution  of  the  issuing task.  Finally an AST is sent to the

error logging task.

Page 13

;+

;  **-ERRINT-PROCESSES ERROR INTERRUPTS

;

; THIS MODULE RECEIVES THE 210 INSTRUCTION CODE.

; IT SAVES THE ERROR BLOCK PARAMETERS, DOES ANY PROCESSING OF THE

; PARAMETERS (CHECKS WHAT TO DO WITH THE ISSUING TASKS PC).

; FORKS AND PLACES THE TASK IN THE APPROPRIATE STATE

;

; INPUTS:

;       R5 POINTS TO THE ITB

;       R4 IS SAVED AND MAY BE USED

;

; OUTPUTS:

;       NONE

;

; SIDE EFFECTS:

;

;    MODIFIED EXTERNALS

;

;    OTHER SIDE EFFECTS

;       MANY, TASK MAY BE ABORTED, MADE TO MARK TIME, TASKS PC MAY BE

;       MOVED AS DIRECTED BY THE PARAMETERS ETC.

;

; REGISTER USAGE:

;

;       R5 POINTS TO ITB

;       R4 IS SAVED AND MAY BE USED, ALL OTHER REGISTERS MUST BE SAVED AND

;       RESTORED

;

;-

;

; LOCAL DATA

;

ERL1:   .WORD   0

ERL2:   .WORD   0

PER1:   .WORD   0

PER2:   .WORD   0

PER3:   .WORD   0

ERRINT:                         ;;; HERE IF ILLEGAL INSTRUCTION

;;; FIRST 2 WORDS ARE RAD50 REP

;;;   OF NAME OF ERROR

MOV     10(SP),R4       ;;; GET PC OF TASK

MFPI    (R4)+           ;;; GET FIRST WORD OF RAD50 ERROR

MOV     (SP)+,ERL1      ;;;

MFPI    (R4)+           ;;;

MOV     (SP)+,ERL2      ;;;

MFPI    (R4)+           ;;;

MOV     (SP)+,PER1      ;;;

MFPI    (R4)+           ;;;

MOV     (SP)+,PER2      ;;;

MFPI    (R4)+           ;;;

MOV     (SP)+,PER3      ;;;

Page 14

TST     PER1            ;;; CHECK IF MESSAGE ONLY

BMI     20$             ;;; IF MI NO

MOV     R4,10(SP)       ;;; SET NEW PC AS ONLY MESSAGE IS REQUIRED

10$:    CALL    @#$FORK2        ;;;

CLR     (R3)            ;; CLEAR THE FORK BLOCK

CALLR   SNDMSG          ;; AND SEND AST TO TASK STATE

20$:    CMP     #-1,PER1        ;;; CHECK IF TO ABORT TASK

BNE     40$             ;;; NE NO

30$:    CALL    @#$FORK2        ;;;

CLR     (R3)            ;; CLEAR THE FORK BLOCK

32$:    MOV     #S.CABO,R0      ;; ABORTING FROM ILLEGAL INSTRUCTION

CALL    $ABCTK          ;; ABORT THE CURRENT TASK

CALLR   SNDMSG          ;; AND SEND AST TO TASK STATE

40$:    CMP     #-2,PER1        ;;; CHECK IF OFFSET JMP

BNE     50$

MOV     PER2,10(SP)     ;;; ADJUST PC

CALL    @#$FORK2        ;;;

CLR     (R3)            ;; CLEAR FORK BLOCK

MOV     #2,-(SP)        ;; SET THE UNITS TO SECONDS

MOVB    PER3,-(SP)      ;; GET THE NUMBER OF SECONDS TO WAIT

BNE     41$             ;; CHECK SPECIFIED

INC     (SP)            ;; DEFAULT IS 1

41$:    CLRB    1(SP)           ;; MAX 255

MOV     SP,R3           ;; POINT R3 TO BLOCK

CALL    $CVRTM          ;; AND CONVERT THE TIME

MOV     R0,2(SP)

MOV     R1,(SP)

42$:    MOVB    PER3+1,R0       ;; GET THE EVENT FLAG TO USE

BNE     43$             ;; EVENT FLAG OF 0 IMPLIES DEFAULT

INCB    PER3+1

BR      42$

43$:    MOV     $TKTCB,R5       ;; AND THE TCB OF THE CURRENT TASK

CALL    $CEFI           ;; AND CONVERT TO MASK WORD AND ADDRESS

BCC     45$

44$:    ADD     #4,SP           ;; CLEAN UP STACK

BR      32$

45$:    CMPB    PER3,#64.       ;; CHECK IF GLOBAL EVENT FLAG

BHI     44$             ;; IF SO GO TO 44 AND ABORT

BIC     R0,(R1)         ;; CLEAR THE EVENT FLAG

MOV     R0,-(SP)        ;; SAVE EVENT FLAG MASK WORD

MOV     R1,-(SP)        ;; AND THE EVENT FLAG MASK ADDRESS

MOV     T.PCB(R5),R4    ;; GENERATE POINTER TO TASK HEADER

MOV     P.HDR(R4),R4

INC     T.ST2(R5)       ;; PUT TASK IN WAIT FOR STATE

MOV     2(SP),H.EFLM(R4) ;; SAVE WAIT FLAG

MOV     (SP),H.EFLM+2(R4)

CALL    $SETRT          ;; SET SCHEDULE REQUEST

MOV     #C.MRKT,R4      ;; MAKE CLOCK BLOCK A MARK TIME BLOCK

Page 15

CALL    $ALCLK          ;; ALLOCATE A CLOCK BLOCK

MOVB    PER3+1,C.EFN(R0) ;; SAVE MARK TIME EVENT FLAG

CLR     C.AST(R0)       ;; CLEAR THE AST ADDRESS

MOV     (SP)+,C.DST(R0) ;; SET THE EVENT FLAG DESTINATION

MOV     (SP)+,C.SRC(R0) ;; AND THE VALUE TO BE SET

MOV     (SP)+,R2        ;; GET THE TIME TO WAIT

MOV     (SP)+,R1

CALL    $CLINS          ;; AND INSERT THE MARK TIME BLOCK IN THE

CALLR   SNDMSG          ;; CLOCK QUEUE, AND SEND AST TO TASK STATE

50$:    CMP     #-3,PER1        ;;; CHECK IF ABSOLUTE JMP

BNE     30$             ;;;

MOV     PER2,10(SP)     ;;;

BR      10$             ;;;

The call to SNDMSG just sends an AST to the user  mode  part  of

the  task.   Starting  at  label 40$ is the code that causes the

task to delay for a time.  One caution  here,  this  method  can

only  be  used with an executing task that is in memory.  We are

sure of that condition in this example, however other cases  may

not be as simple.

The next routine just sends the error  information  to  the  AST

part of this task.  The method has already been explained in the

last multi tasker.

Page 16

;+

;  **-SNDMSG-SEND MESSAGE (VIA AST)

;

; THIS MODULE SENDS AN AST BLOCK TO THIS TASK.

; THE AST PARAMETERS ARE DESCRIBED IN THE AST RECEIVER MODULE

;

; INPUTS:

;       ERL1-PER3 -- INFORMATION FROM THE AST

;

; OUTPUTS:

;       GENERATES AN AST BLOCK WHICH IS QUEUED TO THE TASK

;

; SIDE EFFECTS:

;

;    MODIFIED EXTERNALS

;

;    OTHER SIDE EFFECTS

;

; REGISTER USAGE:

;       ALL REGISTERS ARE FREE TO BE USED

;

;-

SNDMSG:

MOV     #<5+NUMPRM>*2,R1 ;; ALLOCATE AN AST BLOCK FOR ERRLOG

CALL    $ALOCB          ;; ALLOCATE SOME POOL

BCC     10$

IOT                     ;; CRASH SYSTEM

10$:    MOV     R0,R2           ;; COPY ADDRESS OF BLOCK

MOV     R0,ASTADD       ;; SAVE ADDRESS OF BLOCK

CLR     (R2)+           ;; CLEAR THE LINK

MOV     R1,(R2)+        ;; SET IN SIZE OF BLOCK

MOV     #<7+NUMPRM>*2,(R2)+ ;; SET NUMBER OF BYTES FOR STACK

MOV     #ASTAD,(R2)+

MOV     #NUMPRM,(R2)+   ;; SPECIFY THE NUMBER OF PARAMETERS

;

; LOAD STANDARD PARAMETERS

;

MOV     $TKTCB,R0       ;; GET THE POINTER TO THE TCB

MOV     T.NAM(R0),(R2)+ ;; GET THE TASK NAME

MOV     T.NAM+2(R0),(R2)+

MOV     22(SP),(R2)+    ;; GET THE TASK PS

MOV     20(SP),(R2)+    ;; GET THE TASK NEW PC

MFPI    @#$DSW          ;; GET THE USER DSW

MOV     (SP)+,(R2)+

MOV     ERL1,(R2)+      ;; SAVE ERROR LABEL

MOV     ERL2,(R2)+

TST     PER1            ;; CHECK FOR TYPE OF INFO TO TASK

BNE     50$

;

; HERE FOR NORMAL MESSAGE NO TASK AFFECT

;

20$:    MOV     PER2,(R2)+      ;; GET THE FIRST PARAMETER

BNE     30$             ;; IF DEFAULT USE

Page 17

MOV     4(SP),-2(R2)    ;; R0 AS PARAMETER

30$:    MOV     PER3,(R2)       ;; GET THE SECOND PARAMETER

BNE     40$             ;; IF DEFAULT USE

MOV     16(SP),(R2)     ;; R5 AS PARAMETER

40$:    BR      70$

50$:    CMP     PER1,#-1        ;; CHECK FOR FATAL ERROR

BEQ     20$             ;;  TREAT AS NORMAL ERROR

CMP     PER1,#-2        ;; CHECK FOR RECOVERABLE ERROR

BNE     60$

CLR     (R2)+           ;; IF RECOVERABLE ERROR, NO PARAMETER

CLR     (R2)

BR      70$

60$:    CMP     PER1,#-3        ;; CHECK FOR RECOVERABLE ERROR

BNE     20$

CLR     (R2)+           ;; IF RECOVERABLE, NO PARAMETER 1

BR      30$

70$:    MOV     MYTCBA,R0       ;; GET TASK ERRDIS TASK TCB

MOV     ASTADD,R1       ;; GET AST BLOCK ADDRESS

CALLR   @#$QASTT        ;; AND PLACE ON AST QUEUE

All that is left  is  to  see  how  the  table  of  extra  error

information  lines  is  generated.   However,  this  is only the

method I used for the RSX-11S system.  A unified error  file  is

quite possible and is recommended.

Page 18

;+

; FILE DESCRIPTION

;

;       THIS FILE CONTAINS ERROR MESSAGES

;

;       THE MESSAGES ARE STORED UNSORTED IN THE FOLLOWING MANNER:

;

;               PSECT MSGKEY CONTAINS THE FOLLOWING STRUCTURE

;                       FIRST 3 CHARACTERS OF KEY

;                       SECOND 3 CHARACTERS OF KEY

;                       POINTER TO MESSAGE IN PSECT MSGMSG

;

;               THE KEYS ARE TERMINATED BY A KEY WITH SPACES AS THE

;               FIRST 3 CHARACTERS = IE 0

;

;               PSECT MSGMSG CONTAINS THE FOLLOWING STRUCTURE

;                       MESSAGE SIZE, MESSAGE

;

;-

;

;

.MACRO  MAKMSG KEY,MES

.PSECT  MSGMSG

.NCHR   TEMSZ,<MES>

TEMP = .

.BYTE   TEMSZ

.ASCII  /MES/

.PSECT  MSGKEY

.RAD50  /KEY/

.WORD   TEMP

.PSECT

.ENDM   MAKMSG

.PSECT  MSGKEY,RO,D

MSGKYS::

.PSECT  MSGMSG,RO,D

MSG::

.PSECT

MAKMSG  AAAAA,<TEST MESSAGE>

MAKMSG  <      >,<DONE>

.PSECT  MSGMSG

.EVEN

.PSECT  MSGKEY

.EVEN

.PSECT

.END

Page 19

The entire listing of the program, data file, build and assembly

files follow.

;

; PREFIX FILE -- DEFF.MAC

;

.MACRO 	ERROR,P1,P2,P3,P4

;

; MACRO TO SET UP INSTRUCTION BLOCK FOR ERROR MESSAGES

;

;	ERROR	KEY,TYPE,PRAM1,PRAM2

;

;		KEY <= 6 RAD50 CHARACTERS

;		TYPE = -1 FATAL ERROR

;		     = -2 FOR DELAYED RECOVERY

;		     = -3 FOR JUMP RECOVERY

;		     = 0 FOR MESSAGE ONLY

;	SEE DESCRIPTION IN ERRDIS MODULE

;

.WORD	210		; ERROR HANDLING INSTRUCTION

.NCHR	TEMPN,P1

.IIF LT 6-TEMPN .ERROR ; RAD50 NAME > 6 CHARACTERS

.IF  LT 3-TEMPN

.RAD50	/P1/

.IFF

.RAD50	/P1/

.RAD50	/   /

.ENDC

.WORD	P2,P3,P4

.ENDM	ERROR

.MACRO	SAVRG

JSR	R5,$SAVRG

.ENDM

.TITLE	ERRDIS

.IDENT	/X1.0/

;

;

; WRITTEN BY D.B.CURTIS

; SCIENCE APPLICATIONS, INC.

; 14-JUL-81

; ARGONNE NATIONAL LAB

;

;

; VERSION 01

; EDIT NUMBER = 0003

; FILE = ERRDIS.MAC

; EDITED BY: D.B.CURTIS  19 AUG 81  11:25

;

;

; MODIFICATIONS:

;

;+

; FILE DESCRIPTION

;

;	THIS TASK IS AN ERROR LOGGING TASK

;	IT CAN BE USED WITH MOST OTHER RSX11-M/S FACILITIES IF THE ERROR

;	MESSAGES AND KEYS ARE CHANGED.  THIS TASK INTERCEPTS THE ILLEGAL

;	INSTRUCTION TRAP AND IS INVOKED BY THE 210 OP CODE.

;

;	INFORMATION FOLLOWS THAT OPCODE.  THIS INFORMATION DETERMINES HOW

;	THIS TASK PROCESSES THE ERROR.

;

;	THE MAIN PURPOSE OF THIS TASK IS TO DISPLAY ERRORS.  ITS SECONDARY

;	PURPOSE IS TO HELP RECOVER FROM THE ERRORS.  THIS MEANS THAT SOME

;	CONTROL OPERATIONS ARE PROVIDED.

;

;	THE ERROR CONTROL BLOCK LOOKS LIKE:

;

;		-----------------------------------------

;		|     OP CODE (210)			|

;		|---------------------------------------|

;		|     FIRST 3 CHAR. OF RAD50 KEY	|

;		|---------------------------------------|

;		|     SECOND 3 CHAR. OF RAD50 KEY	|

;		|---------------------------------------|

;		|     FIRST PARAMETER			|

;		|---------------------------------------|

;		|     SECOND PARAMETER			|

;		|---------------------------------------|

;		|     THIRD PARAMETER			|

;		-----------------------------------------

;

;	WHERE THE RAD50 KEY MUST MATCH WITH THE KEYS IN THE ERROR MESSAGE

;	TABLE. IF NO MATCH IS FOUND, THE KEY IS PRINTED.

;

;	THE FIRST PARAMETER SPECIFIES THE ACTION THIS TASK TAKES ON THE

;	ISSUING TASK.  THIS PARAMETER TAKES THE FOLLOWING VALUES:

;

;		0 = MESSAGE ONLY, PROCESSING CONTINUES AFTER THE DATA BLOCK

;		THE SECOND AND THIRD PARAMETER CONTAIN DATA TO BE PRINTED

;		IF THE SECOND PARAMETER IS 0 R0 WILL BE PRINTED INSTEAD

;		IF THE THIRD PARAMETER IS 0, R5 WILL BE PRINTED INSTEAD

;

;		-1 = FATAL ERROR, THE MESSAGE IS PRINTED, AND

;		THE TASK IS ABORTED. THE SECOND AND THIRD PARAMETERS ARE

;		TREATED AS IN THE 0 CASE

;

;		-2 = RECOVERABLE ERROR, THE MESSAGE IS PRINTED. THE SECOND

;		PARAMETER CONTAINS THE ADDRESS TO SET THE PC TO.

;		THE FIRST BYTE OF THE THIRD PARAMETER IS THE NUMBER

;		OF SECONDS TO WAIT, AND THE LAST BYTE OF THE THIRD

;		PARAMETER IS THE EVENT FLAG TO USE WHILE WAITING.

;		IF THE SECONDS ARE ZERO, IT WILL DEFAULT TO 1, IF THE EVENT

;		FLAG IS ZERO, IT WILL DEFAULT TO 1.

;

;		-3 = RECOVERABLE ERROR, THE MESSAGE IS PRINTED. THE SECOND

;		PARAMETER CONTAINS THE ADDRESS TO SET THE PC TO.

;		IF PARAMETER 3 IS 0, R5 WILL BE USED AS AN ARGUMENT.

;

;	IF ANOTHER VALUE THE THE ONES LISTED ABOVE, THE ERROR IS PRINTED,

;	AN ERROR MESSAGE FROM THIS TASK IS PRINTED, AND THE OFFENDING

;	TASK IS ABORTED

;

;	YOU WOULD USE -2 FOR AN ERROR THAT REQUIRED OPERATOR INTERVENTION

;	YOU WOULD USE -3 FOR AN ERROR THAT COULD NOT BE TRIED AGAIN,

;	AND JUMP TO AN ERROR EXIT

;

;

;

;-

.MCALL	QIOW$S,CINT$,QIOW$,DIR$,ASTX$S,SPND$S,TCBDF$,ABODF$,HWDDF$

.MCALL	QIO$S,CLKDF$,PCBDF$,HDRDF$

ABODF$

TCBDF$

HWDDF$

CLKDF$

HDRDF$

PCBDF$

;

; NUMBER OF PARAMETERS IN AST

;

NUMPRM = 9.

CINT:	CINT$	ERRVEC,120000,ERRINT,ERRINI,PR7,ASTAD

POLADD:	.WORD	0

POLSIZ:	.WORD	0

MYTCBA:	.WORD	0

ASTADD:	.WORD	0

START:

DIR$	#CINT			; CONNECT TO THE VECTOR, AND INIT IT

BCC	10$			; IF ERROR JUST

IOT				; CRASH

10$:	SPND$S				; ELSE SUSPEND FOREVER

;

; IF THIS TASK IS RESUMED, IT WILL SELF TEST

;

ERROR	AAAAA,0,1,2		; PRINT ERROR MESSAGE WITH DEFAULT PARAMETERS

MOV	#3,R0

MOV	#4,R5

ERROR	BBBBB,0,0,0		; PRINT ERROR MESSAGE WITH R0,R5 PARAM

BR	30$			; NEXT MESSAGE SHOULD BE DDDDD

20$:	ERROR	CCCCC,-3,40$,6		; NEXT MESSAGE WILL BE A DELAY MESSAGE

30$:	ERROR	DDDDD,-3,20$,5		; CCCCC IS NEXT MESSAGE

40$:	ERROR	EEEEE,-2,50$,<<15*400>+5> ; CHECK FOR 5 SECOND DELAY

50$:	ERROR	FFFFF,-2,60$,<<0*400>+0>  ; CHECK DEFAULT DELAY AND EVENT FLAG

60$:	ERROR	DONE,-3,10$,0		; ALL DONE

;+

;  **-ASTAD-AST RECEIVER/ERROR DISPLAY MODULE

;

; THIS MODULE RECEIVES THE AST BLOCK FROM THE FORK CODE.

; IT TAKES THAT INFORMATION, SEARCHES FOR A MATCHING KEY IN THE

; EXPANDED MESSAGE DATA AREA AND IF ONE IS FOUND THE EXPANDED MESSAGE

; IS WRITTEN OUT.  REGARDLESS OF WEITHER THE EXPANDED MESSAGE IS

; FOUND, THE PARAMETERS IN THE AST BLOCK ARE WRITTEN OUT AS:

;

; 'TSKNAM' PS = XXXXXX, PC = XXXXXX, DSW = XXXXXX

; ERROR 'KEYNAM', PRAMS = XXXXXX XXXXXX

;

;

; INPUTS:

;	INFORMATION IS ON THE STACK, ALL REGISTERS MUST BE SAVED

;	THE INFORMATION  ON THE STACK IS:

;		2 WORDS OF RAD50 TASK NAME ISSUING THE INSTRUCTION

;		PS OF THAT TASK

;		PC OF THAT TASK

;		DSW OF THAT TASK

;		2 WORDS OF RAD50 ERROR KEYWORD

;		FIRST PARAMETER

;		SECOND PARAMETER

;

; OUTPUTS:

;	NONE

;

; SIDE EFFECTS:

;

;    MODIFIED EXTERNALS

;

;    OTHER SIDE EFFECTS

;	STACK IS CLEANED, MESSAGE IS PRINTED

;

; REGISTER USAGE:

;	ALL REGISTERS SAVED

;

;-

;

; REFERENCED GLOBALS

;

.GLOBL	$EDMSG 	; EDIT MESSAGE ( SYSTEM LIBRARY ROUTINES MANUAL)

.GLOBL	MSGKYS	; ADDRESS OF THE ERROR KEY TABLE

.GLOBL	TTLUN	; LUN OF THE TERMINAL I/O

.GLOBL	TTEV	; EVENT FLAG FOR TERMINAL OPERATIONS

;

; EQUATED SYMBOLS

;

BEL = 7			; ASCII CODE FOR BELL

.ASECT

. = 0

REGS:	.BLKW	6	; R5-R0 R5 IS FIRST

ERTNM:	.BLKW	2	; 2 WORDS OF RAD50 TASK NAME FROM ERROR

ETPS:	.BLKW	1	; ERRORING TASKS PS

ETPC:	.BLKW	1	; ERRORING TASKS PC

ETDSW:	.BLKW	1	; ERRORING TASKS DSW

EKEY:	.BLKW	2	; 2 RAD50 WORDS WITH KEY NAME

ETP1:	.BLKW	1	; FIRST PARAMETER

ETP2:	.BLKW	1	; SECOND PARAMETER

.PSECT

.ENABL	LSB

ASTAD:

.IRPC	REG,<012345>		; SAVE THE REGISTERS

MOV	R'REG,-(SP)

.ENDM

;

; GET THE INFORMATION FROM THE INTERRUPT PART AND DISPLAY IT

;

MOV	SP,R0			; GET THE ADDR OF THE ERROR DATA BLOCK

MOV	EKEY(R0),R1		; GET THE FIRST WORD OF THE KEY

MOV	#MSGKYS,R2		; AND THE ADDRESS OF THE KEYS

10$:	CMP	R1,(R2)			; IS THE FIRST PART OF THE KEY THE SAME?

BNE	20$

; HERE THE FIRST PART OF THE KEY MATCHES

CMP	EKEY+2(R0),2(R2)	; DOES THE SECOND PART MATCH?

BNE	20$			; IF NE NO, SO CONTINUE

BR	50$

20$:	ADD	#6,R2			; ADVANCE TO NEXT KEY

TST	(R2)			; CHECK NOT END OF KEYS

BNE	10$			; KEY NOT FOUND, => NO MESSAGE OTHER

;   OTHER THEN KEY EITHER

;

; HERE, CONVERT KEY TO ASCII, AND PRINT IT AND OTHER VARIABLES

;

.PSECT	MESFMT,RO,D

.NLIST	BEX

MG1:	.ASCII	<BEL><BEL>/'%2R' PS = %P, PC = %P, DSW = %P%N/

.ASCIZ	/ERROR '%2R', PRAMS = %2P%N/

.LIST	BEX

.PSECT

30$:	MOV	SP,R2			; SAVE POINTER TO PARAMETERS

ADD	#ERTNM,R2

SUB	#200.,SP		; SAVE SPACE FOR LINE ON STACK

MOV	SP,R0

MOV	SP,R5

MOV	#MG1,R1			; POINT TO FORMAT INFORMATION

CALL	$EDMSG			; FORMAT IT

CMP	#200.,R1		; CHECK STACK DIDN'T GET WIPED

BGE	40$

IOT

40$:	QIOW$S	#IO.WBT,#TTLUN,#TTEV,,,,<R5,R1,#40>

ADD	#200.,SP		; CLEAN UP STACK

BR	60$

;

; HERE PRINT MESSAGE, AND VARIABLES

;

50$:

MOV	4(R2),R1		; GET ADDRESS OF MESSAGE

MOVB	(R1)+,R2		; GET SIZE OF MESSAGE

QIO$S	#IO.WBT,#TTLUN,,,,,<R1,R2,#40>

BR	30$

60$:	.IRPC	REG,<543210>		; RESTORE THE REGISTERS

MOV	(SP)+,R'REG

.ENDM

ADD	#2*NUMPRM,SP		; CLEAN OFF PARAMETERS

ASTX$S

.DSABL	LSB

;+

;  **-ERRINI-CONNECT TO INTERRUPT ENABLE/DISABLE ROUTINE

;

; THIS ROUTINE ENABLES/DISABLES THE ILLEGAL INSTRUCTION INTERCEPT

; IF CARRY IS CLEAR, IT ENABLES, IF SET, IT DISABLES

; THIS MODULE ALLOCATES SOME POOL MEMORY, PLACES THE ILLEGAL INSTRUCTION

; INTERCEPT CODE INTO THE POOL AREA, MODIFIES XDT TO NOT PLAY WITH THE

; ILLEGAL INSTRUCTION TRAP VECTOR (ONLY IF XDT IS PRESENT) AND RETURNS.

;

; THE DISABLE OF THIS OPERATION UNDOES ALL OF THE ABOVE

;

; INPUTS:

;	CARRY SET TO DETACH FROM VECTOR, CLEAR TO ATTACH TO VECTOR

;

; OUTPUTS:

;	NONE

;

; SIDE EFFECTS:

;

;    MODIFIED EXTERNALS

;

;    OTHER SIDE EFFECTS

;	POLADD,POLSIZ ARE SET TO THE ADDRESS OF THE POOL AREA, AND ITS SIZE

;	THE ILLEGAL INSTRUCTION TRAP VECTOR IS INTERCEPTED.

;

; REGISTER USAGE:

;	R1,R4,R5 ARE SAVED

;

;-

;

; REFERENCED GLOBALS

;

.GLOBL	$ALOCB		; ALLOCATE CORE BLOCK

.GLOBL	$TKTCB		; CURRENT TASK TCB ADDRESS

.GLOBL	$DEACB		; DEALLOCATE CORE BLOCK

.GLOBL	ERRVEC		; VECTOR TO SEND ERROR INSTRUCTION TOO

.GLOBL	XDTFIX		; OFFSET FROM $XDT TO PATCH XDT ILLEGAL

; INSTRUCTION VECTOR RESET

.GLOBL	$XDT 		; ENTRY POINT TO XDT

.GLOBL	ILLVEC		; TRAP VECTOR ADDRESS FOR ILLEGAL INSTRUCTION

ERRINI:

BCC	10$			;; CONNECT TO ILLEGAL INSTRUCTION VEC

JMP	ABOPRO			;; IF CARRY SET, DISCONNECT

10$:	MOV	R5,-(SP)		;; SAVE REGISTERS

MOV	R4,-(SP)

MOV	R1,-(SP)

MOV	#EPOLCD-POOLCD,R1	;; NEED TO ALLOCATE POOL SPACE TO PUT

CALL	$ALOCB			;; TRAP INTERCEPT CODE

BCC	20$

IOT				;; IF NO POOL SPACE, CRASH SYSTEM

20$:	MOV	R0,POLADD		;; SAVE ADDRESS OF ALLOCATED BLOCK

MOV	R1,POLSIZ		;; AND SIZE OF ALLOCATED BLOCK

MOV	$TKTCB,MYTCBA		;; AND WHILE AT IT, SET MY TCB ADDRESS

MOV	#<EPOLCD-POOLCD>/2,R2	;; GET THE NUMBER OF BYTES TO COPY

MOV	#POOLCD,R3		;; AND THE ADDRESS OF CODE TO MOVE

;; TOO THE POOL

30$:	MOV	(R3)+,(R0)+		;; DO THE MOVE

SOB	R2,30$			;; UNTIL FULLY COPIED

MOV	POLADD,R0		;; GET THE START ADDRESS OF POOL CODE

ADD	#2,R0			;; BUMP PAST THE "ILL"

MOV	POLSIZ,(R0)+		;; SAVE THE NUMBER OF BYTES IN CODE

MOV	@#ILLVEC,(R0)+		;; SAVE THE ORIGINAL VECTOR

MOV	#NOP,(R0)		;; TURN ON THE CODE

MOV	#$XDT,R2		;; CHECK IF XDT IS AVAILABLE

BEQ	40$			;; IF EQ NO

SUB	#XDTFIX,R2		;; CHECK IF XDT IS CONSISTENT

CMP	@#ILLVEC,(R2)

BNE	40$

MOV	R0,(R2)			;; ADJUST XDT

40$:	MOV	R0,@#ILLVEC		;; POINT ILLEGAL INSTRUCTION TRAP TO

;; THIS NEW CODE

MOV	(SP)+,R1		;; RESTORE REGISTERS

MOV	(SP)+,R4

MOV	(SP)+,R5

RETURN				;; AND RETURN TO TASK

;

; CODE THAT GETS LOADED INTO THE POOL

;  >>>>>> THIS CODE MUST BE PIC <<<<<

;

POOLCD:

.RAD50	/ILL/			;;; SHOW THIS CODE IS ATTACHED TO THE

.WORD	0,0			;;; ILLEGAL INSTRUCTION VECTOR

RTI				;;; INITIALLY, TURN OFF CODE

EP:	MOV	R5,-(SP)		;;; SAVE R5

MOV	2(SP),R5		;;; AND GET TASK PC FROM STACK

MFPI	-(R5)			;;; GET THE INSTRUCTION THAT CAUSED THE TRAP

CMP	(SP),#210		;;; IS IT OUR INSTRUCTION?

BEQ	100$

ADD	#2,SP			;;; NO! SO CLEAR THE STACK

MOV	(SP)+,R5		;;; RESTORE R5

JMP	@EP-4			;;; AND PROCESS NORMALLY

;

; HERE IT IS OUR ILLEGAL INSTRUCTION

;

100$:	ADD	#2,SP			;;; CLEAN THE STACK

MOV	(SP)+,R5		;;; RESTORE R5

MOV	@#ERRVEC,-(SP)		;;; AND JUMP VIA THE VECTOR WE ARE CONNECTED TO

JMP	@(SP)+			;;; POSITION INDEPENDENT INDIRECT JUMP

EPOLCD:					;;; END OF THE CODE INSERTED INTO THE POOL

ABOPRO:

MOV	R1,-(SP)		;; SAVE ITB POINTER

MOV	@#ILLVEC,R0		;; GET CURRENT VECTOR TO FIND OUR CODE

MOV	-2(R0),@#ILLVEC		;; RESET THE ILLEGAL INSTRUCTION VEC

MOV	#$XDT,R2		;; RESET XDT

BR	1$			;; IF IT IS IN THE SYSTEM

SUB	#XDTFIX,R2

MOV	-2(R0),(R2)

1$:	MOV	POLADD,R0		;; GET THE ADDRESS OF THE POOL CODE

MOV	POLSIZ,R1		;; AND THE SIZE

CALL	$DEACB

MOV	(SP)+,R1		;; RESTORE ITB POINTER

RETURN

;+

;  **-ERRINT-PROCESSES ERROR INTERRUPTS

;

; THIS MODULE RECEIVES THE 210 INSTRUCTION CODE.

; IT SAVES THE ERROR BLOCK PARAMETERS, DOES ANY PROCESSING OF THE

; PARAMETERS (CHECKS WHAT TO DO WITH THE ISSUING TASKS PC).

; FORKS AND PLACES THE TASK IN THE APPROPRIATE STATE

;

; INPUTS:

;	R5 POINTS TO THE ITB

;	R4 IS SAVED AND MAY BE USED

;

; OUTPUTS:

;	NONE

;

; SIDE EFFECTS:

;

;    MODIFIED EXTERNALS

;

;    OTHER SIDE EFFECTS

;	MANY, TASK MAY BE ABORTED, MADE TO MARK TIME, TASKS PC MAY BE

;	MOVED AS DIRECTED BY THE PARAMETERS ETC.

;

; REGISTER USAGE:

;

;	R5 POINTS TO ITB

;	R4 IS SAVED AND MAY BE USED, ALL OTHER REGISTERS MUST BE SAVED AND

;	RESTORED

;

;-

;

; REFERENCED GLOBALS

;

.GLOBL	$FORK2

;

; LOCAL DATA

;

ERL1:	.WORD	0		; TEMP STORAGE FOR ERROR KEY AND PARAMETERS

ERL2:	.WORD	0

PER1:	.WORD	0

PER2:	.WORD	0

PER3:	.WORD	0

ERRINT:					;;; HERE IF ILLEGAL INSTRUCTION

;;; FIRST 2 WORDS ARE RAD50 REP OF NAME OF ERROR

MOV	10(SP),R4		;;; GET PC OF TASK

MFPI	(R4)+			;;; GET FIRST WORD OF RAD50 ERROR

MOV	(SP)+,ERL1		;;;

MFPI	(R4)+			;;;

MOV	(SP)+,ERL2		;;;

MFPI	(R4)+			;;;

MOV	(SP)+,PER1		;;;

MFPI	(R4)+			;;;

MOV	(SP)+,PER2		;;;

MFPI	(R4)+			;;;

MOV	(SP)+,PER3		;;;

TST	PER1			;;; CHECK IF MESSAGE ONLY

BMI	20$			;;; IF MI NO

MOV	R4,10(SP)		;;; SET NEW PC AS ONLY MESSAGE IS REQUIRED

10$:	CALL	@#$FORK2		;;;

CLR	(R3)			;; CLEAR THE FORK BLOCK

CALLR	SNDMSG			;; AND SEND AST TO TASK STATE

20$:	CMP	#-1,PER1		;;; CHECK IF TO ABORT TASK

BNE	40$			;;; NE NO

30$:	CALL	@#$FORK2		;;;

CLR	(R3)			;; CLEAR THE FORK BLOCK

32$:	MOV	#S.CABO,R0		;; ABORTING FROM ILLEGAL INSTRUCTION

CALL	$ABCTK			;; ABORT THE CURRENT TASK

CALLR	SNDMSG			;; AND SEND AST TO TASK STATE

40$:	CMP	#-2,PER1		;;; CHECK IF OFFSET JMP

BNE	50$

MOV	PER2,10(SP)		;;; ADJUST PC

CALL	@#$FORK2		;;;

CLR	(R3)			;; CLEAR FORK BLOCK

MOV	#2,-(SP)		;; SET THE UNITS TO SECONDS

MOVB	PER3,-(SP)		;; GET THE NUMBER OF SECONDS TO WAIT

BNE	41$			;; CHECK SPECIFIED

INC	(SP)			;; DEFAULT IS 1

41$:	CLRB	1(SP)			;; MAX 255

MOV	SP,R3			;; POINT R3 TO BLOCK

CALL	$CVRTM			;; AND CONVERT THE TIME

MOV	R0,2(SP)

MOV	R1,(SP)

42$:	MOVB	PER3+1,R0		;; GET THE EVENT FLAG TO USE

BNE	43$			;; EVENT FLAG OF 0 IMPLIES DEFAULT

INCB	PER3+1

BR	42$

43$:	MOV	$TKTCB,R5		;; AND THE TCB OF THE CURRENT TASK

CALL	$CEFI			;; AND CONVERT TO MASK WORD AND ADDRESS

BCC	45$

44$:	ADD	#4,SP			;; CLEAN UP STACK

BR	32$

45$:	CMPB	PER3,#64.		;; CHECK IF GLOBAL EVENT FLAG

BHI	44$			;; IF SO GO TO 44 AND ABORT

BIC	R0,(R1)			;; CLEAR THE EVENT FLAG

MOV	R0,-(SP)		;; SAVE EVENT FLAG MASK WORD

MOV	R1,-(SP)		;; AND THE EVENT FLAG MASK ADDRESS

MOV	T.PCB(R5),R4		;; GENERATE POINTER TO TASK HEADER

MOV	P.HDR(R4),R4

INC	T.ST2(R5)		;; PUT TASK IN WAIT FOR STATE

MOV	2(SP),H.EFLM(R4)	;; SAVE WAIT FLAG

MOV	(SP),H.EFLM+2(R4)

CALL	$SETRT			;; SET SCHEDIUAL REQUEST

MOV	#C.MRKT,R4		;; MAKE CLOCK BLOCK A MARK TIME BLOCK

CALL	$ALCLK			;; ALLOCATE A CLOCK BLOCK

MOVB	PER3+1,C.EFN(R0)	;; SAVE MARK TIME EVENT FLAG

CLR	C.AST(R0)		;; CLEAR THE AST ADDRESS

MOV	(SP)+,C.DST(R0)		;; SET THE EVENT FLAG DESTINATION

MOV	(SP)+,C.SRC(R0)		;; AND THE VALUE TO BE SET

MOV	(SP)+,R2		;; GET THE TIME TO WAIT

MOV	(SP)+,R1

CALL	$CLINS			;; AND INSERT THE MARK TIME BLOCK IN THE

CALLR	SNDMSG			;; CLOCK QUEUE, AND SEND AST TO TASK STATE

50$:	CMP	#-3,PER1		;;; CHECK IF ABSOLUTE JMP

BNE	30$			;;;

MOV	PER2,10(SP)		;;;

BR	10$			;;;

;+

;  **-SNDMSG-SEND MESSAGE (VIA AST)

;

; THIS MODULE SENDS AN AST BLOCK TO THIS TASK.

; THE AST PARAMETERS ARE DESCRIBED IN THE AST RECEIVER MODULE

;

; INPUTS:

;	ERL1-PER3 -- INFORMATION FROM THE AST

;

; OUTPUTS:

;	GENERATES AN AST BLOCK WHICH IS QUEUED TO THE TASK

;

; SIDE EFFECTS:

;

;    MODIFIED EXTERNALS

;

;    OTHER SIDE EFFECTS

;

; REGISTER USAGE:

;	ALL REGISTERS ARE FREE TO BE USED

;

;-

;

; REFERENCED GLOBALS

;

.GLOBL	$QASTT		; QUEUE AST TO TASK

SNDMSG:

MOV	#<5+NUMPRM>*2,R1	;; ALLOCATE AN AST BLOCK FOR ERRLOG

CALL	$ALOCB			;; ALLOCATE SOME POOL

BCC	10$

IOT				;; CRASH SYSTEM

10$:	MOV	R0,R2			;; COPY ADDRESS OF BLOCK

MOV	R0,ASTADD		;; SAVE ADDRESS OF BLOCK

CLR	(R2)+			;; CLEAR THE LINK

MOV	R1,(R2)+		;; SET IN SIZE OF BLOCK

MOV	#<7+NUMPRM>*2,(R2)+	;; SET NUMBER OF BYTES FOR STACK

MOV	#ASTAD,(R2)+

MOV	#NUMPRM,(R2)+		;; SPECIFY THE NUMBER OF PARAMETERS

;

; LOAD STANDARD PARAMETERS

;

MOV	$TKTCB,R0		;; GET THE POINTER TO THE TCB

MOV	T.NAM(R0),(R2)+		;; GET THE TASK NAME

MOV	T.NAM+2(R0),(R2)+

MOV	22(SP),(R2)+		;; GET THE TASK PS

MOV	20(SP),(R2)+		;; GET THE TASK NEW PC

MFPI	@#$DSW			;; GET THE USER DSW

MOV	(SP)+,(R2)+

MOV	ERL1,(R2)+		;; SAVE ERROR LABEL

MOV	ERL2,(R2)+

TST	PER1			;; CHECK FOR TYPE OF INFO TO TASK

BNE	50$

;

; HERE FOR NORMAL MESSAGE NO TASK AFFECT

;

20$:	MOV	PER2,(R2)+		;; GET THE FIRST PARAMETER

BNE	30$			;; IF DEFAULT USE

MOV	4(SP),-2(R2)		;; R0 AS PARAMETER

30$:	MOV	PER3,(R2)		;; GET THE SECOND PARAMETER

BNE	40$			;; IF DEFAULT USE

MOV	16(SP),(R2)		;; R5 AS PARAMETER

40$:	BR	70$

50$:	CMP	PER1,#-1		;; CHECK FOR FATAL ERROR

BEQ	20$			;;  TREAT AS NORMAL ERROR

CMP	PER1,#-2		;; CHECK FOR RECOVERABLE ERROR

BNE	60$

CLR	(R2)+			;; IF RECOVERABLE ERROR, NO PARAMETERS

CLR	(R2)

BR	70$

60$:	CMP	PER1,#-3		;; CHECK FOR RECOVERABLE ERROR

BNE	20$

CLR	(R2)+			;; IF RECOVERABLE, NO PARAMETER 1

BR	30$

70$:	MOV	MYTCBA,R0		;; GET TASK ERRDIS TASK TCB

MOV	ASTADD,R1		;; GET AST BLOCK ADDRESS

CALLR	@#$QASTT		;; AND PLACE ON AST QUEUE

.END	START

.TITLE	ERRDAT - ERROR DISPLAY DATA/MESSAGES

.IDENT	/X1.0/

;

;

; WRITTEN BY D.B.CURTIS

; SCIENCE APPLICATIONS, INC.

; 14-JUL-81

; ARGONNE NATIONAL LAB

;

;

; VERSION 01

; EDIT NUMBER = 0001

; FILE = ERRDISDAT.MAC

; EDITED BY: D.B.CURTIS  14-JUL-81 10:11:46

;

;

; MODIFICATIONS:

;

;+

; FILE DESCRIPTION

;

;	THIS FILE CONTAINS ERROR MESSAGES

;

;	THE MESSAGES ARE STORED UNSORTED IN THE FOLLOWING MANNER:

;

;		PSECT MSGKEY CONTAINS THE FOLLOWING STRUCTURE

;			FIRST 3 CHARACTERS OF KEY

;			SECOND 3 CHARACTERS OF KEY

;			POINTER TO MESSAGE IN PSECT MSGMSG

;

;		THE KEYS ARE TERMINATED BY A KEY WITH SPACES AS THE

;		FIRST 3 CHARACTERS = IE 0

;

;		PSECT MSGMSG CONTAINS THE FOLLOWING STRUCTURE

;			MESSAGE SIZE, MESSAGE

;

;-

;

;

.MACRO	MAKMSG KEY,MES

.PSECT	MSGMSG

.NCHR	TEMSZ,<MES>

TEMP = .

.BYTE	TEMSZ

.ASCII	/MES/

.PSECT	MSGKEY

.RAD50	/KEY/

.WORD	TEMP

.PSECT

.ENDM	MAKMSG

.PSECT	MSGKEY,RO,D

MSGKYS::

.PSECT	MSGMSG,RO,D

MSG::

.PSECT

MAKMSG	AAAAA,<TEST MESSAGE>

MAKMSG	<      >,<DONE>

.PSECT	MSGMSG

.EVEN

.PSECT	MSGKEY

.EVEN

.PSECT

.END

;

; ERRDATASM.CMD

;	COMMAND FILE TO ASSEMBLE ERRDIS AND ERRDAT MODULES

;

ERRDIS,ERRDIS=LB:[1,1]EXEMC/ML,[11,10]RSXMC,SY:[125,5]DEFF,ERRDIS

ERRDAT,ERRDAT=LB:[1,1]EXEMC/ML,[11,10]RSXMC,SY:[125,5]DEFF,ERRDAT

;

; ERRDISBLD.CMD

;	COMMAND FILE TO BUILD ERROR DISPLAY TASK

;

;   YOU MAY GET A UNDEFINED SYMBOLE '$XDT' IF XDT ISN'T IN THE SYSTEM.

;     THIS IS EXPECTED AND SHOULD NOT BE FIXED! (IT DEFAULTS TO 0)

;

ERRDIS/PR:5/-DA, = ERRDIS,ERRDAT

LB:[1,54]RSX11M.STB/SS

/

TASK = ERRDIS

;

; SET UP TERMINAL INTERACTION USE LUN4 AND EVENT FLAG 4

;

ASG = TI:4

GBLDEF = TTLUN:4

GBLDEF = TTEV:4

;

; OFFSET IN XDT TO INITIALLIZATION TABLE

;

GBLDEF = XDTFIX:300

;

; ILLEGAL INSTRUCTION TRAP VECTOR

;

GBLDEF= ILLVEC:10

;

; UNUSED VECTOR TO CONNECT TO

;

GBLDEF = ERRVEC:774

//