KERMIT PROTOCOL MANUAL

Fourth Edition

Revision 1

1

Frank da Cruz, Bill Catchings

Columbia University Center for Computing Activities

New York, New York 10027

4 November 1983

Copyright (C) 1981,1982,1983

Trustees of Columbia University in the City of New York

Permission is granted to any individual or institution to copy or

use this document and the programs described in it, except for

explicitly commercial purposes.

_______________

1

Present address: Lehman Bros Kuhn Loeb, NYC

- 1 -

Preface to the Fourth Edition

The  fourth  edition  of the KERMIT Protocol Manual incorporates some new ideas

that grew from our experience in attempting to implement some of  the  features

described  in  earlier editions, particularly user/server functions.  These in-

clude a mechanism to allow batch transfers to be interrupted gracefully for ei-

ther the current file or the entire batch of  files;  a  "capability  mask";  a

protocol  extension  for passing file attributes.  In addition, numbers are now

written in decimal notation rather than octal,  which  was  confusing  to  many

readers.   Also, several incompatible changes were made in minor areas where no

attempts at an implementation had yet been made; these include:

- The format and interpretation of the operands to the server commands.

- Usurpation of the reserved fields 10-11 of the Send-Init packet,  and

addition of new reserved fields.

Most of the remaining material has been rewritten and reorganized, and much new

material  added, including a section on the recommonded vocabulary for documen-

tation and commands.

The previous edition of the Protocol Manual attempted to define "protocol  ver-

sion  3";  this  edition abandons that concept.  Since KERMIT development is an

unorganized, disorderly, distributed enterprise, no requirement can be  imposed

on  KERMIT  implementors  to include a certain set of capabilities in their im-

plementations.  Rather,  in  this  edition  we  attempt  to  define  the  basic

functionality of KERMIT, and then describe various optional functions.

The  key  principle  is  that any implementation of KERMIT should work with any

other, no matter how advanced the one or how primitive the other.  The capabily

mask and other Send-Init fields attempt to promote this principle.

Before attempting to write a new Kermit program, be sure to check with the  au-

thors  to make sure that no one else is working on the same thing, and that you

have the latest information.

The Columbia University DEC-20 implementation of KERMIT  should  be  considered

the definitive implementation of the protocol.  The true test of any new Kermit

is whether it can talk to Columbia DEC-20 Kermit.

KERMIT  is distributed from Columbia University on magnetic tape.  Complete or-

dering instructions are in the Kermit Users Guide.

KERMIT Distribution

Columbia University Center for Computing Activities

7th Floor, Watson Laboratory

612 West 115th Street

New York, NY  10025

No warranty of the software nor of the accuracy of the documentation  surround-

ing it is expressed or implied, and neither the authors nor Columbia University

acknowledge any liability resulting from program or documentation errors.

- 2 -

1. Introduction

This  manual  describes the KERMIT protocol.  It is assumed that you understand

the purpose and operation of the Kermit file transfer  facility,  described  in

the  Kermit  Users Guide, and basic terminology of data communications and com-

puter programming.

1.1. Background

The KERMIT file transfer protocol is intended for use in an  environment  where

there  may  be  a  diverse  mixture of computers -- micros, personal computers,

workstations, laboratory computers, timesharing systems -- from  a  variety  of

manufacturers.    All  these systems need have in common is the ability to com-

municate in ASCII over ordinary serial telecommunication lines.

KERMIT was originally designed at Columbia University to meet the need for file

transfer between our DECSYSTEM-20 and IBM  370-series  mainframes  and  various

microcomputers.   It turned out that the diverse characteristics of these three

kinds of systems resulted in a design that was general enough to fit almost any

system.  The IBM mainframe, in  particular,  strains  most  common  assumptions

about how computers communicate.

1.2. Overview

The  KERMIT  protocol is specifically designed for character-oriented transmis-

sion over serial telecommunication lines.  The design allows for  the  restric-

tions and peculiarities of the medium and the requirements of diverse operating

environments  --  buffering,  duplex, parity, character set, file organization,

etc.  The protocol is carried out by KERMIT programs on each end of the  serial

connection  sending "packets" back and forth; the sender sends file names, file

contents, and control information; the  receiver  acknowledges  (positively  or

negatively) each packet.

The  packets  have  a  layered  design,  in  keeping  with  the  ANSI  and  ISO

philosophies, with the outermost fields used by the data link layer  to  verify

data  integrity,  the  next  by the session layer to verify continuity, and the

data itself at the application level.

Connections between systems are established by the ordinary user.  In a typical

case, the user runs KERMIT on a microcomputer, enters terminal emulation,  con-

nects  to  a remote host computer (perhaps by dialing up), logs in, runs KERMIT

on the remote host, and then issues commands to that KERMIT  to  start  a  file

transfer,  "escapes"  back  to the micro, and issues commands to that KERMIT to

start its side of the file transfer.  Files may be  transferred  singly  or  in

groups.

Basic  KERMIT  provides only file transfer, and that is provided for sequential

files only, though the protocol attempts to allow for various types of  sequen-

tial  files.    Microcomputer  implementations  of  KERMIT are also expected to

provide terminal emulation, to facilitate the initial connection.

More advanced implementations simplify the "user interface" somewhat by  allow-

ing  the  KERMIT  on  the  remote host to run as a "server", which can transfer

files in either direction upon command from the local "user" Kermit.  The serv-

- 3 -

er  can  also  provide  additional functionality, such as file management, mes-

sages, mail, and so forth.  Other optional features  also  exist,  including  a

variety  of  block  check  types,  a mechanism for passing 8-bit data through a

7-bit communication link, a way to compressing a repeated sequence  of  charac-

ters, and so forth.

As  local area networks become more popular, inexpensive, and standardized, the

demand for KERMIT and similar protocols may dwindle, but will never wither away

entirely.  Unlike hardwired networks, KERMIT gives the ordinary user the  power

to  establish  reliable  error-free connections between any two computers; this

may always be necessary for one-shot or long-haul connections.

- 4 -

2. Definitions

2.1. General Terminology

TTY:  This  is the term commonly used for a device which is connected to a com-

puter over an EIA RS-232 serial telecommunication line.  This  device  is  most

commonly  an  ASCII  terminal,  but  it  may be a microcomputer or even a large

multi-user computer emulating  an  ASCII  terminal.    Most  computers  provide

hardware (RS-232 connectors and UARTs) and software (device drivers) to support

TTY  connections;  this is what makes TTY-oriented file transfer protocols like

KERMIT possible on almost any system at little or no cost.

LOCAL: When two machines are connected, the LOCAL machine is the one which  you

interact  with  directly,  and which is in control of the terminal.  The "local

Kermit" is the one that runs on the local machine.  A local Kermit always  com-

municates  over an external device (the micro's communication port, an assigned

TTY line, etc).

REMOTE: The REMOTE machine is the one on the far side of the connection,  which

you  must  interact with "through" the local machine.  The "remote Kermit" runs

on the remote machine.  A remote  Kermit  usually  communicates  over  its  own

"console", "controlling terminal", or "standard i/o" device.

HOST:  Another word for "computer", usually meaning a computer that can provide

a home for multiple users or applications.  This term should be avoided in KER-

MIT lore, unless preceded immediately by LOCAL or REMOTE, to denote which  host

is meant.

SERVER:  An  implementation  of  remote  Kermit that can accept commands from a

local Kermit program, instead of directly from the user.

USER: In addition to its usual use to denote  the  person  using  a  system  or

program,  "user"  will also be used refer to the local Kermit program, when the

remote Kermit is a server.

2.2. Numbers

All numbers in the following text are expressed in decimal (base  10)  notation

unless otherwise specified.

Numbers  are  also  referred  to  in terms of their bit positions in a computer

word.  Since KERMIT may be implemented on computers with various word sizes, we

start numbering the bits from the "right" -- bit 0 is  the  least  significant.

Bits  0-5  are  the  6 least significant bits; if they were all set to one, the

value would be 63.

A special quirk in terminology, however, refers to the  high  order  bit  of  a

character  as  it  is  transmitted on the communication line, as the "8th bit".

More properly, it is bit 7.  References to the "8th  bit"  generally  are  with

regard to that bit which ASCII transmission sets aside for use as a parity bit.

KERMIT  concerns  itself with whether this bit can be usurped for the transmis-

sion of data, and if not, it may resort to "8th-bit prefixing".

- 5 -

2.3. Character Set

All  characters  are  in ASCII (American national Standard Code for Information

Interchange) representation, ANSI standard X3.4-1968.  All  implementations  of

KERMIT transmit and receive characters only in ASCII.

ASCII character mnemonics:

NUL     Null, idle, ASCII character 0.

SOH     Start-of-header, ASCII character 1 (Control-A).

SP      Space, blank, ASCII 32.

CR      Carriage return, ASCII 13 (Control-M).

LF      Linefeed, ASCII 10 (Control-J).

CRLF    A carriage-return linefeed sequence.

DEL     Delete, rubout, ASCII 127.

A  control  character  is  considered  to be any ASCII character in the range 0

through 31, or the DEL character (127).  In this document,  control  characters

are written in several ways:

Control-A

This  denotes  ASCII  character 1, commonly referred to as "Control-A".

Control-B is ASCII character 2, and so forth.

CTRL-A  This is a common abbreviation for "Control-A".  A control character  is

generally  typed  at a computer terminal by holding down the key marked

CTRL and pressing the corresponding alphabetic character, in this  case

"A".

^A      "Upparow"  notation  for  CTRL-A.  Many computer systems "echo" control

characters in this fashion.

A printable ASCII character is considered to be any character in the  range  32

(SP) through 126 (tilde).

2.4. Conversion Functions

Several  conversion functions are useful in the description of the protocol and

in the program example.  The machine that Kermit runs on need operate  only  on

integer data; these are functions that operate upon the numeric value of single

ASCII characters.

char(x) = x+32  Transforms  the integer x, which is assumed to lie in the range

0 to 94, into a printable ASCII character; 0 becomes SP, 1  be-

comes "!", etc.

unchar(x) = x-32

Transforms  the  character  x,  which  is  assumed to be in the

printable range (SP through tilde),  into  an  integer  in  the

range 0 to 94.

ctl(x) = x XOR 64

Maps  between  control characters and their printable represen-

- 6 -

2

tations,  preserving  the  high-order  bit .  If x is a control

character, then

x = ctl(ctl(x))

that is, the same function is used to  controllify  and  uncon-

trollify.  The argument is assumed to be a true control charac-

ter  (0 to 31, or 127), or the result of applying CTL to a true

control character (i.e. 63 to 95).   The  transformation  is  a

mnemonic one -- ^A becomes A and vice versa.

2.5. Protocol Jargon

A  Packet  is  a  clearly  delimited  string  of characters, comprised "control

fields" nested around data; the control fields allow a KERMIT program to deter-

mine whether the data has been transmitted correctly and completely.  A  packet

is the smallest unit of transmission in the KERMIT protocol.

ACK  stands  for "Acknowledge".  An ACK is a packet that is sent to acknowledge

receipt of another packet.  Not to be confused with the ASCII character ACK.

NAK stands for "Negative Acknowledge".  A NAK is a packet sent to  say  that  a

corrupted  or incomplete packet was received, the wrong packet was received, or

an expected packet was not received.  Not to be confused with the ASCII charac-

ter NAK.

_______________

2

The high order bit is normally the parity bit, but Kermit uses this  bit  as

data  in  order to do 8-bit transmission for binary files when the systems per-

mit.

- 7 -

3. System Requirements

The KERMIT protocol requires that:

- The  host can send and receive characters using 7- or 8-bit ASCII en-

coding over an EIA RS-232 physical connection,  either  hardwired  or

dialup.

- All  printable  ASCII  characters are acceptable as input to the host

3

and will not be transformed in any way .  Similarly, any  intervening

network or communications equipment (TELENET, terminal concentrators,

port  selectors, etc) must not transform or swallow any printable AS-

CII characters.

- A single ASCII control character can pass  from  one  system  to  the

other  without  transformation.    This  character is used for packet

synchronization.  The character is normally Control-A (SOH, ASCII 1),

but can be redefined.

- If a host requires a line terminator for terminal  input,  that  ter-

minator  must  be a single ASCII control character, such as CR or LF,

distinct from the packet synchronization character.

- When using a job's controlling terminal for file transfer, the system

must allow the KERMIT program to set the terminal to half duplex, in-

finite width (no "wraparound" or  CRLF  insertion  by  the  operating

system),  and no "formatting" of incoming or outgoing characters (for

instance, raising lowercase letters to uppercase,  transforming  con-

trol characters to printable sequences, etc).  In short, the terminal

must  be  put in "binary" or "raw" mode, and, hopefully, restored af-

terwards to normal operation.

- The host's terminal input processor should be capable of receiving  a

single  burst  of 40 to 100 characters at normal transmission speeds.

This is the typical size of packet.

- If a host requires padding, the padding character must be  a  control

character (including DEL).

KERMIT does not require:

- That the connection run at any particular baud rate.

- That  the  system  can do XON/XOFF or any other kind of flow control.

System- or hardware-level flow control can help, but it's not  neces-

sary.  See section 7.6.

- That  the system is capable of full duplex operation.  Any mixture of

_______________

3

If they are translated to another character set,  like  EBCDIC,  the  KERMIT

program must be able to reconstruct the packet as it appeared on the communica-

tion line, before transformation.

- 8 -

half and full duplex systems is supported.

- That  the  system  can  transmit or receive 8-bit bytes.  KERMIT will

take advantage of 8-bit connections to send binary files; if an 8-bit

connection is not possible, then binary files may be  sent  using  an

optional prefix encoding.

- 9 -

4. Printable Text versus Binary Data

For  transmission  between  unlike systems, files must be assigned to either of

two catagories: printable text or binary.

A printable text file is one that can make sense on an unlike system -- a docu-

ment, program source, textual data, etc.  A binary file is one  that  will  not

(and probably can not) make sense on an unlike system -- an executable program,

numbers stored in internal format, etc.  On systems with 8-bit bytes, printable

4

ASCII files will have the high order bit of each byte set to zero  (since ASCII

is  a 7-bit code) whereas binary files will use the high order bit of each byte

for data, in which case its value can vary from byte to byte.

Many computers have no way to distinguish a printable file from a  binary  file

--  especially one originating from an unlike system -- so the user may have to

give an explicit command to Kermit to tell it whether to perform these  conver-

sions.

4.1. Printable Text Files

A primary goal of KERMIT is for printable text files to be useful on the target

system after transfer.  This requires a standard representation for text during

transmission.    KERMIT's  standard  is simple: ASCII characters, with "logical

records" (lines) delimited by CRLFs.  It is the responsibility of systems  that

do  not  store printable files in this fashion to perform the necessary conver-

sions upon input and output.  For instance, IBM mainframes might strip trailing

blanks on output and add them back on input; UNIX would prepend  a  CR  to  its

normal record terminator, LF, upon output and discard it upon input.

This  representation is chosen because it corresponds to the way text files are

stored on many systems.  In many common cases, no  transformations  are  neces-

sary.

4.2. Binary Files

Binary files are transmitted as though they were a sequence of characters.  The

difference  from  printable  files  is that the status of the "8th bit" must be

preserved.  When binary files are transmitted to an unlike system, the main ob-

jective is that they can be brought back to the original system  (or  one  like

it)  intact;  no special conversions should be done during transmission, except

to make the data fit the transmission medium.

For binary files, eight bit character transmission is permissible  as  long  as

the  two  Kermit programs involved can control the value of the parity bit, and

no intervening communications equipment will change its value.  In  that  case,

the  8th  bit  of  a transmitted character will match that of the original data

_______________

4

There are some exceptions, such as systems that  store  text  files  in  so-

called "negative ASCII", or text files produced by word processors that use the

high order bit to indicate underline or boldface attributes.

- 10 -

byte,  after control-quoting has been done.  When one or both sides cannot con-

trol the parity bit, a special prefix character may be prepended, as  described

below.

Systems that do not store binary data in 8-bit bytes, or whose word size is not

a  multiple  of 8, may make special provisions for "image mode" transfer of bi-

nary files.  This may be done within the basic protocol by having the two sides

implicitly agree upon a scheme for packing the data  into  7-  or  8-bit  ASCII

characters,  or  else  the more flexible (but optional) file attributes feature

may be used.

- 11 -

5. Packet Format

The  KERMIT  protocol is built around exchange of packets of the following for-

mat:

+------+-----------+-----------+------+------------+-------+

| MARK | char(LEN) | char(SEQ) | TYPE |    DATA    | CHECK |

+------+-----------+-----------+------+------------+-------+

where all fields consist of ASCII characters.  The fields are:

MARK    The synchronization character that marks the beginning of  the  packet.

This should normally be CTRL-A, but may be redefined.

LEN     The  number  of  ASCII  characters  within  the packet that follow this

field, in other words the packet length minus two.  Since  this  number

is  transformed  to  a single character via the char() function, packet

character counts of 0 to 94 (decimal) are permitted, and  96  (decimal)

is  the  maximum total packet length.  The length does not include end-

of-line or padding characters, which are outside  the  packet  and  are

strictly  for  the benefit of the operating system, but it does include

the block check characters.

SEQ     The packet sequence number, modulo 64, ranging from 0 to 63.   Sequence

numbers "wrap around" to 0 after each group of 64 packets.

TYPE    The  packet type, a single ASCII character.  The following packet types

are required:

D   Data packet

Y   Acknowledge (ACK)

N   Negative acknowledge (NAK)

S   Send initiate (exchange parameters)

B   Break transmission (EOT)

F   File header

Z   End of file (EOF)

E   Error

DATA    The "contents" of the packet, if any contents are required in the given

type of packet, interpreted according to  the  packet  type.    Control

characters  are  preceded  by a special prefix character, normally "#",

and "uncontrollified" via ctl().  A prefixed sequence may not be broken

across packets.  Logical records in printable files are delimited  with

CRLFs,  suitably prefixed (e.g. "#M#J").  Any prefix characters are in-

cluded in the count.  Optional encoding for  8-bit  data  and  repeated

characters is described later.

CHECK    A block check on the characters in the packet between, but not includ-

ing, the mark and the block check itself.  The check for each packet is

computed  by  both hosts, and must agree if a packet is to be accepted.

A single-character arithmetic checksum is the normal and required block

check.  Only six bits of the arithmetic sum are  included.    In  order

that  all  the bits of each data character contribute to this quantity,

bits 6 and 7 of the final value are added to  the  quantity  formed  by

bits  0-5.    Thus  if s is the arithmetic sum of the ASCII characters,

then

- 12 -

check = char((s + ((s AND 192)/64)) AND 63)

This  is  the  default  block check, and all Kermits must be capable of

performing it.  Other optional block check types are described later.

The block check is based on the ASCII values of the characters  in  the

packet.    Non-ASCII  systems must translate to ASCII before performing

the block check calculation.

Any line terminator that is required by the  system  may  be  appended  to  the

packet;  this  is  carriage return (ASCII 15) by default.  Line terminators are

not considered part of the packet, and are included for in the count or  check-

sum.    Terminators are not necessary to the protocol, and are invisible to it,

as are any characters that may appear between packets.  If  a  host  cannot  do

single character input from a TTY line, then a terminator will be required when

sending  to  that host.  The terminator can be specified in the initial connec-

tion exchange.

- 13 -

6. Initial Connection

Initial connection occurs when the user has started up a Kermit program on both

ends  of  the physical connection.  One Kermit has been directed (in one way or

another) to send a file, and the other to receive it.

The receiving Kermit waits for a "Send-Init" packet from  the  sending  Kermit.

It  doesn't  matter  whether  the sending Kermit is started before or after the

receiving Kermit (if before,  the  Send-Init  packet  should  be  retransmitted

periodically  until  the  receiving Kermit acknowledges it).  The data field of

the Send-Init packet is optional; trailing  fields  can  be  omitted  (or  left

blank, i.e. contain a space) to accept or specify default values.

It  is  important to note that newly invented fields are added at the right, so

that old KERMIT programs that do not have code to handle the  new  fields  will

act  as  if  they  were  not there.  For this reason, the default value for any

field, indicated by blank, should result in the behavior that  occurred  before

the field was added.

1      2      3      4      5      6      7      8      9      10...

+------+------+------+------+------+------+------+------+------+-------

| MAXL | TIME | NPAD | PADC | EOL  | QCTL | QBIN | CHKT | REPT | CAPAS

+------+------+------+------+------+------+------+------+------+-------

The  fields  are as follows (the first and second person "I" and "you" are used

to distinguish the two sides).  Fields are encoded printably using  the  char()

function unless indicated otherwise.

1. MAXL   The  maximum  length  packet  I  want  to  receive, a number up to 94

(decimal).  You respond with the maximum you want me to send.    This

allows systems to adjust to each other's buffer sizes, or to the con-

dition of the transmission medium.

2. TIME   The  number  of  seconds  after which I want you to time me out while

waiting for a packet from me.  You respond with the amount of time  I

should  wait  for packets from you.  This allows the two sides to ac-

commodate to different line speeds or other factors that could  cause

timing problems.

3. NPAD   The  number  of  padding  characters  I want to precede each incoming

packet; you respond in kind.  Padding may be necessary  when  sending

to  a half duplex system that requires some time to change the direc-

tion of transmission, although in practice  this  situation  is  more

commonly handled by a "handshake" mechanism.

4. PADC   The  control  character  I  need  for padding, if any, transformed by

ctl() (not char()) to make it printable.  You respond in kind.   Nor-

mally NUL (ASCII 0), some systems use DEL (ASCII 127).  This field is

ignored if the value NPAD is zero.

5. EOL    The  character  I  need to terminate an incoming packet, if any.  You

respond in kind.  Most systems that require  a  line  terminator  for

terminal  input  accept  carriage  return  for this purpose (note, it

would have been better to use ctl() for this field, but it's too late

now).

- 14 -

6. QCTL   (verbatim)  The printable ASCII character I will use to quote control

characters, normally "#".  You respond with the one you will use.

7. QBIN   (verbatim) The printable ASCII character  I  want  to  use  to  quote

characters  which have the 8th bit set, for transmitting binary files

when the parity bit cannot be used for data.    Since  this  kind  of

quoting  increases  both  processor  and transmission overhead, it is

normally to be avoided.  If used, the quote character must be in  the

range  ASCII 33-62 ("!" through ">") or 96-126 ("`" through "~"), but

different from the control-quoting character.  This field  is  inter-

preted as follows:

Y   I agree to 8-bit quoting if you request it.

N   I will not do 8-bit quoting.

&   (or  any  other  character in the range 33-62 or 96-126 besides Y

and N) I want to do 8-bit quoting using this character  (it  will

be  done if the other Kermit puts a Y in this field).  The recom-

mended 8th-bit quoting prefix character is "&".

Anything Else: 8-bit quoting will not be done.

8. CHKT   Check Type, the method for detecting errors.  "1" for  single-charac-

ter  checksum (the normal and required method), "2" for two-character

checksum (optional), "3" for  three-character  CRC-CCITT  (optional).

If  your  response agrees, the designated method will be used; other-

wise the single-character checksum will be used.

9. REPT   The prefix character I will use to  indicate  a  repeated  character.

This  can  be  any  printable  character  in the range ASCII 33-62 or

96-126, but different from the control and 8th-bit prefixes.  SP (32)

denotes no repeat count processing is to be done.  Tilde ("~") is the

recommended and normal repeat prefix.  If  you  don't  respond  iden-

tically,  repeat  counts will not be done.  Groups of at least 3 or 4

identical characters may be  transmitted  more  efficiently  using  a

repeat  count,  though an individual implementation may wish to set a

different threshhold.

10-?. CAPAS

A bit mask, in which each bit position corresponds to a capability of

KERMIT, and is set to 1 if that capability is present, or 0 if it  is

not.   Each character contains a 6-bit field (transformed by CHAR()),

whose low order bit is set to 1 if another capability  byte  follows,

and  to  0  in the last capability byte.  The capabilities defined so

far are:

#1  Ability to time out.

#2  Ability to accept server commands.

#3  Ability to accept "A" packets (file attributes)

The capability byte as defined so far would then look like:

bit5 bit4 bit3 bit2 bit1 bit0

+----+----+----+----+----+----+

| #1 | #2 | #3 | -- | -- |  0 |

+----+----+----+----+----+----+

If all these capabilities were "on", the value of the byte  would  be

- 15 -

70 (octal).  When capabilities 5 and 6 are added, the capability mask

will look like this:

bit5 bit4 bit3 bit2 bit1 bit0    bit5 bit4 bit3 bit2 bit1 bit0

+----+----+----+----+----+----+   +----+----+----+----+----+----+

| #1 | #2 | #3 | #4 | #5 |  1 |   | #6 | -- | -- | -- | -- |  0 |

+----+----+----+----+----+----+   +----+----+----+----+----+----+

Next 4: Reserved Fields

Sites that wish to add their own parameters to the initial connection

negotiation  must  start  at  the 5th field after the last capability

byte.  Any intervening fields may be left blank (that  is,  they  may

contain  the  space character).  These fields are reserved for future

use by the standard KERMIT protocol.

The control, 8th-bit, and repeat prefixes must be distinct.

The receiving Kermit responds with an ACK ("Y") packet in the  same  format  to

indicate  its  own preferences, options, and parameters.  The ACK need not con-

tain the same number of fields as the the Send-Init.  From that point, the  two

KERMIT  programs  are  "configured"  to  communicate  with  each  other for the

remainder of the transaction.  In the case of 8th-bit quoting,  one  side  must

specify  the  character  to be used, and the other must agree with a "Y" in the

same field, but the order in which this occurs does not matter.  Similarly  for

checksums  --  if  one  side  requests 2 character checksums and the other side

responds with a "1" or with nothing at  all,  then  single-character  checksums

will  be  done, since not all implementations can be expected to do 2-character

checksums or CRCs.  And for repeat counts; if the repeat field of the send-init

and the ACK do not agree, repeat processing will not be done.

All Send-Init fields are optional.  The data field may be left  totally  empty.

Similarly,  intervening fields may be defaulted by setting them to blank.  Ker-

mit implementations should know what to do in these  cases,  namely  apply  ap-

propriate defaults.  The defaults should be:

MAXL:   80

NPAD:   0, no padding

PADC:   0 (NUL)

EOL:    CR (carriage return)

QCTL:   the character "#"

QBIN:   none, don't do 8-bit quoting

CHKT:   "1", single-character checksum

REPT:   No repeat count processing

MASK:   All zeros (no special capabilities)

Note  that  there  are  no prolonged negotiations in the initial connection se-

quence -- there is one Send-Init and one ACK in  reply.    Everything  must  be

settled in this exchange.

The  very first Send-Init may not get through if the sending Kermit makes wrong

assumptions about the receiving host.  For instance, the receiving host may re-

quire certain parity, some padding, or a special end of line character in order

to read the Send-Init packet.  For this reason, there should be a way  for  the

user  the  user  to  specify  whatever may be necessary to get the first packet

through.

- 16 -

A  parity field is not provided in the Send-Init packet because it could not be

of use.  If the sender requires a certain kind of parity, it will also be send-

ing it.  If the receiver does not know this in advance, i.e. before getting the

Send-Init, it will not be able to read the Send-Init packet.

- 17 -

7. File Transfer

The file transfer protocol takes place over a transaction.  A transaction is an

exchange  of  packets  beginning with a Send-Init (S) packet, and ending with a

5

Break Transmission (B) or Error (E) packet , and may include  the  transfer  of

one  or  more files, all in the same direction.  The following sequence charac-

terizes basic Kermit operation; the sender  is  the  machine  that  is  sending

files; the receiver is the machine receiving the files.

1. The  sender  transmits  a  Send-Initiate  (S)  packet to specify its

parameters (packet length, timeout, etc).

2. The receiver sends an ACK (Y) packet, with its own parameters in the

data field.

3. The sender transmits a File-Header (F) packet,  which  contains  the

file's name in the data field.  The receiver ACKs the F packet, with

no data in the data field of the ACK (optionally, it may contain the

name under which the receiver will store the file under).

4. The  sender  sends  the  contents  of the file, in Data (D) packets.

Each D packet must be acknowledged before the next one is sent.

5. When all the file data has been sent, the sender  sends  an  End-Of-

File (Z) packet.  The receiver ACKs it.

6. If there is another file to be sent, the process is repeated, begin-

ning at step 3.

7. When  no  more files remain to be sent, the sender transmits an End-

Of-Transmission (B) packet.  The receiver ACKs it.   This  ends  the

transaction, and closes the logical connection (the physical connec-

tion remains open).

Each packet has a sequence number, starting with 0.  The acknowledgment (ACK or

NAK) for a packet has the same packet number as the packet it is acknowledging.

Once  an  acknowledgment  is  successfully  received, the sender increments the

packet number by one, modulo 64.

If the sender is remote, it waits for a certain amount of  time  (somewhere  in

the 5-30 second range) before transmitting the Send-Init, to give the user time

to escape back to the local KERMIT and tell it to receive files.

_______________

5

A  transaction  should  also  be  considered terminated when one side or the

other has stopped without sending an Error packet.

- 18 -

7.1. Timeouts, NAKs, and Retries

If  a KERMIT program is capable of setting a timer interrupt, or setting a time

limit on an input request, it should do so whenever attempting to read a packet

from the communication line, whether sending or receiving files.  Having read a

packet, it should turn off the timer.

If the sender times out waiting for an acknowledgement, it should send the same

packet again, repeating the process a certain number of times  up  to  a  retry

limit,  or  until  an  acknowledgement  is received.  If the receiver times out

waiting for a packet, it can send either a NAK packet for the  expected  packet

or another ACK for the last packet it got.

If  a  packet from the sender is garbled or lost in transmission (the latter is

detected when the sequence number increases by more  than  1,  modulo  64,  the

former  by a bad checksum), the receiver sends a NAK for the garbled or missing

packet.  If an ACK or a NAK from the receiver is garbled or  lost,  the  sender

ignores it; in that case, one side or the other will time out and retransmit.

A  retry  count  is maintained, and there is a retry threshold, normally set at

about 5.  Whenever a packet is resent -- because of a timeout,  or  because  it

was  NAK'd  --  the counter is incremented.  When it reaches the threshold, the

transaction is terminated.

If neither side is capable of timing out, a facility  for  manual  intervention

must  be  available on the local KERMIT.  Typically, this will work by sampling

the keyboard (console) periodically; if input, such as a CR, appears, then  the

same  action  is  taken as if a timeout had occurred.  The local KERMIT keeps a

running display of the packet number or byte count on the screen to  allow  the

user  to  detect  when traffic has stopped.  At this point, manual intervention

should break the deadlock.

Shared systems which can become sluggish when heavily used should adjust  their

own  timeout intervals on a per-packet basis, based on the system load, so that

file transfers won't fail simply because the system was too slow.

7.2. Errors

During file transfer, the sender may encounter an i/o error on the disk, or the

receiver may attempt to write to a full or write-protected device.    Any  con-

dition that will prevent successful transmission of the file is called a "fatal

error".    Fatal  errors  should be detected, and the transfer shut down grace-

fully, with the pertinent information provided to  the  user.    Error  packets

provide a mechanism to do this.

If  a fatal error takes place on either the sending or receiving side, the side

which encountered the error should send an Error (E) packet.  The E packet con-

tains a brief English error message in the data field.   Both  the  sender  and

receiver  should  be prepared to receive an Error packet at any time during the

transaction.  Both the sender and receiver of the Error packet should halt,  or

go  back  into into user command mode.  The side that is local should print the

error message on the screen.

- 19 -

7.3. Heuristics

A few heuristics are useful:

1. A  NAK  for  the current packet is equivalent to an ACK for the pre-

vious packet.  This covers the common situation in which a packet is

successfully received, and then ACK'd, but the ACK  is  lost.    The

ACKing  side then times out waiting for the next packet and NAKs it.

The side that receives a NAK for packet n+1 while waiting for an ACK

for packet n simply sends packet n+1.

2. If packet n arrives more than once, simply ACK it  and  discard  it.

This  can  happen when the first ACK was lost.  Resending the ACK is

necessary and sufficient -- don't write the packet out to  the  file

again!

3. When  opening a connection, discard the contents of the line's input

buffer before reading or sending the first packet.   This  is  espe-

cially  important if the other side is in receive mode (or acting as

a server), in which case it may have been sending out periodic  NAKs

for  your  expected  SEND-INIT  or  command packet.  If you don't do

this, you may find that there are sufficient  NAKs  to  prevent  the

transfer -- you send a Send-Init, read the response, which is an old

NAK,  so  you  send another Send-Init, read the next old NAK, and so

forth, up to the retransmission limit, and give up before getting to

the ACKs that are waiting in line behind all the old NAKs.   If  the

number  of  NAKs is below the cutoff, then each packet may be trans-

mitted multiply.

4. Similarly, after reading a packet (successfully or not), you  should

clear  the  input  buffer.    There  should be nothing there for you

anyway, since the other side must normally wait for you to send your

packet in response.  Failure to clear the  buffer  could  result  in

propogation of the repetition of a packet caused by stacked-up NAKs.

7.4. File Names

The  syntax  for  file  names  can vary widely from system to system.  To avoid

problems, it is suggested that filenames be represented in the File Header  (F)

packet  in  a  "normal form", by default (that is, there should be an option to

override such conversions).

1. Delete all pathnames and attributes  from  the  file  specification.

The file header packet should not contain directory or device names;

if  it  does, it may cause the recipient to try to store the file in

an inaccessible or nonexistent area, or it  may  result  in  a  very

strange filename.

2. After  stripping  any  pathname,  convert  the remainder of the file

specification to the form "name.type", with no restriction on length

(except that it fit in the data field of the F packet), and:

a. Include no more than one dot.

b. Use digits, uppercase letters only in name and type.

- 20 -

Special  characters like "$", "_", "-", "&", and so forth should be disallowed,

since they're sure to cause problems on one system or another.

The recipient, of course, cannot depend upon the sender to follow this  conven-

tion,  and should still take precautions.  However, since most file systems em-

body the notion of a file name and a file  type,  this  convention  will  allow

these items to be expressed in a way that an unlike system can understand.  The

particular notation is chosen simply because it is the most common.

The  recipient  must  worry about the length of the name and type fields of the

file name.  If either is too long, they must  be  truncated.    If  the  result

(whether  truncated  or not) is the same as the name of a file that already ex-

ists in the same area, the recipient should have the ability to take some  spe-

cial action to avoid writing over the original file.

KERMIT  implementations  that  convert  file  specifications  to normal form by

default should have an option to override this feature.   This  would  be  most

useful  when  transferring files between like systems, perhaps used in conjunc-

tion with "image mode" file transfer.  This could allow, for instance, one UNIX

system to send an entire directory tree to another UNIX system.

7.5. Robustness

A major feature of the KERMIT protocol is  the  ability  to  transfer  multiple

files.    Whether  a particular KERMIT program can actually send multiple files

depends on the capabilities of the program and the host operating  system  (any

KERMIT program can receive multiple files).

If  a  KERMIT  program can send multiple files, it should make every attempt to

send the entire group specified.  If it fails to send  a  particular  file,  it

should  not  terminate the entire batch, but should go on the the next one, and

proceed until an attempt has been made to send each file in the group.

Operating in this robust manner, however, gives rise to  a  problem:  the  user

must  be  notified of a failure to send any particular file.  Unfortunately, it

is not sufficient to print a message to the screen;  users  who  initiate  long

batch  transfers  may have gone out to lunch and such messages may be long gone

by the time they return.  Furthermore, there is no way for a remote  sender  to

inform  the  local  receiver  of  such an error without killing the whole batch

transfer, since Error (E) packets always cause the transaction to terminate.

One solution might be to invent a non-fatal warning or general purpose  display

message packet, but this would still not help the user who was not there to see

it.  A better solution would be to have the sender optionally keep a log of the

transaction,  giving the name of each file for which an attempt was made, and

stating whether the attempt was successful, and if not, the reason.

- 21 -

7.6. Flow Control

On full duplex connections, XON/XOFF flow control can generally be used in con-

junction  with KERMIT file transfer with no ill effects.  This is because XOFFs

are sent in the opposite direction of packet flow, so they will  not  interfere

with  the  packets themselves.  XON/XOFF, therefore, need not be implemented by

the KERMIT program, but can done by the  host  system.    If  the  host  system

provides  this  capability,  it  should  be  used  -- if both sides can respond

XON/XOFF  signals,  then  buffer  overruns  and  the  resulting  costly  packet

retransmissions can be avoided.

Flow control via modem signals can also be used when available.

- 22 -

8. Basic KERMIT Protocol State Table

The  KERMIT  protocol  can be described as a set of states and transitions, and

rules for what to do when changing from one state to another.    State  changes

occur  based  on  the type of packets that are sent or received, or errors that

may occur.  Packets always go back and forth; the sender of a file always sends

data packets of some kind (init, header, data) and the receiver always  returns

ACK  or  NAK packets.  Timeouts and error conditions have been omitted from the

following state table for simplicity, but the action is as described above.

Upon entering a given state, a certain kind of packet is either being  sent  or

is expected to arrive -- this is shown on top of the description of that state.

As  a  result  of the action, various events may take place; these are shown in

the EVENT column.  For each event, an appropriate  ACTION  is  taken,  and  the

protocol enters a NEW STATE.

- 23 -

STATE   EVENT           ACTION                      NEW STATE

-- SEND STATES --

Send Send-Init Packet

S       Get NAK,bad ACK (None)                          S

Get good ACK    Set remote's params, open file  SF

(Other)         (None)                          A

Send File-Header Packet

SF      Get NAK,bad ACK (None)                          SF

Get good ACK    Get bufferful of file data      SD

(Other)         (None)                          A

Send File-Data Packet

SD      Get NAK,bad ACK (None)                          SD

Get good ACK    Get bufferful of file data      SD

(End of file)   (None)                          SZ

(Other)         (None)                          A

Send EOF Packet

SZ      Get NAK,bad ACK (None)                          SZ

Get good ACK    Get next file to send           SF

(No more files) (None)                          SB

(Other)         (None)                          A

Send Break (EOT) Packet

SB      Get NAK,bad ACK (None)                          SB

Get good ACK    (None)                          C

(Other)         (None)                          A

-- RECEIVE STATES --

Wait for Send-Init Packet

R       Get Send-Init   ACK w/local params              RF

(Other)         (None)                          A

Wait for File-Header Packet

RF      Get Send-Init   ACK w/local params

(previous ACK was lost)        RF

Get Send-EOF    ACK (prev ACK lost)             RF

Get Break       ACK                             C

Get File-Header Open file, ACK                  RD

(Other)         (None)                          A

Wait for File-Data Packet

RD      Get previous

packet(D,F)    ACK it again                    RD

Get EOF         ACK it, close the file          RF

Get good data   Write to file, ACK              RD

(Other)         (None)                          A

- 24 -

-- STATES COMMON TO SENDING AND RECEIVING --

C       (Send Complete)                                 start

A       ("Abort")                                       start

- 25 -

9. Optional Features

The foregoing sections have discussed basic, required operations for any KERMIT

implementation.  The following sections discuss optional features.

9.1. 8th-Bit and Repeat Count Prefixing

Prefix  quoting of control characters is mandatory.  In addition, prefixing may

also be used for 8-bit quantities or repeat counts, when both  KERMIT  programs

agree  to  do  so.   8th-bit prefixing can allow 8-bit binary data pass through

7-bit physical links.  Repeat count prefixing can  improve  the  throughput  of

certain  kinds  of  files  dramatically;  binary files (particularly executable

programs) and structured text (highly indented or columnar text) tend to be the

major beneficiaries.

When more than one type of prefixing is in effect, a single data character  can

be  preceded  by  more than one prefix character.  A receiver will never do any

kind of prefixing, since prefixing can only occur in the data  field,  and  the

receiver  only  responds  with ACKs and NAKs with empty or special data fields.

Repeat count processing can only be requested by the sender, and will  only  be

used by the sender if the receiver agrees.  8th-bit prefixing is a special case

because  it  is  normally  not  desirable  to  use  it, since it increases both

processing  and  transmission  overhead.    However,  since  it  is  the   only

straightforward  mechanism  for binary file transfer available to those systems

that usurp the parity bit, a receiver must be able to request the sender to  do

8th-bit quoting, since most senders will not normally do it by default.

The  repeat  prefix is followed immediately by a single-character repeat count,

encoded printably  via  char(),  followed  by  the  character  itself  (perhaps

prefixed  by  control or 8th bit quotes, as explained below).  The repeat count

may express values from 0 to 94.  If a character appears more than 94 times  in

a  row,  it must be "cut off" at 94, emitted with all appropriate prefixes, and

"restarted".  The following table should  clarify  Kermit's  quoting  mechanism

(the final line shows how a sequence of 120 consecutive NULs would be encoded):

Quoted             With

Character    Representation     Repeat Count for 6

A            A                ~(A   ["(" is ASCII 40 - 32 = 6]

^A           #A               ~(#A

'A           &A               ~(&A

'^A          &#A              ~(&#A

#            ##               ~(##

'#           &##              ~(&##

&            #&               ~(#&

'&           &#&              ~(&#&

~            #~               ~(#~

'~           &#~              ~(&#~

NUL          #@               ~~#@~:#@ [120 NULs]

A  represents  any printable character, ^A represents any control character, 'x

represents any character with the 8th bit set.  The #  character  is  used  for

control-character  quoting,  and the & character for 8-bit quoting.  The repeat

count must always precede any other prefix character.    The  repeat  count  is

taken literally (after transformation by unchar(); for instance "#" and "&" im-

mediately following a "~" denote repeat counts, not control characters or 8-bit

- 26 -

characters.   The control quote character "#" is most closely bound to the data

character, then the 8-bit prefix, then the repeat count; in  other  words,  the

order  should  be: repeat prefix and count, 8-bit quote, control quote, and the

data character itself.  To illustrate, note that &#A is not equivalent to  #&A.

And finally, remember:

- Prefixed sequences must not be broken across packets.

- Control, 8th-bit, and repeat count prefixes must be distinct.

9.2. Server Operation

A  KERMIT server is a KERMIT program running remotely with no "user interface".

All commands to the server arrive in packets from the  local  KERMIT.    SERVER

operation  is  much  more  convenient than basic operation, since the user need

never again interact directly with the remote KERMIT program after starting  it

up  in server mode, and therefore need not issue complementary SEND and RECEIVE

commands on the two sides to get a file transfer started; rather, a single com-

mand (SEND or GET) to the local KERMIT  suffices.    KERMIT  servers  can  also

provide services beyond file transfer.

Between  transactions, a Kermit server waits for packets containing server com-

mands.  The packet sequence number is always set back to 0 after a transaction.

A Kermit server in command wait should be looking for  packet  0,  and  command

packets  sent to servers should also be packet 0.  Certain server commands will

result in the exchange of multiple packets.  Those operations  proceed  exactly

like file transfer.

Server  operation  must be implemented in two places: in the server itself, and

in any KERMIT program that will be communicating with a  server.    The  server

must  have  code  to read the server commands from packets and respond to them.

The user KERMIT must have code to parse commands to send requests  to  servers,

to form the server command packets, and to handle the responses to those server

commands.

Server  commands  are listed below.  Not all of them have been implemented, and

some may never be, but their use should  be  reserved.    Although  server-mode

operation  is optional, certain commands should be implemented in every server.

These include Send-Init (S), Receive-Init (R),  and  the  Generic  Logout  (GL)

and/or  Finish (GF) commands.  If the server receives a command it does not un-

derstand, or cannot execute, it should respond with an Error  (E)  packet  con-

taining a message like "Unimplemented Server Command" and both sides should set

the  packet  sequence  number back to 0, but the server should remain in server

command wait.  Only a GL or GF command should terminate server operation.

Server commands are as follows:

S   Send Initiate (exchange parameters, server waits for a file).

R   Receive Initiate (ask the server to send the specified files).

I   Initialize (exchange parameters).

C   Host Command.  The data field contains a string to be executed as a command

by the host system command processor.

G   Generic Kermit Command.  Single character in data field (possibly  followed

by  operands,  shown  in {braces}, optional fields in [brackets]) specifies

the command:

- 27 -

I   Login [{~user[~password[~account]]}]

C   CWD, Change Working Directory [{~directory[~password]}]

L   Logout, Bye

F   Finish (Shut down the server, but don't logout).

D   Directory [{~filespec}]

U   Disk Usage Query [{~area}]

E   Erase (delete) {~filespec}

T   Type {~filespec}

R   Rename {~oldname~newname}

K   Copy {~source~destination}

W   Who's logged in? (Finger) [{~user ID or network host[~options]}]

M   Send a short Message {~destination~text}

H   Help [{~topic}]

Q   Server Status Query

Tilde  as used above ("~") is a length field, encoded using char(), for the

6

operand that follows it.

X   Text header.  Allows transfer of text to the user's screen in response to a

generic or host command.  This works just like file  transfer  except  that

the destination "device" is the screen rather than a file.

KERMIT  does not provide a mechanism for suspending and continuing a trans-

action.  This means that text sent to  the  user's  screen  should  not  be

frozen  for long periods (i.e. not longer than the timeout period times the

retry threshold).

Between transactions, when the server has no tasks pending,  it  may  send  out

periodic  NAKs  (always  with type 1 checksums) to prevent a deadlock in case a

command was sent to it but was lost.  These NAKs  can  pile  up  in  the  local

"user"  Kermit's  input  buffer  (if  it has one), so the user Kermit should be

prepared to clear its input buffer before sending a command to a server.

9.2.1. The R Command

The R packet tells the server to send the files specified by the  name  in  the

data  field  of  the  R packet.  Since we can't assume that the two Kermits are

running on like systems, the local (user) Kermit must parse the file specifica-

tion as a character string and let the server to check it.  If the server likes

the filespec, it sends a Send-Init (S) packet -- not an acknowledgement! --  to

the user, and then completes the file-sending transaction, as described above.

If the server cannot send the file, it should respond with an error packet con-

taining a reason, like "File not found" or "Read access required".

_______________

6

This  is  an  incompatible  change from previous editions of this manual, in

which the tilde was inserted literally.

- 28 -

9.2.2. Short and Long Replies

Any  request  made  of  a server may be answered in either of two ways, and any

User Kermit that makes such a request should be prepared  for  either  kind  of

reply:

- A  short reply.  This consists of a single ACK packet, which may con-

tain text in its data field.  For instance, the  user  might  send  a

disk  space query to the server, and the server might ACK the request

with a short character string in the data field, such as  "12K  bytes

free".  The user KERMIT should display this text on the screen.

- A  long  reply.    This proceeds exactly like a file transfer (and in

some cases it may be a file transfer).    It  begins  with  either  a

File-Header (F) packet (optionally followed by one or more Attributes7

packets                                                               ), or a Text-Header (X) packet.  Then comes an arbitrary num-

ber  of Data (D) packets, then an End-Of-File (Z) packet, and finally

a Break-Transmission (B) packet.

9.2.3. Additional Server Commands

The following server commands request the server to perform  tasks  other  than

sending  or receiving files.  Almost any of these can have either short or long

replies.  For instance, the Generic Erase (GE) command may elicit a simple ACK,

or a stream of packets containing the names of all  the  files  it  erased  (or

didn't  erase).  These commands are now described in more detail; arguments are

as provided in commands typed to the user KERMIT;  no  transformations  to  any

kind  of normal or canonic form are done -- filenames and other operands are in

the syntax of the server's host system.

I   Login.  For use when a KERMIT server is kept perpetually running on a dedi-

cated line.  This lets a new user obtain an identity on the  server's  host

system.    If the data field is empty, this removes the user's identity, so

that the next user does not get access to it.  A dedicated server  such  as

this should probably disallow the L and F generic commands, listed below.

L   Logout,  Bye.   This shuts down the server entirely, causing the server it-

self to log out its own job.  This is for use  when  the  server  has  been

started up manually by the user, who then wishes to shut it down remotely.

F   Finish.    This  is  to allow the user to shut down the server, putting its

terminal back into normal (as opposed to binary or raw) mode,  and  putting

the server's job back at system command level, still logged in, so that the

user can connect back to the job.

C   CWD.    This  sets  the  default directory or area for file transfer on the

server's host.  With no operands, this command sets the default area to  be

the user's own default area.

_______________

7

See section 9.5 for a description of the Attributes packet.

- 29 -

D   Directory.    Send  a  directory listing to the user.  The user program can

display it on the terminal or store it in a  file,  as  it  chooses.    The

directory  listing  should contain file sizes and creation dates as well as

file names, if possible.  A wildcard or other file-group designator may  be

specified  to  ask  the  server  list  only  those files that match.  If no

operand is given, all files in the current area will be shown.

U   Disk Usage Query.  The server responds with the amount of  space  used  and

the  amount  left  free to use, in K bytes (or other units, which should be

specified).

E   Erase (delete).  Delete the specified file or file group.

T   Type.  Send the specified file or file group, indicating (by starting  with

an  X  packet rather than an F packet, or else by using the Type attribute)

that the file is to be displayed on the screen, rather than stored.

R   Rename.  Change the name of the file as indicated.  The  string  indicating

the  new  name  may contain other attributes, such as protection code, per-

mitted by the host.

K   Copy.  Produce a new copy of the file or file group, as indicated,  leaving

the source file(s) unmodified.

W   Who's  logged  in? (Finger).  With no arguments, list all the users who are

logged in on the server's host  system.    If  an  argument  is  specified,

provide more detailed information on the specified user or network host.

M   Short  Message.    Send  the given short (single-packet) message to the in-

dicated user's screen.

9.2.4. Host Commands

Host commands are conceptually simple, but may be hard  to  implement  on  some

systems.  The C packet contains a text string in its data field which is simply

fed  to the server's host system command processor; any output from the proces-

sor is sent back to the user in KERMIT packets,  as  either  a  short  or  long

reply.

Implementation  of this facility under UNIX, with its forking process structure

and i/o redirection via pipes, is quite natural.  On other systems, it could be

virtually impossible.

9.2.5. Exchanging Parameters Before Server Commands

In basic KERMIT, the Send-Init exchange is always sufficient to  configure  the

two  sides  to  each  other.   During server operation, on the other hand, some

transactions may not begin with a Send-Init packet.   For  instance,  when  the

user  sends  an  R  packet to ask the server to send a file, the server chooses

what block check option to use.  Or if the user requests a  directory  listing,

the server does not know what packet length to use.

The solution to this problem is the "I" (Init-Info) packet.  It is exactly like

a Send-Init packet, and the ACK works the same way too.  However, receipt of an

- 30 -

I  packet  does not cause transition to file-send state.  The I-packet exchange

simply allows the two sides to set their parameters,  in  preparation  for  the

next transaction.

Servers  should  be  able to receive and ACK "I" packets when in server command

wait.  User KERMITs need not send "I" packets, however; in that case, the serv-

er will assume all the defaults for the user listed on page 15.

9.3. Alternate Block Check Types

There are two optional kinds of block checks:

Type 2

A two-character checksum based on the low order 12 bits of  the  arithmetic

sum  of  the  characters in the packet (from the LEN field through the last

data character, inclusive) as follows:

1              2

--------+--------------+-------------+

...data | char(b6-b11) | char(b0-b5) |

--------+--------------+-------------+

For instance, if the 16-bit result is 154321 (octal), then the 2  character

block check would be "C1".

Type 3

Three-character  16-bit  CRC-CCITT.  The CRC calculation treats the data it

operates upon as a string of bits with the  low  order  bit  of  the  first

character first and the high order bit of the last character last.  The in-

itial value of the CRC is taken as 0; the 16-bit CRC is the remainder after

16  12  5

dividing  the data bit string by the polynomial X  +X  +X +1 (this calcula-

tion can actually be done a character at  a  time,  using  a  simple  table

lookup algorithm).  The result is represented as three printable characters

at the end of the packet, as follows:

1               2              3

--------+---------------+--------------+-------------+

...data | char(b12-b15) | char(b6-b11) | char(b0-b5) |

--------+---------------+--------------+-------------+

For  instance, if the 16-bit result is 154321 (octal), then the 3 character

block check would be "-C1".

The single-character checksum has proven quite adequate in practice.  The other

options can be used only if both sides agree to do so via Init packet (S or  I)

exchange.    The  2 and 3 character block checks should only be used under con-

ditions of severe line noise and packet corruption.

Since type 2 and 3 block checks are optional, not all KERMITs can  be  expected

to  understand  them.  Therefore, during initial connection, communication must

begin using the type 1 block check.  If type 2 or 3 block checks are agreed  to

during  the  "I"  or  "S" packet exchange, the switch will occur only after the

Send-Init has been sent and ACK'd with a type 1 block check.  This  means  that

the  first  packet  with a type 2 or 3 block check must always be an "F" or "X"

- 31 -

packet.   Upon completion of a transaction, both sides must switch back to type

1. The transaction is over after a "B" or "E" packet has been sent  and  ACK'd,

or after any error that terminates the transaction prematurely or abnormally.

A  server  that  has  completed a transaction and is awaiting a new command may

send out periodic NAKs for that command (packet 0).  Those NAKs must have  type

1 block checks.

9.4. Interrupting a File Transfer

This  section  describes  an  optional  feature of the KERMIT protocol to allow

graceful interruption of file transfer.  This feature is  unrelated  to  server

operation.

To interrupt sending a file, send an EOF ("Z") packet in place of the next data

packet,  including  a  "D" (for Discard) in the data field.  The recipient ACKs

the Z packet normally, but does not retain the file.  This does  not  interfere

with  older  Kermits on the receiving end; they will not inspect the data field

and will close the file normally.  The mechanism can be triggered by typing  an

interrupt  character  at  the  console  of  the  sending  KERMIT program.  If a

(wildcard) file group is being sent, it is possible to skip to the next file or

to terminate the entire batch; the protocol is the same in either case, but the

desired action could be selected by different interrupt characters, e.g. CTRL-X

to skip the current file, CTRL-Z to skip the rest of the batch.

To interrupt receiving a file, put an "X" in the data field of  an  ACK  for  a

data packet.  To interrupt receiving an entire file group, use a "Z".  The user

could  trigger  this mechanism by typing an interrupt character by typing, say,

CTRL-X and CTRL-Z, respectively, at the receiving KERMIT's console.   A  sender

that  was  aware of the new feature, upon finding one of these codes, would act

as described above, i.e. send a "Z" packet with a "D" code; a sender  that  did

not  implement this feature would simply ignore the codes and continue sending.

In this case, and if the user wanted the whole batch to be cancelled  (or  only

one  file was being sent), the receiving KERMIT program, after determining that

the sender had ignored the "X" or "Z" code, could send an Error (E)  packet  to

stop the transfer.

The sender may also choose to send a Z packet containing the D code when the it

detects  that  the  file  it is sending cannot be sent correctly and completely

-- for instance, after sending some packets correctly, it  gets  an  i/o  error

reading the file.  Or, it notices that the "8th bit" of a file byte is set when

the file is being sent as a text file and no provision has been made for trans-

mitting the 8th bit.

9.5. Transmitting File Attributes

The  optional  Attributes  (A)  packet provides a mechanism for the sender of a

file to provide additional information about it.  This packet can  be  sent  if

the  receiver has indicated its ability to process it by setting the Attributes

bit in the capability mask.    If  both  sides  set  this  bit  in  the  Kermit

Capability  Word, then the sender, after sending the filename in the "F" packet

and receiving an acknowledgement, may (but does not have to) send an "A" packet

to provide file attribute information.

- 32 -

Setting the Attributes bit in the capability mask does not indicate support for

any particular attributes, only that the receiver is prepared to accept the "A"

packet.

The  attributes  are given in the data field of the "A" packet.  The data field

consists of 0 or more subfields, which may occur in any order.   Each  subfield

is of the following form:

+-----------+--------------+------+

| ATTRIBUTE | char(LENGTH) | DATA |

+-----------+--------------+------+

where

ATTRIBUTE is a single printable character other than space,

LENGTH    is  the  length  of  the  data characters (0 to 94), with 32 added to

produce a single printable character, and

DATA      is length characters worth of data, all printable characters.

No quoting or prefixing is done on any of this data.

More than one attribute packet may be sent.  The only requirement is  that  all

the A packets for a file must immediately follow its File header (or X) packet,

and precede the first Data packet.

There  may  be  93 different attributes, one for each of the 93 printable ASCII

characters other than space.  These are assigned in ASCII order.

! (ASCII 33)

Length.  The data field gives the length in  K  (1024)  bytes,  as  a

printable decimal number, e.g. "!#109".  This will allow the receiver

to  determine  in  advance  whether  there is sufficient room for the

file, and/or how long the transfer will take.

" (ASCII 34)

Type.  The data field can contain some indicator of the nature of the

file.  Note that only sequential files can be supported by the KERMIT

protocol.  Operands are  enclosed  in  {braces},  optional  items  in

[brackets].

A[{xx}] ASCII  text,  containing no 8-bit quantities, logical records

(lines) delimited by the (quoted) control character  sequence

{xx},  represented  here  by  its printable counterpart (MJ =

CRLF, J = LF, etc).  For instance  AMJ  means  that  the  ap-

pearance  of  #M#J  (the  normal prefixed CRLF sequence) in a

file data packet indicates the end of a record.  If  {xx}  is

omitted, MJ will be assumed.

B[{xx}] Binary.  {xx} indicates in what manner the file is binary:

8   (default)  The  file  is a sequence of 8-bit bytes, which

must be saved as is.  The 8th bit may be sent "bare",  or

prefixed  according  to  the  Send-Init negotiation about

8th-bit prefixing.

- 33 -

36  The  file  is  a PDP-10 format binary file, in which five

7-bit bytes are fit into one 36-bit word, with the  final

bit of each word being represented as the "parity bit" of

every 5th character (perhaps prefixed).

D{x}    Variable  length  undelimited  records.   Each logical record

begins with an {x}-character ASCII length field  (similar  to

ANSI tape format "D").

F{xxxx} Fixed-length  undelimited  records.    Each logical record is

{xxxx} bytes long.

I[{x}]  Image.  The file is being sent exactly as it  is  represented

on  the  system  of  origin.    For use between like systems.

There are {x} usable bits per  character,  before  prefixing.

For  instance,  to  send binary data from a system with 9-bit

bytes, it might be convenient to send three 6-bit  characters

for every two 9-bit bytes.  Default {x} is 8.

# (ASCII 35)

Creation  Date,  expressed as "[yy]yymmdd[ hh:mm[:ss]]" (ISO standard

format), e.g. 831009 23:59.  The  time  is  optional;  if  given,  it

should  be  in  24-hour format, and the seconds may be omitted, and a

single space should separate the time from the date.

$ (ASCII 36)

Creator's ID, expressed as a character string of the given length.

% (ASCII 37)

Account to charge the file to, character string.

& (ASCII 38)

Area in which to store the file, character string.

' (ASCII 39)

Password for above, character string.

( (ASCII 40)

Block Size.  The file is to be stored with the given block size. This

may be useful when sending files to or from IBM mainframes,  VAX/VMS,

or other systems where files may have this attribute.

) (ASCII 41)

Access:

N   New, the normal case -- create a new file of the given name.

S   Supersede (overwrite) any file of the same name.

A   Append to file of the given name.

* (ASCII 42)

Encoding:

A   ASCII, normal ASCII encoding with any necessary prefixing, etc.

H   Hexidecimal "nibble" encoding.

X   Encrypted.

E   EBCDIC (sent as if it were a binary file).

- 34 -

S   Sixbit Character set (equivalent to I6).

R   Radix50

# (ASCII 43)

Disposition  (operands  are specified in the syntax of the receiver's

host system):

M{user(s)}      Send the file as Mail to the specified user(s).

O{destination}  Send the file as  a  lOng  terminal  message  to  the

specified destination (terminal, job, or user).

S[{options}]    Submit  the  file  as a batch job, with any specified

options.

P[{options}]    Print  the  file  on  a  system  printer,  with   any

specified  options,  which  may  specify a particular

printer, forms, etc.

T               Type the file on the screen.

L[{aaa}]        Load the file into memory at the  given  address,  if

any.

X[{aaa}]        Load  the  file  into memory at the given address and

eXecute it.

, (ASCII 44)

Protection.  Protection code for the  file,  in  the  syntax  of  the

receiver's host file system.  With no operand, store according to the

system's default protection for the destination area.

- (ASCII 45)

Protection.    Protection  code  for  the  file  with  respect to the

"public" or "world", expressed in generically  in  a  6-bit  quantity

(made  printable  by  char()),  in  which the bits have the following

meaning:

b0: Read Access

b1: Write Access

b2: Execute Access

b3: Append Access

b4: Delete Access

b5: Directory Listing

A one in the bit position means allow the corresponding type  of  ac-

cess,  a zero means prohibit it.  For example, the letter "E" in this

field  would  allow  read,  execute,  and  directory  listing  access

(unchar("E") = 69-32 = 37 = 100101 binary).

Other  attributes  can be imagined, and can be added later if needed.  However,

two important points should be noted:

- The receiver may have absolutely no way of honoring, or even  record-

ing,  a given attribute.  For instance, CP/M-80 has no slot for crea-

tion date or creator's ID in its FCB; the DEC-20 has  no  concept  of

- 35 -

block size, etc.

- The  sender  may have no way of determining the correct values of any

of the attributes.  This is particularly true when sending  files  of

foreign origin.

The  "A" packet mechanism only provides a way to send certain information about

a file to the receiver, with no provision or guarantee about what the  receiver

may  do  with  it.    That information may be obtained directly from the file's

directory entry (FCB, FDB, ...), or specified via user command.

The ACK to the "A" packet may in turn  have  information  in  its  data  field.

However,  no  complicated negotiations about file attributes may take place, so

the net result is that the receiver may either refuse the file  or  accept  it.

The receiver may reply to the "A" packet with any of the following codes in the

data field of the ACK packet:

<null>  (empty data field) I accept the file, go ahead and send it.

N[{xxx}]

I  refuse  the  file  as specified, don't send it; {xxx} is a string of

zero or more of the attribute characters listed above, to specify  what

attributes I object to (e.g. "!" means it's too long, "&" means I don't

have write access to the specified area, etc).

Y[{xxx}]

I  agree to receive the file, but I cannot honor attributes {xxx}, so I

will store the file according to my own defaults.

Y       (degenerate case of Y{xxx}, equivalent to <null>, above)

How the receiver actually replies is an implementation  decision.    A  NAK  in

response  to the "A" packet means, of course, that the receiver did not receive

the "A" correctly, not that it refuses to receive the file.

- 36 -

10. KERMIT Commands

The  following  list  of KERMIT commands and terms is suggested.  It is not in-

tended to recommend a particular style of command parsing, only  to  promote  a

consistent vocabulary, both in documentation and in choosing the names for com-

mands.

10.1. Basic Commands

SEND    This verb tells a Kermit program to send one or more files from its own

file structure.

RECEIVE This  verb  should tell a Kermit program to expect one or more files to

arrive.

GET     This verb should tell a user Kermit to send one or more  files.    Some

Kermit  implementations  have separate RECEIVE and GET commands; others

use RECEIVE for both purposes, which creates confusion.

Since it can be useful, even necessary, to specify different names  for  source

and destination files, these commands should take operands as follows (optional

operands in [brackets]):

SEND local-source-filespec [remote-destination-filespec]

If  the destination file specification is included, this will go in the

file header packet, instead of the file's local name.

RECEIVE [local-destination-filespec]

If the destination filespec is given, the incoming file will be  stored

under that name, rather than the one in the file header pakcet.

GET remote-source-filespec [local-destination-filespec]

If  the destination filespec is given, the incoming file will be stored

under that name, rather than the one in the file header packet.

If a file group is being sent or received, alternate names should not be used.

10.2. Program Management Commands

EXIT    Leave the KERMIT program, doing  whatever  cleaning  up  must  be  done

-- deassigning of devices, closing of files, etc.

QUIT    Leave  the  KERMIT  program without cleaning up, in such a manner as to

allow further manipulation of the files and devices.

PUSH    Preserve the current KERMIT environment and enter  the  system  command

processor.

TAKE    Read and execute KERMIT program commands from a local file.

- 37 -

10.3. Terminal Emulation Commands

CONNECT This  verb,  valid  only  for a local Kermit, means to go into terminal

emulation mode; present the illusion of being directly connected  as  a

terminal  to the remote system.  Provide an "escape character" to allow

the user to "get back" to the local system.  The escape character, when

typed, should take a single-character argument; the following are  sug-

gested:

0   (zero) Transmit a NUL

B   Transmit a BREAK

C   Close the connection, return to local KERMIT command level

P   Push to system command processor

Q   Quit logging (if logging is being done)

R   Resume logging

S   Show status of connection

?   Show the available arguments to the escape character

(a  second  copy  of  the  escape  character):  Transmit the escape

character itself

Lower case equivalents should be accepted.  If any invalid argument  is

typed, issue a beep.

Also see the SET command.

10.4. Special User-Mode Commands

These commands are used only by Users of Servers.

BYE     This  command  sends  a message to the remote server to log itself out,

and upon successful completion, terminate the local Kermit program.

FINISH  This command causes the remote server to shut  itself  down  gracefully

without logging out its job, leaving the local KERMIT at KERMIT command

level, allowing the user to re-CONNECT to the remote job.

10.5. Commands Whose Object Should Be Specified

Some  Kermit implementations include various local file management services and

commands to invoke them.  For instance, an implementation might  have  commands

to  let  you  get  directory  listings, delete files, switch disks, and inquire

about free disk space without having to exit and restart the program.   In  ad-

dition,  remote  servers may also provide such services.  A user Kermit must be

able to distinguish between commands aimed at its own system and those aimed at

the remote one.  When any confusion is possible, such a command may be prefixed

by one of the following "object prefixes":

REMOTE  Ask the remote Kermit server to provide this service.

LOCAL   Perform the service locally.

If the "object prefix" is omitted, the command should be executed locally.  The

services include:

- 38 -

LOGIN   This  should  be  used  in its timesharing sense, to create an identity

("job", "session", "access", "account") on the system.

LOGOUT  To terminate a session that was initiated by LOGIN.

CWD     Change Working Directory.  This is ugly, but more  natural  verbs  like

CONNECT and ATTACH are too imprecise.  CWD is the ARPAnet file transfer

standard command to invoke this function.

DIRECTORY

Provide  a  list  of  the  names, and possibly other attributes, of the

files in the current working directory (or the specified directory).

SPACE   Tell how much space is used and available for storing files in the cur-

rent working directory (or the specified directory).

DELETE  Delete the specified files.

ERASE   This could be a synomym for DELETE, since its meaning is clear.

(It doesn't seem wise to include UNDELETE  or  UNERASE  in  the

standard  list; most systems don't support such a function, and

users' expectations should not be toyed with...)

COPY    Make a new copy of the specified file with the specified name.

RENAME  Change the name of the specified file.

TYPE    Display the contents of the specified file(s) at the terminal.

SUBMIT  Submit the specified file(s) for background (batch) processing.

PRINT   Print the specified file(s) on a printer.

MOUNT   Request a mount of the specified tape, disk, or other removable storage

medium.

WHO     Show who is logged in (e.g. to a timesharing system),  or  give  infor-

mation about a specified user or network host.

MAIL    Send electronic mail to the specified user(s).

MESSAGE Send a terminal message (on a network or timesharing system).

HELP    Give brief information about how to use KERMIT.

SET     Set  various parameters relating to debugging, transmission, file mode,

and so forth.

SHOW    Display settings of SET parameters, capabilities in force, etc.

STATISTICS

Give information about the performance of the most recent file transfer

-- elapsed time, effective baud rate, various counts, etc.

HOST    Pass the given command string to the specified (i.e. remote  or  local)

- 39 -

host for execution in its own command language.

10.6. The SET Command

A  SET  command  should be provided to allow the user to tailor a connection to

the peculiarities of the communication path, the local or remote  file  system,

etc.  Here are some parameters that should be SET-able:

BLOCK-CHECK

Specify  the type of block check to be used: single character checksum,

two-character checksum, 3-character CRC.

DEBUGGING

Display or log the  packet  traffic,  packet  numbers,  and/or  program

states.    Useful  for debugging new versions of KERMIT, novel combina-

tions of KERMIT programs, etc.

DELAY   How many seconds a remote (non-server) KERMIT should wait before  send-

ing  the  Send-Init packet, to give the user time to escape back to the

local KERMIT and type a RECEIVE command.

DUPLEX  For terminal emulation, specify FULL or HALF duplex echoing.

EIGHT-BIT-PREFIXING

Specify that "8th bit" prefixing must be done; normally it will not  be

done.

END-OF-LINE

Specify any line terminator that must be used after a packet.

ESCAPE  Specify the escape character for terminal emulation.

FILE attributes

Almost  any  of  the  attributes listed above in the Attributes section

(9.5).  The most common need is to tell the KERMIT program  whether  an

incoming or outbound file is text or binary.

HANDSHAKE

Specify  any  line-access  negotiation  that  must be used or simulated

during file transfer.  For instance, a half duplex  system  will  often

need to "turn the line around" after sending a packet, in order to give

you  permission  to reply.  A common handshake is XON (^Q); the current

user of the line transmits an XON when done transmitting data.

LINE    Specify the line or device designator for the connection.  This is  for

use  in  a  KERMIT program that can run in either remote or local mode;

the default line is the controlling terminal  (for  remote  operation).

If an external device is used, local operation is presumed.

LOG     Specify  a local file in which to keep a log of the transaction.  There

may be logs for debugging purposes (packet traffic, state  transitions,

etc)  and  for auditing purposes (to record the name and disposition of

each file transferred).

MARKER  Change the start-of-packet marker from the default of SOH  (CTRL-A)  to

- 40 -

some  other control character, in case one or both systems has problems

using CTRL-A for this purpose.

PACKET-LENGTH

The maximum length for a packet.  This should normally be no less  than

30  or  40,  and can never be greater than 96.  Short packets can be an

advantage on noisy lines; they reduce the  probabily  of  a  particular

packet  being  corrupted,  as  well as the retransmission overhead when

corruption does occur.

PADDING The number of padding  characters  that  should  be  sent  before  each

packet, and what the padding character should be.  Rarely necessary.

PARITY  Specify  the parity (ODD, EVEN, MARK, SPACE, NONE) of the physical con-

nection.  If other than none, the "8th bit" cannot be used to  transmit

data and must not be used by either side in block check computation.

PAUSE   How  many  seconds to pause after receiving a packet before sending the

next packet.  Normally 0, but when a system communication processor  or

front  end  has  trouble keeping up with the traffic, a short pause be-

tween packets may allow it to recover its  wits;  hopefully,  something

under a second will suffice.

PREFIX  Change  the default prefix for control characters, 8-bit characters, or

repeated quantities.

REPEAT-COUNT-PROCESSING

Change the default for repeat count processing.  Normally, it  will  be

done if both KERMIT programs agree to do it.

RETRY   The  maximum number of times to attempt to send or receive a packet be-

fore giving up.  The normal number is about 5, but the user  should  be

able  to  adjust it according to the condition of the line, the load on

the systems, etc.

TIMEOUT Specify the length of the timer to set when waiting for a packet to ar-

rive.

10.7. Macros

In addition to the individual set commands, a "macro" facility  is  recommended

to allow users to combine the characteristics of specific systems into a single

SET option.  For example:

DEFINE IBM = PARITY ODD, DUPLEX HALF, HANDSHAKE XON

DEFINE UNIX = PARITY NONE, DUPLEX FULL

DEFINE TELENET = PARITY MARK

This  could  be done by providing a fancy runtime parser for commands like this

(which could be automatically TAKEn from the user's KERMIT initialization  file

upon program startup), or simply hardwired into the SET command table.

With  these  definitions  in  place, the user would simply type "SET IBM", "SET

UNIX", and so forth, to set up the program to communication to the remote  sys-

tem.

- 41 -

11. Terminal Emulation

The local system must be able to act as a terminal so that the user can connect

to the remote system, log in, and start up the remote KERMIT.

Terminal  emulation should be provided by any KERMIT program that runs locally,

so that the user need not exit and restart the local KERMIT program in order to

switch between terminal and protocol operation.  On smaller  systems,  this  is

particularly important for various reasons -- restarting the program and typing

in  all  the  necessary SET commands is too inconvenient and time-consuming; in

some micros, switching in and out of terminal emulation may  cause  carrier  to

drop, etc.

Only bare-bones terminal emulation need be supplied by KERMIT; there is no need

to  emulate  any  particular  kind of "smart" terminal.  Simple "dumb" terminal

emulation is sufficient to do the job.  Emulation of fancier terminals is  nice

to  have, however, to take advantage of the remote system's editing and display

capabilities.  In some cases, microcomputer firmware will take  care  of  this.

To build emulation for a particular type of terminal into the program, you must

interpret and act upon escape sequences as they arrive at the port.

No  error  checking  is  done  during  terminal  emulation.  It is "outside the

protocol"; characters go back and forth "bare".  In this sense, terminal emula-

tion through KERMIT is no better than actually using a real terminal.

Some KERMIT implementations may allow logging of the terminal emulation session

to a local file.  Such a facility allows "capture" of  remote  typescripts  and

files,  again  with  no  error  checking  or correction.  When this facility is

provided, it is also desirable to have a convenient way of "toggling" the  log-

ging on and off.

If  the  local  system does not provide system- or firmware-level flow control,

like XON/XOFF, the terminal emulation program should attempt  to  simulate  it,

especially if logging is being done.

The terminal emulation facility should be able to handle either remote or local

echoing (full or half duplex), any required handshake, and it should be able to

transmit any parity required by the remote side or the communication medium.

A  terminal emulator works by continuously sampling both console input from the

local terminal and input from the communication line.  Simple input and  output

functions  will not suffice, however, since if you ask for input from a certain

device and there is none available, you will generally block until  input  does

become  available,  during  which time you will be missing input from the other

device.  Thus you must have a way  to  bounce  back  and  forth  regardless  of

whether input is available.  Several mechanisms are commonly used:

- Continuously jump back and forth between the port status register and

the  console  status  register,  checking  the  status bits for input

available.  This is only  practical  on  single-user,  single-process

systems, where the CPU has nothing else to do.

- Issue an ordinary blocking input request for the port, but enable in-

terrupts on console input, or vice versa.

- Handle port input in one process and console input in another, paral-

- 42 -

lel process.  The UNIX KERMIT program listed in this manual uses this

method.

Any input at the port should be displayed immediately on the screen.  Any input

from the console should be output immediately to the port.  In addition, if the

connection is half duplex, console input should also be sent immediately to the

screen.

The  terminal  emulation  code must examine each console character to determine

whether it is the "escape character".  If so, it should take the next character

as a special command, which it executes.  These commands are  described  above,

in section 10.3.

The terminal emulator should be able to send every ASCII character, NUL through

DEL,  and  it  should  also  be able to transmit a BREAK signal (BREAK is not a

character, but an "escape" from ASCII transmission in which the a 0 is  put  on

the  line for about a quarter of a second, regardless of the baud rate, with no

framing bits).  BREAK is important when  communicating  with  various  systems,

such as IBM mainframes.

- 43 -

12. Writing a KERMIT Program

Before writing a new implementation of KERMIT or modifying an old one, first be

sure  to  contact the KERMIT Distribution center at Columbia University to make

sure that you're not duplicating someone else's effort, and that you  have  all

the  latest material to work from.  If you do write or significantly modify (or

document) a KERMIT program, please send it back to Columbia so that it  can  be

included  in  the  standard KERMIT distribution and others can benifit from it.

It is only through this kind of sharing that KERMIT has grown from  its  modest

beginnings to its present scale.

The following sections provide some hints on KERMIT programming.

12.1. Program Organization

A  basic  KERMIT  implementation  can  usually be written as a relatively small

program, self-contained in a single source file.  However, it is often the case

that a program written to run on one system will be adapted  to  run  on  other

systems  as  well.   In that case, it is best to avoid having totally divergent

sources, because when new features are added to (or bugs fixed in) the  system-

independent parts of the program -- i.e. to the protocol itself -- only one im-

plementation will reap the benefits initially, and the other will require pain-

ful, error-prone "retrofitting" to bring it up to the same level.

Thus,  if  there  is any chance that a KERMIT program will run on more than one

machine, or under more than one operating system, or support more than one kind

of port or modem, etc, it is desirable to isolate the system-dependent parts in

a way that makes the common parts usable by the various implementations.  There

are several approaches:

1. Runtime support.  If possible, the program can inspect the  hardware

or  inquire  of  the system about relevant parameters, and configure

itself dynamically at startup time.  This is hardly ever possible.

2. Conditional compilation (or assembly).  If the number of systems  or

options  to  be supported is small, the system dependent code can be

enclosed in conditional compilation brackets  (like  IF  WIDGET-8000

.... ENDIF).  An example is provided in UNIX KERMIT listing included

with  this manual.  However, as the number of system dependencies to

be supported grows, this method  becomes  unwieldy  and  error-prone

--  installing  support for system X may well break the pre-existing

support for system Y.

3. Modular composition.  When there is a potentially  large  number  of

options  a  program  should  support,  it  should  be broken up into

separate modules (source  files),  with  clearly  specified,  simple

calling conventions.  This allows people with new options to provide

their  own  support for them in an easy way, without endangering any

existing support.  Suggested modules for a KERMIT program are:

- System-Indendent protocol  handling:  state  switching,  packet

formation, encoding (prefixing) and decoding, etc.

- User Interface: the command parser.  Putting this in a separate

module  allows  plug-in  of  command parsers to suit the user's

- 44 -

taste,  to mimic the style of the host system command parser or

some popular application, etc.

- Screen i/o: Allows various  terminals  to  be  supported;  this

module would contain the screen control codes, cursor position-

ing routines, etc.

- Port i/o: Allows support of various port hardware.  This module

can  define the port status register location, the status bits,

and so forth, and can implement the functions to read and write

characters at the port.

- Modem  control:  This  module  would  support   any   kind   of

"intelligent"  modem,  which is not simply a transparent exten-

sion of the communications port.  Such modems may  accept  spe-

cial  commands to perform functions like dialing out, redialing

a recent number, hanging up, etc.

- Console input: This module would supply  the  function  to  get

characters  from  the  console;  it would know about the status

register  locations  and  bits,  interrupt  structure,  key-to-

character mappings, etc.

- File i/o: This module contains all the knowledge about the host

system's  file  structure; how to open and close files, perform

"get next file" operations, read and write files, determine and

set their attributes, detect the end of a file, and  so  forth,

and  provides  the  functions,  including  buffering,  to get a

character from a file and put a character  to  a  file.    This

module  may  also  provide  file  management services for local

files -- directory listings, deleting, renaming,  copying,  and

so forth.

- Terminal  emulation:  This module would provide terminal emula-

tion by calling upon the other system-dependent modules.

- Definitions and Data: Separate modules might also be  kept  for

compile-time parameter definitions and for global runtime data.

12.2. Programming Language

The  language  to  be used in writing a KERMIT program is more than a matter of

taste.  The primary consideration is that the language  provide  the  necessary

functionality and speed.  For instance, a microcomputer implementation of BASIC

may  not  allow  the  kind of low-level access to device registers needed to do

terminal emulation, or to detect console input during file transfer, or even if

it can do these things, it might not be able to run fast enough  do  drive  the

communication line at the desired baud rate.

The  second  consideration in choosing a language is portability.  This is used

in two senses: (1)  whether  the  language  public  domain  (or,  equivalently,

provided  "free" as part of the basic system), and (2) whether it is well stan-

dardized and in wide use on a variety of systems.  A language that is  portable

in both senses is to be preferred.

- 45 -

Whatever  programming  language  is selected, it is important that all lines in

the program source be kept to 80 characters or less (after expansion of  tabs).

This  is because KERMIT material must often be shipped over RJE and other card-

format communication links.

12.3. Documentation

A new KERMIT program should be thoroughly documented; one of the  hallmarks  of

KERMIT  is  its  documentation.    The documentation should be at both the user

level (how to use the program, what the  commands  are,  etc,  similar  to  the

documentation  presently  found  in  the Kermit Users Guide), and the implemen-

tation level (describe system dependencies, give pointers for adapting  to  new

systems,  and  so  forth).    In  addition,  programs themselves should contain

copious commentary.  Like program source, documentation should be  kept  within

80-character lines.

KERMIT programs should also contain a revision history, in which each change is

briefly  explained,  assigned an "edit number", and the programmer and site are

identified.  The lines or sections comprising the edit should  be  marked  with

the corresponding edit number, and the KERMIT program, upon startup, should an-

nounce  its  version  and  edit  numbers.   The version number changes when the

functionality has been changed sufficiently to require major revisions of  user

documentation.  The edit number should increase (monotonically, irrespective of

version number) whenever a change is made to the program.  The edit numbers are

very  important  for  program management; after shipping out a version of, say,

CP/M KERMIT-80, we often receive many copies of it, each containing its own set

of changes, which we must reconcile in some manner.  Edit numbers help a  great

deal here.

12.4. Bootstrapping

Finally,  a  bootstrap  procedure should be provided.  KERMIT is generally dis-

tributed on magnetic tape to large central sites; the users at those sites need

ways of "downloading" the various implementations to  their  micros  and  other

local  systems.  A simple bootstrap procedure would consist of precise instruc-

tions on how to accomplish an "unguarded" capture of the program.    Perhaps  a

simple,  short  program  can be written for each each end that will do the job;

listings and instructions can be provided for the user to type in and run these

programs.

- 46 -

I. Packet Format and Types

KERMIT Packet Layout:

+------+-----+-----+------+------------+-------+

| MARK | LEN | SEQ | TYPE |    DATA    | CHECK |

+------+-----+-----+------+------------+-------+

Send-Init Data Field Layout:

1      2      3      4      5      6      7      8      9      10...

+------+------+------+------+------+------+------+------+------+-------

| MAXL | TIME | NPAD | PADC | EOL  | QCTL | QBIN | CHKT | REPT | CAPAS

+------+------+------+------+------+------+------+------+------+-------

KERMIT  packet  types  and subtypes; required types are marked with an asterisk

(*):

Y*  Acknowledge (ACK)

N*  Negative acknowledge (NAK)

S*  Send initiate (exchange parameters)

I   Initialize (exchange parameters).

F*  File header

A   File attributes

D*  Data packet

Z*  End of file (EOF)

B*  Break transmission (EOT)

E*  Error

R   Receive Initiate (ask the server to send the specified files).

C   Host Command.

G   Generic Kermit Command.  Subcommands:

I   Login (or logout)

C   CWD, Change Working Directory

L   Bye

F   Finish

D   Directory

U   Disk Usage Query

E   Erase, Delete

T   Type

R   Rename

K   Copy

W   Who's logged in?

M   Short Message

H   Help

Q   Server Status Query

- 47 -

II. List of Features

There's  no  true  linear  scale along which to rate Kermit implementations.  A

basic, minimal implementation provides file transfer in both  directions,  and,

for  microcomputers  (PC's,  workstations, other single user systems), terminal

emulation.  Even within this framework, there can be variations.  For instance,

can the program send a file group in a single command, or  must  a  command  be

issued for each file?  Can it time out?  Here is a list of features that may be

present;  for  any KERMIT implementation, the documentation should show whether

these features exist, and how to invoke them.

- File groups.  Can it send a group of files  with  a  single  command,

using  "wildcard",  pattern,  or  list notation?  Can it successfully

send or receive a group of files of mixed types?  Can it recover from

an error on a particular file and go on to the next one?  Can it keep

a log of the files involved showing the disposition of each?

- Filenames.  Can it take action to avoid overwriting a local file when

a new file of the same name arrives?  Can it convert filenames to and

from "normal form"?

- File Attributes.  Can transfer binary as well as text files?  Can  it

send and receive Attribute packets?

- 8th-Bit  prefixing.    Can  it  send and receive 8-bit data through a

7-bit channel using the prefixing mechanism?

- Repeat-Count processing.  Can it send and receive data with  repeated

characters replaced by a prefix sequence?

- Terminal  Emulation.    Does  it  have a terminal emulation facility?

Does it  provide  various  communication  options,  such  as  duplex,

parity,  and  handshake selection?  Can it transmit all ASCII charac-

ters?  Can it transmit BREAK?  Can it log the remote session locally,

to provide unguarded file capture?

- Communications Options.  Can duplex, parity, handshake, and line ter-

minator be specified for file transfer?

- Block Check Options.   In  addition  to  the  basic  single-character

checksum,  can the two-character checksum and the three-character CRC

be selected?

- Basic Server.  Can it run in server mode, accepting commands to  send

or receive files, and to shut itself down?

- Advanced  Server.    Can  it  accept server commands to delete files,

provide directory listings, send messages, and forth?

- Issue Commands to Server.  Can it send  commands  to  a  server,  and

handle all possible responses?

- Host  Commands.  Can it parse and send remote "host commands"?  If it

is a server, can it pass these commands to the  host  system  command

processor and return the results to the local user Kermit?

- 48 -

- Interrupt  File  Transfers.   Can it interrupt sending or receiving a

file?  Can it respond to interrupt requests from the other side?

- Local File Management Services.  Are  there  commands  to  get  local

directory directory listings, delete local files, and so forth?

- File  Attributes.  Can it send file attribute information about local

files, and can deal with incoming file attribute information?

- Debugging Capability.  Can packet traffic be logged or examined?

- 49 -

III. Unresolved Issues

KERMIT doesn't do everything.  Here is a short list of things it doesn't do, or

could do better.

- Control  character  prefixing.  This can get pretty expensive for bi-

nary files or other files that contain lots  of  control  characters.

Since  most  hosts  won't choke on every single control character, it

might be a good idea for each host to inform the other of  what  con-

trol characters it can receive "bare".  On the other hand, this could

backfire when the two hosts are connected by a network or device that

chokes on control characters that neither of the hosts choke on.

- When  long  sequences  of  characters  in the control range are being

sent, individually prefixing each character  is  costly.    Shift-in/

shift-out  codes  might be used more effectively here.  Same for long

strings of characters with the parity bit on, when 8th-bit  prefixing

is  being  done.  The cost would be yet another set of prefix charac-

ters, and the associated complexity of packet formation and decoding.

- In some situations, certain printable characters can't get through  a

communication  link.    For instance, a network terminal server might

reserve "@" as its attention character.  The  protocol  should  allow

some way to encode such characters for translation.

- KERMIT  can transfer only sequential files.  No mechanism is provided

to transfer random access or "sparse" files.  Such a mechanism  could

be  devised;  for instance, control packets intermixed with file data

packets could specify "where" to store the chunk of  data  that  fol-

lows.

- Ironically,  some systems do not fare well with KERMIT's small packet

sizes.  These are typically big mainframes that expect to communicate

with block mode terminals, receiving thousands  of  characters  at  a

time.   Such systems find that KERMIT is simply too expensive to run.

Meanwhile, other systems that can run KERMIT with no difficulty  find

the  performance  disappointing  (the  efficiency generally works out

somewhere between 50 and 80 percent, i.e. data bits  divided  by  the

speed of the the transmission line, in characters per second).  These

two  problems  could  be  solved in either of two ways:  allow longer

packets, or allow multiple data packets to be sent end to end.

- Without changing the format or encoding of the packet control fields,

it might be possible to  have  longer  packets  by  reserving  packet

lengths  of  0,  1,  2,  3 be codes for bigger numbers like 200, 500,

1000, 2000.  However, the longer the packet, the  greater  the  prob-

ability of corruption, and the longer to retransmit once corrupted.

- It  would be possible to extend the protocol to allow transmission of

data packets while ACKs were still outstanding, and any ACK would  be

taken  as  an  ACK for the specified packet and all previous ones.  A

limit on the number of outstanding ACKs could  be  agreed  upon;  the

maximum size of this "floating window" would have to be less than 64,

which  is  where  KERMIT  packet  numbers wrap around, and the window

could not extend over a wraparound or else KERMIT would have  no  way

of  knowing  whether n times 64 packets had been skipped.  A floating

- 50 -

window of 10 and a packet size of 95 would give about the same effect

as 1000-character packets.  A NAK would require the sender to back up

all  the  way  to the NAK'd packet.  A future edition of the Protocol

Manual might include a specification for floating windows.

- 51 -

IV. A KERMIT Program

What  follows  is  a listing of a real production version of KERMIT, written in

8

the C language .  It is designed to run under  various  versions  of  the  UNIX

operating  system.   The basic KERMIT functionality is present, but very little

in the way of optional features.  Only the most rudimentary  (UNIX-style)  com-

mand  parser  is  provided.  The program illustrates many of the considerations

described in this manual, with respect to the protocol, the program design, and

so forth.

It must be emphasized that this is a bare  minimum  implementation  of  Kermit.

Anyone  writing  a  new Kermit from scratch is encouraged to look at the source

for one of the more advanced implementations as a model (check the Kermit Users

Guide for a list of KERMIT implementations and their features).   Although  you

may  not  understand the language it's written in, there should be profuse com-

ments that can be useful.

The following program uses decimal notation for numbers,  and  tochar()  rather

than char() for integer-to-character conversion.

/*

*  K e r m i t  File Transfer Utility

*

*  UNIX Kermit, Columbia University, 1981, 1982, 1983

*      Bill Catchings, Bob Cattani, Chris Maio, Frank da Cruz, Alan Crosswell

*

*  Also:   Jim Guyton, Rand Corporation

*          Walter Underwood, Ford Aerospace

*

*  usage:  kermit c [lbe line baud escapechar]         to connect

*          kermit s [d..iflb line baud] file ...       to send files

*          kermit r [d..iflb line baud]                to receive files

*

*  where   c=connect, s=send, r=receive,

*          d=debug, i=image mode, f=no filename conversion, l=tty line,

*          b=baud rate, e=escape char.

*

*  For remote Kermit, format is either:

*          kermit r                                    to receive files

*  or      kermit s file ...                           to send files

*

*/

_______________

8

Kernighan & Ritchie, The C Programming Language:  Prentice-Hall (1978)

- 52 -

/*

*  Modification History:

*

*  Oct. 17 Included fixes from Alan Crosswell (CUCCA) for IBM_UTS:

*          - Changed MYEOL character from \n to \r.

*          - Change char to int in bufill so getc would return -1 on

*            EOF instead of 255 (-1 truncated to 8 bits)

*          - Added read() in rpack to eat the EOL character

*          - Added fflush() call in printmsg to force the output

*          NOTE: The last three changes are not conditionally compiled

*                since they should work equally well on any system.

*

*          Changed Berkeley 4.x conditional compilation flag from

*              UNIX4X to UCB4X.

*          Added support for error packets and cleaned up the printing

*              routines.

*/

#include <stdio.h>          /* Standard UNIX definitions */

/* Conditional compilation for different machines/operating systems */

/* One and only one of the following lines should be 1 */

#define UCB4X       1       /* Berkeley 4.x UNIX */

#define TOPS_20     0       /* TOPS-20 */

#define IBM_UTS     0       /* Amdahl UTS on IBM systems */

#define VAX_VMS     0       /* VAX/VMS (not yet implemented) */

/* Conditional compilation for the different Unix variants */

/* 0 means don't compile it, nonzero means do */

#if UCB4X

#define V6_LIBS     0       /* Dont't use retrofit libraries */

#define NO_FIONREAD 0       /* We have ioctl(FIONREAD,...) for flushinput() */

#define NO_TANDEM   0       /* We have TANDEM line discipline (xon/xoff) */

#endif

#if IBM_UTS

#define V6_LIBS     0       /* Don't use retrofit libraries */

#define NO_FIONREAD 1       /* No ioctl(FIONREAD,...) for flushinput() */

#define NO_TANDEM   1       /* No TANDEM line discipline (xon/xoff) */

#endif

#if V6_LIBS

#include <retrofit/sgtty.h>

#include <retrofit/signal.h>

#include <retrofit/setjmp.h>

#else

#include <sgtty.h>

#include <signal.h>

#include <setjmp.h>

#endif

- 53 -

#if NO_TANDEM

#define TANDEM      0       /* define it to be nothing if it's unsupported */

#endif

/* Symbol Definitions */

#define MAXPACKSIZ  94      /* Maximum packet size */

#define SOH         1       /* Start of header */

#define CR          13      /* ASCII Carriage Return */

#define SP          32      /* ASCII space */

#define DEL         127     /* Delete (rubout) */

#define ESCCHR      '^'     /* Default escape character for CONNECT */

#define MAXTRY      10      /* Times to retry a packet */

#define MYQUOTE     '#'     /* Quote character I will use */

#define MYPAD       0       /* Number of padding characters I will need */

#define MYPCHAR     0       /* Padding character I need (NULL) */

#if IBM_UTS

#define MYEOL       '\r'    /* End-Of-Line character for UTS systems */

#else

#define MYEOL       '\n'    /* End-Of-Line character I need */

#endif

#define MYTIME      10      /* Seconds after which I should be timed out */

#define MAXTIM      60      /* Maximum timeout interval */

#define MINTIM      2       /* Minumum timeout interval */

#define TRUE        -1      /* Boolean constants */

#define FALSE       0

/* Macro Definitions */

/*

* tochar: converts a control character to a printable one by adding a space.

*

* unchar: undoes tochar.

*

* ctl:    converts between control characters and printable characters by

*         toggling the control bit (ie. ^A becomes A and A becomes ^A).

*/

#define tochar(ch)  ((ch) + ' ')

#define unchar(ch)  ((ch) - ' ')

#define ctl(ch)     ((ch) ^ 64 )

/* Global Variables */

- 54 -

int     size,               /* Size of present data */

rpsiz,              /* Maximum receive packet size */

spsiz,              /* Maximum send packet size */

pad,                /* How much padding to send */

timint,             /* Timeout for foreign host on sends */

n,                  /* Packet number */

numtry,             /* Times this packet retried */

oldtry,             /* Times previous packet retried */

ttyfd,              /* File descriptor of tty for I/O, 0 if remote */

remote,             /* -1 means we're a remote kermit */

image,              /* -1 means 8-bit mode */

debug,              /* indicates level of debugging output (0=none) */

filnamcnv,          /* -1 means do file name case conversions */

filecount;          /* Number of files left to send */

char    state,              /* Present state of the automaton */

padchar,            /* Padding character to send */

eol,                /* End-Of-Line character to send */

escchr,             /* Connect command escape character */

quote,              /* Quote character in incoming data */

**filelist,         /* List of files to be sent */

*filnam,            /* Current file name */

recpkt[MAXPACKSIZ], /* Receive packet buffer */

packet[MAXPACKSIZ]; /* Packet buffer */

FILE    *fp,                /* File pointer for current disk file */

*log;               /* File pointer for Logfile */

jmp_buf env;                /* Environment ptr for timeout longjump */

/*

*  m a i n

*

*  Main routine - parse command and options, set up the

*  tty lines, and dispatch to the appropriate routine.

*/

main(argc,argv)

int argc;                           /* Character pointers to and count of */

char **argv;                            /* command line arguments */

{

char *ttyname,                      /* tty name for LINE argument */

*cp;                            /* char pointer */

int speed,                          /* speed of assigned tty, */

cflg, rflg, sflg;               /* flags for CONNECT, RECEIVE, SEND */

struct sgttyb

rawmode,                        /* Controlling tty raw mode */

cookedmode,                     /* Controlling tty cooked mode */

ttymode;                        /* mode of tty line in LINE option */

if (argc < 2) usage();              /* Make sure there's a command line */

cp = *++argv; argv++; argc -= 2;    /* Set up pointers to args */

- 55 -

/*  Initialize these values and hope the first packet will get across OK */

eol = CR;                           /* EOL for outgoing packets */

quote = '#';                        /* Standard control-quote char "#" */

pad = 0;                            /* No padding */

padchar = NULL;                     /* Use null if any padding wanted */

speed = cflg = sflg = rflg = 0;     /* Turn off all parse flags */

ttyname = 0;                        /* Default is remote mode */

#if UCB4X                               /* Default to 7-bit masking, CRLF */

image = FALSE;                      /* translation and filename case */

filnamcnv = TRUE;                   /* conversion for UNIX systems */

#else

image = TRUE;                       /* Default to no processing for */

filnamcnv = FALSE;                  /* non-UNIX systems */

#endif

escchr = ESCCHR;                    /* Default escape character */

while ((*cp) != NULL)               /* Parse characters in first arg. */

switch (*cp++)

{

case 'c': cflg++; break;    /* C = Connect command */

case 's': sflg++; break;    /* S = Send command */

case 'r': rflg++; break;    /* R = Receive command */

case 'd':                   /* D = Increment debug mode count */

debug++; break;

case 'f':

filnamcnv = FALSE;      /* F = don't do case conversion */

break;                  /*     on filenames */

case 'i':                   /* I = Image (8-bit) mode */

image = TRUE; break;    /* (this is default for non-UNIX) */

case 'l':                   /* L = specify tty line to use */

if (argc--) ttyname = *argv++;

else usage();

if (debug) printf("Line to remote host is %s\n",ttyname);

break;

case 'e':                   /* E = specify escape char */

if (argc--) escchr = **argv++;

else usage();

if (debug) printf("Escape char is \"%c\"\n",escchr);

break;

- 56 -

case 'b':                   /* B = specify baud rate */

#if UCB4X

if (argc--) speed = atoi(*argv++);

else usage();

if (debug) printf("Line speed to remote host is %d\n",speed);

break;

#else

printmsg("Speed setting implemented for Unix only.");

exit(1);

#endif

}

/* Done parsing */

if ((cflg+sflg+rflg) != 1)          /* Only one command allowed */

usage();

if (ttyname)                        /* If LINE was specified, we */

{                                   /* operate in local mode */

ttyfd = open(ttyname,2);        /* Open the tty line */

if (ttyfd < 0)

{

printmsg("Cannot open %s",ttyname);

exit(1);

}

remote = FALSE;                 /* Indicate we're in local mode */

}

else                                /* No LINE specified so we operate */

{                                   /* in remote mode (ie. controlling */

ttyfd = 0;                      /* tty is the communications line) */

remote = TRUE;

}

/* Put the proper tty into the correct mode */

if (remote)                         /* If remote, use controlling tty */

{

gtty(0,&cookedmode);            /* Save current mode so we can */

gtty(0,&rawmode);               /* restore it later */

rawmode.sg_flags |= (RAW|TANDEM);

rawmode.sg_flags &= ~(ECHO|CRMOD);

stty(0,&rawmode);               /* Put tty in raw mode */

}

else                                /* Local, use assigned line */

{

gtty(ttyfd,&ttymode);

ttymode.sg_flags |= (RAW|TANDEM);

ttymode.sg_flags &= ~(ECHO|CRMOD);

- 57 -

#if UCB4X                               /* Speed changing for UNIX only */

if (speed)                      /* User specified a speed? */

{

switch(speed)               /* Get internal system code */

{

case 110: speed = B110; break;

case 150: speed = B150; break;

case 300: speed = B300; break;

case 1200: speed = B1200; break;

case 2400: speed = B2400; break;

case 4800: speed = B4800; break;

case 9600: speed = B9600; break;

default:

printmsg("Bad line speed.");

exit(1);

}

ttymode.sg_ispeed = speed;

ttymode.sg_ospeed = speed;

}

#endif /* UCB4X */

stty(ttyfd,&ttymode);           /* Put asg'd tty in raw mode */

}

/* All set up, now execute the command that was given. */

if (debug)

{

printf("Debugging level = %d\n\n",debug);

if (cflg) printf("Connect command\n\n");

if (sflg) printf("Send command\n\n");

if (rflg) printf("Receive command\n\n");

}

if (cflg) connect();                /* Connect command */

if (sflg)                           /* Send command */

{

if (argc--) filnam = *argv++;   /* Get file to send */

else

{   if (remote)

stty(0,&cookedmode);    /* Restore controlling tty's modes */

usage();                    /* and give error */

}

fp = NULL;                      /* Indicate no file open yet */

filelist = argv;                /* Set up the rest of the file list */

filecount = argc;               /* Number of files left to send */

if (sendsw() == FALSE)          /* Send the file(s) */

printmsg("Send failed.");   /* Report failure */

else                            /*  or */

printmsg("done.");          /* success */

}

- 58 -

if (rflg)                           /* Receive command */

{

if (recsw() == FALSE)           /* Receive the file(s) */

printmsg("Receive failed.");

else                            /* Report failure */

printmsg("done.");          /* or success */

}

if (remote) stty(0,&cookedmode);    /* Restore controlling tty's modes */

}

/*

*  s e n d s w

*

*  Sendsw is the state table switcher for sending files.  It loops until

*  either it finishes, or an error is encountered.  The routines called

*  by sendsw are responsible for changing the state.

*

*/

sendsw()

{

char sinit(), sfile(), sdata(), seof(), sbreak();

state = 'S';                        /* Send initiate is the start state */

n = 0;                              /* Initialize message number */

numtry = 0;                         /* Say no tries yet */

while(TRUE)                         /* Do this as long as necessary */

{

if (debug) printf("sendsw state: %c\n",state);

switch(state)

{

case 'S':   state = sinit();  break; /* Send-Init */

case 'F':   state = sfile();  break; /* Send-File */

case 'D':   state = sdata();  break; /* Send-Data */

case 'Z':   state = seof();   break; /* Send-End-of-File */

case 'B':   state = sbreak(); break; /* Send-Break */

case 'C':   return (TRUE);           /* Complete */

case 'A':   return (FALSE);          /* "Abort" */

default:    return (FALSE);          /* Unknown, fail */

}

}

}

/*

*  s i n i t

*

*  Send Initiate: send this host's parameters and get other side's back.

*/

char sinit()

{

int num, len;                       /* Packet number, length */

- 59 -

if (numtry++ > MAXTRY) return('A'); /* If too many tries, give up */

spar(packet);                       /* Fill up init info packet */

flushinput();                       /* Flush pending input */

spack('S',n,6,packet);              /* Send an S packet */

switch(rpack(&len,&num,recpkt))     /* What was the reply? */

{

case 'N':  return(state);       /* NAK, try it again */

case 'Y':                       /* ACK */

if (n != num)               /* If wrong ACK, stay in S state */

return(state);          /* and try again */

rpar(recpkt);               /* Get other side's init info */

if (eol == 0) eol = '\n';   /* Check and set defaults */

if (quote == 0) quote = '#';

numtry = 0;                 /* Reset try counter */

n = (n+1)%64;               /* Bump packet count */

return('F');                /* OK, switch state to F */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE: return(state);      /* Receive failure, try again */

default: return('A');           /* Anything else, just "abort" */

}

}

/*

*  s f i l e

*

*  Send File Header.

*/

char sfile()

{

int num, len;                       /* Packet number, length */

char filnam1[50],                   /* Converted file name */

*newfilnam,                     /* Pointer to file name to send */

*cp;                            /* char pointer */

if (numtry++ > MAXTRY) return('A'); /* If too many tries, give up */

- 60 -

if (fp == NULL)                     /* If not already open, */

{   if (debug) printf("   Opening %s for sending.\n",filnam);

fp = fopen(filnam,"r");         /* open the file to be sent */

if (fp == NULL)                 /* If bad file pointer, give up */

{

error("Cannot open file %s",filnam);

return('A');

}

}

strcpy(filnam1, filnam);            /* Copy file name */

newfilnam = cp = filnam1;

while (*cp != '\0')                 /* Strip off all leading directory */

if (*cp++ == '/')               /* names (ie. up to the last /). */

newfilnam = cp;

if (filnamcnv)                      /* Convert lower case to upper  */

for (cp = newfilnam; *cp != '\0'; cp++)

if (*cp >= 'a' && *cp <= 'z')

*cp ^= 040;

len = cp - newfilnam;               /* Compute length of new filename */

printmsg("Sending %s as %s",filnam,newfilnam);

spack('F',n,len,newfilnam);         /* Send an F packet */

switch(rpack(&len,&num,recpkt))     /* What was the reply? */

{

case 'N':                       /* NAK, just stay in this state, */

num = (--num<0 ? 63:num);   /* unless it's NAK for next packet */

if (n != num)               /* which is just like an ACK for */

return(state);          /* this packet so fall thru to... */

case 'Y':                       /* ACK */

if (n != num) return(state); /* If wrong ACK, stay in F state */

numtry = 0;                 /* Reset try counter */

n = (n+1)%64;               /* Bump packet count */

size = bufill(packet);      /* Get first data from file */

return('D');                /* Switch state to D */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE: return(state);      /* Receive failure, stay in F state */

default:    return('A');        /* Something else, just "abort" */

}

}

- 61 -

/*

*  s d a t a

*

*  Send File Data

*/

char sdata()

{

int num, len;                       /* Packet number, length */

if (numtry++ > MAXTRY) return('A'); /* If too many tries, give up */

spack('D',n,size,packet);           /* Send a D packet */

switch(rpack(&len,&num,recpkt))     /* What was the reply? */

{

case 'N':                       /* NAK, just stay in this state, */

num = (--num<0 ? 63:num);   /* unless it's NAK for next packet */

if (n != num)               /* which is just like an ACK for */

return(state);          /* this packet so fall thru to... */

case 'Y':                       /* ACK */

if (n != num) return(state); /* If wrong ACK, fail */

numtry = 0;                 /* Reset try counter */

n = (n+1)%64;               /* Bump packet count */

if ((size = bufill(packet)) == EOF) /* Get data from file */

return('Z');            /* If EOF set state to that */

return('D');                /* Got data, stay in state D */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE: return(state);      /* Receive failure, stay in D */

default:    return('A');        /* Anything else, "abort" */

}

}

/*

*  s e o f

*

*  Send End-Of-File.

*/

char seof()

{

int num, len;                       /* Packet number, length */

if (numtry++ > MAXTRY) return('A'); /* If too many tries, "abort" */

- 62 -

spack('Z',n,0,packet);              /* Send a 'Z' packet */

switch(rpack(&len,&num,recpkt))     /* What was the reply? */

{

case 'N':                       /* NAK, just stay in this state, */

num = (--num<0 ? 63:num);   /* unless it's NAK for next packet, */

if (n != num)               /* which is just like an ACK for */

return(state);          /* this packet so fall thru to... */

case 'Y':                       /* ACK */

if (n != num) return(state); /* If wrong ACK, hold out */

numtry = 0;                 /* Reset try counter */

n = (n+1)%64;               /* and bump packet count */

if (debug) printf("   Closing input file %s, ",filnam);

fclose(fp);                 /* Close the input file */

fp = NULL;                  /* Set flag indicating no file open */

if (debug) printf("looking for next file...\n");

if (gnxtfl() == FALSE)      /* No more files go? */

return('B');            /* if not, break, EOT, all done */

if (debug) printf("   New file is %s\n",filnam);

return('F');                /* More files, switch state to F */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE: return(state);      /* Receive failure, stay in Z */

default:    return('A');        /* Something else, "abort" */

}

}

/*

*  s b r e a k

*

*  Send Break (EOT)

*/

char sbreak()

{

int num, len;                       /* Packet number, length */

if (numtry++ > MAXTRY) return('A'); /* If too many tries "abort" */

spack('B',n,0,packet);              /* Send a B packet */

switch (rpack(&len,&num,recpkt))    /* What was the reply? */

{

case 'N':                       /* NAK, just stay in this state, */

num = (--num<0 ? 63:num);   /* unless NAK for previous packet, */

if (n != num)               /* which is just like an ACK for */

return(state);          /* this packet so fall thru to... */

- 63 -

case 'Y':                       /* ACK */

if (n != num) return(state); /* If wrong ACK, fail */

numtry = 0;                 /* Reset try counter */

n = (n+1)%64;               /* and bump packet count */

return('C');                /* Switch state to Complete */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE: return(state);      /* Receive failure, stay in B */

default:    return ('A');       /* Other, "abort" */

}

}

/*

*  r e c s w

*

*  This is the state table switcher for receiving files.

*/

recsw()

{

char rinit(), rfile(), rdata();     /* Use these procedures */

state = 'R';                        /* Receive-Init is the start state */

n = 0;                              /* Initialize message number */

numtry = 0;                         /* Say no tries yet */

while(TRUE)

{

if (debug) printf(" recsw state: %c\n",state);

switch(state)                   /* Do until done */

{

case 'R':   state = rinit(); break; /* Receive-Init */

case 'F':   state = rfile(); break; /* Receive-File */

case 'D':   state = rdata(); break; /* Receive-Data */

case 'C':   return(TRUE);           /* Complete state */

case 'A':   return(FALSE);          /* "Abort" state */

}

}

}

/*

*  r i n i t

*

*  Receive Initialization

*/

char rinit()

{

int len, num;                       /* Packet length, number */

- 64 -

if (numtry++ > MAXTRY) return('A'); /* If too many tries, "abort" */

switch(rpack(&len,&num,packet))     /* Get a packet */

{

case 'S':                       /* Send-Init */

rpar(packet);               /* Get the other side's init data */

spar(packet);               /* Fill up packet with my init info */

spack('Y',n,6,packet);      /* ACK with my parameters */

oldtry = numtry;            /* Save old try count */

numtry = 0;                 /* Start a new counter */

n = (n+1)%64;               /* Bump packet number, mod 64 */

return('F');                /* Enter File-Receive state */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE:                     /* Didn't get packet */

spack('N',n,0,0);           /* Return a NAK */

return(state);              /* Keep trying */

default:     return('A');       /* Some other packet type, "abort" */

}

}

/*

*  r f i l e

*

*  Receive File Header

*/

char rfile()

{

int num, len;                       /* Packet number, length */

char filnam1[50];                   /* Holds the converted file name */

if (numtry++ > MAXTRY) return('A'); /* "abort" if too many tries */

switch(rpack(&len,&num,packet))     /* Get a packet */

{

case 'S':                       /* Send-Init, maybe our ACK lost */

if (oldtry++ > MAXTRY) return('A'); /* If too many tries "abort" */

if (num == ((n==0) ? 63:n-1)) /* Previous packet, mod 64? */

{                           /* Yes, ACK it again with  */

spar(packet);           /* our Send-Init parameters */

spack('Y',num,6,packet);

numtry = 0;             /* Reset try counter */

return(state);          /* Stay in this state */

}

else return('A');           /* Not previous packet, "abort" */

- 65 -

case 'Z':                       /* End-Of-File */

if (oldtry++ > MAXTRY) return('A');

if (num == ((n==0) ? 63:n-1)) /* Previous packet, mod 64? */

{                           /* Yes, ACK it again. */

spack('Y',num,0,0);

numtry = 0;

return(state);          /* Stay in this state */

}

else return('A');           /* Not previous packet, "abort" */

case 'F':                       /* File Header (just what we want) */

if (num != n) return('A');  /* The packet number must be right */

strcpy(filnam1, packet);    /* Copy the file name */

if (filnamcnv)              /* Convert upper case to lower */

for (filnam=filnam1; *filnam != '\0'; filnam++)

if (*filnam >= 'A' && *filnam <= 'Z')

*filnam |= 040;

if ((fp=fopen(filnam1,"w"))==NULL) /* Try to open a new file */

{

error("Cannot create %s",filnam1); /* Give up if can't */

return('A');

}

else                        /* OK, give message */

printmsg("Receiving %s as %s",packet,filnam1);

spack('Y',n,0,0);           /* Acknowledge the file header */

oldtry = numtry;            /* Reset try counters */

numtry = 0;                 /* ... */

n = (n+1)%64;               /* Bump packet number, mod 64 */

return('D');                /* Switch to Data state */

case 'B':                       /* Break transmission (EOT) */

if (num != n) return ('A'); /* Need right packet number here */

spack('Y',n,0,0);           /* Say OK */

return('C');                /* Go to complete state */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

case FALSE:                     /* Didn't get packet */

spack('N',n,0,0);           /* Return a NAK */

return(state);              /* Keep trying */

default:    return ('A');       /* Some other packet, "abort" */

}

}

- 66 -

/*

*  r d a t a

*

*  Receive Data

*/

char rdata()

{

int num, len;                       /* Packet number, length */

if (numtry++ > MAXTRY) return('A'); /* "abort" if too many tries */

switch(rpack(&len,&num,packet))     /* Get packet */

{

case 'D':                       /* Got Data packet */

if (num != n)               /* Right packet? */

{                           /* No */

if (oldtry++ > MAXTRY)

return('A');        /* If too many tries, abort */

if (num == ((n==0) ? 63:n-1)) /* Else check packet number */

{                       /* Previous packet again? */

spack('Y',num,6,packet); /* Yes, re-ACK it */

numtry = 0;         /* Reset try counter */

return(state);      /* Don't write out data! */

}

else return('A');       /* sorry, wrong number */

}

/* Got data with right packet number */

bufemp(packet,len);         /* Write the data to the file */

spack('Y',n,0,0);           /* Acknowledge the packet */

oldtry = numtry;            /* Reset the try counters */

numtry = 0;                 /* ... */

n = (n+1)%64;               /* Bump packet number, mod 64 */

return('D');                /* Remain in data state */

case 'F':                       /* Got a File Header */

if (oldtry++ > MAXTRY)

return('A');            /* If too many tries, "abort" */

if (num == ((n==0) ? 63:n-1)) /* Else check packet number */

{                           /* It was the previous one */

spack('Y',num,0,0);     /* ACK it again */

numtry = 0;             /* Reset try counter */

return(state);          /* Stay in Data state */

}

else return('A');           /* Not previous packet, "abort" */

case 'Z':                       /* End-Of-File */

if (num != n) return('A');  /* Must have right packet number */

spack('Y',n,0,0);           /* OK, ACK it. */

fclose(fp);                 /* Close the file */

n = (n+1)%64;               /* Bump packet number */

return('F');                /* Go back to Receive File state */

case 'E':                       /* Error packet received */

prerrpkt(recpkt);           /* Print it out and */

return('A');                /* abort */

- 67 -

case FALSE:                     /* Didn't get packet */

spack('N',n,0,0);           /* Return a NAK */

return(state);              /* Keep trying */

default:     return('A');       /* Some other packet, "abort" */

}

}

/*

*  c o n n e c t

*

*  Establish a virtual terminal connection with the remote host, over an

*  assigned tty line.

*/

connect()

{

int pid,                            /* Holds process id of child */

connected;                      /* Boolean connect flag */

char bel = '\07',

c;

struct sgttyb

rawmode,                        /* Controlling tty raw mode */

cookedmode;                     /* Controlling tty cooked mode */

if (remote)                         /* Nothing to connect to in remote */

{                                   /* mode, so just return */

printmsg("No line specified for connection.");

return;

}

gtty(0,&cookedmode);                /* Save current mode so we can */

gtty(0,&rawmode);                   /* restore it later */

rawmode.sg_flags |= (RAW|TANDEM);

rawmode.sg_flags &= ~(ECHO|CRMOD);

stty(0,&rawmode);                   /* Put tty in raw mode */

pid = fork();           /* Start fork to get typeout from remote host */

- 68 -

if (pid)                        /* Parent: send type-in to remote host */

{

printmsg("connected...\r");

connected = TRUE;               /* Put us in "connect mode" */

while (connected)

{

read(0,&c,1);               /* Get a character */

if ((c&0177) == escchr)     /* Check for escape character */

{

read(0,&c,1);

if ((c&0177) == escchr)

write(ttyfd,&c,1);

else

switch (c&0177)

{

case 'c':

case 'C':

connected = FALSE;

write(0,"\r\n",2);

break;

case 'h':

case 'H':

write(0,"\r\nYes, I'm still here...\r\n",26);

break;

default:

write(0,&bel,1);

break;

}

}

else

{                           /* If not escape charater, */

write(ttyfd,&c,1);      /* write it out */

c = NULL;               /* Nullify it (why?) */

}

}

kill(pid,9);                    /* Done, kill the child */

wait(0);                        /* and bury him */

stty(0,&cookedmode);            /* Restore tty mode */

printmsg("disconnected.");

return;                         /* Done */

}

else                  /* Child does the reading from the remote host */

{

while(1)                        /* Do this forever */

{

read(ttyfd,&c,1);

write(1,&c,1);

}

}

}

- 69 -

/*

*      KERMIT utilities.

*/

clkint()                                /* Timer interrupt handler */

{

longjmp(env,TRUE);                  /* Tell rpack to give up */

}

/*

*  s p a c k

*

*  Send a Packet

*/

spack(type,num,len,data)

char type, *data;

int num, len;

{

int i;                              /* Character loop counter */

char chksum, buffer[100];           /* Checksum, packet buffer */

register char *bufp;                /* Buffer pointer */

if (debug>1)                        /* Display outgoing packet */

{

if (data != NULL)

data[len] = '\0';           /* Null-terminate data to print it */

printf("  spack type: %c\n",type);

printf("         num:  %d\n",num);

printf("         len:  %d\n",len);

if (data != NULL)

printf("        data: \"%s\"\n",data);

}

bufp = buffer;                      /* Set up buffer pointer */

for (i=1; i<=pad; i++) write(ttyfd,&padchar,1); /* Issue any padding */

*bufp++ = SOH;                      /* Packet marker, ASCII 1 (SOH) */

*bufp++ = tochar(len+3);            /* Send the character count */

chksum  = tochar(len+3);            /* Initialize the checksum */

*bufp++ = tochar(num);              /* Packet number */

chksum += tochar(num);              /* Update checksum */

*bufp++ = type;                     /* Packet type */

chksum += type;                     /* Update checksum */

- 70 -

for (i=0; i<len; i++)               /* Loop for all data characters */

{

*bufp++ = data[i];              /* Get a character */

chksum += data[i];              /* Update checksum */

}

chksum = (((chksum&0300) >> 6)+chksum)&077; /* Compute final checksum */

*bufp++ = tochar(chksum);           /* Put it in the packet */

*bufp = eol;                        /* Extra-packet line terminator */

write(ttyfd, buffer,bufp-buffer+1); /* Send the packet */

}

/*

*  r p a c k

*

*  Read a Packet

*/

rpack(len,num,data)

int *len, *num;                         /* Packet length, number */

char *data;                             /* Packet data */

{

int i, done;                        /* Data character number, loop exit */

char t,                             /* Current input character */

type,                           /* Packet type */

cchksum,                        /* Our (computed) checksum */

rchksum;                        /* Checksum received from other host */

#if UCB4X                               /* TOPS-20 can't handle timeouts... */

if (setjmp(env)) return FALSE;      /* Timed out, fail */

signal(SIGALRM,clkint);             /* Setup the timeout */

if ((timint > MAXTIM) || (timint < MINTIM)) timint = MYTIME;

alarm(timint);

#endif /* UCB4X */

while (t != SOH)                    /* Wait for packet header */

{

read(ttyfd,&t,1);

t &= 0177;                      /* Handle parity */

}

done = FALSE;                       /* Got SOH, init loop */

while (!done)                       /* Loop to get a packet */

{

read(ttyfd,&t,1);               /* Get character */

if (!image) t &= 0177;          /* Handle parity */

if (t == SOH) continue;         /* Resynchronize if SOH */

cchksum = t;                    /* Start the checksum */

*len = unchar(t)-3;             /* Character count */

read(ttyfd,&t,1);               /* Get character */

if (!image) t &= 0177;          /* Handle parity */

if (t == SOH) continue;         /* Resynchronize if SOH */

cchksum = cchksum + t;          /* Update checksum */

*num = unchar(t);               /* Packet number */

- 71 -

read(ttyfd,&t,1);               /* Get character */

if (!image) t &= 0177;          /* Handle parity */

if (t == SOH) continue;         /* Resynchronize if SOH */

cchksum = cchksum + t;          /* Update checksum */

type = t;                       /* Packet type */

for (i=0; i<*len; i++)          /* The data itself, if any */

{                               /* Loop for character count */

read(ttyfd,&t,1);           /* Get character */

if (!image) t &= 0177;      /* Handle parity */

if (t == SOH) continue;     /* Resynch if SOH */

cchksum = cchksum + t;      /* Update checksum */

data[i] = t;                /* Put it in the data buffer */

}

data[*len] = 0;                 /* Mark the end of the data */

read(ttyfd,&t,1);               /* Get last character (checksum) */

rchksum = unchar(t);            /* Convert to numeric */

read(ttyfd,&t,1);               /* get EOL character and toss it */

if (!image) t &= 0177;          /* Handle parity */

if (t == SOH) continue;         /* Resynchronize if SOH */

done = TRUE;                    /* Got checksum, done */

}

#if UCB4X

alarm(0);                           /* Disable the timer interrupt */

#endif

if (debug>1)                        /* Display incoming packet */

{

if (data != NULL)

data[*len] = '\0';          /* Null-terminate data to print it */

printf("  rpack type: %c\n",type);

printf("         num:  %d\n",*num);

printf("         len:  %d\n",*len);

if (data != NULL)

printf("        data: \"%s\"\n",data);

}

/* Fold in bits 7,8 to compute */

cchksum = (((cchksum&0300) >> 6)+cchksum)&077; /* final checksum */

if (cchksum != rchksum) return(FALSE);

return(type);                       /* All OK, return packet type */

}

/*

*  b u f i l l

*

*  Get a bufferful of data from the file that's being sent.

*  Only control-quoting is done; 8-bit & repeat count prefixes are

*  not handled.

*/

- 72 -

bufill(buffer)

char buffer[];                          /* Buffer */

{

int i,                              /* Loop index */

t;                              /* Char read from file */

char t7;                            /* 7-bit version of above */

i = 0;                              /* Init data buffer pointer */

while((t = getc(fp)) != EOF)        /* Get the next character */

{

t7 = t & 0177;                  /* Get low order 7 bits */

if (t7 < SP || t7==DEL || t7==quote) /* Does this char require */

{                                   /* special handling? */

if (t=='\n' && !image)

{                           /* Do LF->CRLF mapping if !image */

buffer[i++] = quote;

buffer[i++] = ctl('\r');

}

buffer[i++] = quote;        /* Quote the character */

if (t7 != quote)

{

t = ctl(t);             /* and uncontrolify */

t7 = ctl(t7);

}

}

if (image)

buffer[i++] = t;            /* Deposit the character itself */

else

buffer[i++] = t7;

if (i >= spsiz-8) return(i);    /* Check length */

}

if (i==0) return(EOF);              /* Wind up here only on EOF */

return(i);                          /* Handle partial buffer */

}

/*

*      b u f e m p

*

*  Put data from an incoming packet into a file.

*/

bufemp(buffer,len)

char  buffer[];                         /* Buffer */

int   len;                              /* Length */

{

int i;                              /* Counter */

char t;                             /* Character holder */

- 73 -

for (i=0; i<len; i++)               /* Loop thru the data field */

{

t = buffer[i];                  /* Get character */

if (t == MYQUOTE)               /* Control quote? */

{                               /* Yes */

t = buffer[++i];            /* Get the quoted character */

if ((t & 0177) != MYQUOTE)  /* Low order bits match quote char? */

t = ctl(t);             /* No, uncontrollify it */

}

if (t==CR && !image)            /* Don't pass CR if in image mode */

continue;

putc(t,fp);

}

}

/*

*  g n x t f l

*

*  Get next file in a file group

*/

gnxtfl()

{

if (debug) printf("   gnxtfl: filelist = \"%s\"\n",*filelist);

filnam = *(filelist++);

if (filecount-- == 0) return FALSE; /* If no more, fail */

else return TRUE;                   /* else succeed */

}

/*

*  s p a r

*

*  Fill the data array with my send-init parameters

*

*/

spar(data)

char data[];

{

data[0] = tochar(MAXPACKSIZ);          /* Biggest packet I can receive */

data[1] = tochar(MYTIME);           /* When I want to be timed out */

data[2] = tochar(MYPAD);            /* How much padding I need */

data[3] = ctl(MYPCHAR);             /* Padding character I want */

data[4] = tochar(MYEOL);            /* End-Of-Line character I want */

data[5] = MYQUOTE;                  /* Control-Quote character I send */

}

- 74 -

/*  r p a r

*

*  Get the other host's send-init parameters

*

*/

rpar(data)

char data[];

{

spsiz = unchar(data[0]);            /* Maximum send packet size */

timint = unchar(data[1]);           /* When I should time out */

pad = unchar(data[2]);              /* Number of pads to send */

padchar = ctl(data[3]);             /* Padding character to send */

eol = unchar(data[4]);              /* EOL character I must send */

quote = data[5];                    /* Incoming data quote character */

}

/*

*  f l u s h i n p u t

*

*  Dump all pending input to clear stacked up NACK's.

*  (Implemented only for Berkeley Unix at this time).

*/

#if UCB4X&(~NO_FIONREAD)

flushinput()

{

long int count;                     /* Number of bytes ready to read */

long int i;                         /* Number of bytes to read in loop */

ioctl(ttyfd, FIONREAD, &count);     /* See how many bytes pending read */

if (!count) return;                 /* If zero, then no input to flush */

while (count)                       /* Loop till all are flushed */

{

i = (count<sizeof(recpkt)) ?    /* Read min of count and size of */

count : sizeof(recpkt);     /*  the read buffer */

read(ttyfd, recpkt, i);         /* Read a bunch */

count -= i;                     /* Subtract from amount to read */

}

}

#else

flushinput()            /* Null version for non-Berkeley Unix */

{}

#endif /* UCB4X&(~FIONREAD) */

- 75 -

/*

*  Kermit printing routines:

*

*  usage - print command line options showing proper syntax

*  printmsg -  like printf with "Kermit: " prepended

*  error - like printmsg if local kermit; sends a error packet if remote

*  prerrpkt - print contents of error packet received from remote host

*/

/*

*  u s a g e

*

*  Print summary of usage info and quit

*/

usage()

{

#if UCB4X

printf("Usage: kermit c[lbe line baud esc.char]      (connect mode)\n");

printf("or:    kermit s[diflb line baud] file ...    (send mode)\n");

printf("or:    kermit r[diflb line baud]             (receive mode)\n");

#else

printf("Usage: kermit c[le line esc.char]            (connect mode)\n");

printf("or:    kermit s[difl line] file ...          (send mode)\n");

printf("or:    kermit r[difl line]                   (receive mode)\n");

#endif

exit(1);

}

/*

*  p r i n t m s g

*

*  Print message on standard output if not remote.

*/

/*VARARGS1*/

printmsg(fmt, a1, a2, a3, a4, a5)

char *fmt;

{

if (!remote)

{

printf("Kermit: ");

printf(fmt,a1,a2,a3,a4,a5);

printf("\n");

fflush(stdout);                 /* force output (UTS needs it) */

}

}

- 76 -

/*

*  e r r o r

*

*  Print error message.

*

*  If local, print error message with printmsg.

*  If remote, send an error packet with the message.

*/

/*VARARGS1*/

error(fmt, a1, a2, a3, a4, a5)

char *fmt;

{

char msg[80];

int len;

if (remote)

{

sprintf(msg,fmt,a1,a2,a3,a4,a5); /* Make it a string */

len = strlen(msg);

spack('E',n,len,msg);           /* Send the error packet */

}

else

printmsg(fmt, a1, a2, a3, a4, a5);

return;

}

/*

*  p r e r r p k t

*

*  Print contents of error packet received from remote host.

*/

prerrpkt(msg)

char *msg;

{

printf("Kermit aborting with following error from remote host:\n%s\n",msg);

return;

}

- 77 -

V. The ASCII Character Set

ASCII Code (ANSI X3.4-1968)

There  are 128 characters in the ASCII (American national Standard Code for In-

formation Interchange) "alphabet".  The characters are listed in order of ASCII

value; the columns are labeled as follows:

Bit             Even parity bit for ASCII character.

ASCII Dec       Decimal (base 10) representation.

ASCII Oct       Octal (base 8) representation.

ASCII Hex       Hexadecimal (base 16) representation.

EBCDIC Hex      EBCDIC hexadecimal equivalent for Kermit translate tables.

Char            Name or graphical representation of character.

Remark          Description of character.

The first group consists of nonprintable 'control' characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   000   000   00   00    NUL   ^@, Null, Idle

1   001   001   01   01    SOH   ^A, Start of heading

1   002   002   02   02    STX   ^B, Start of text

0   003   003   03   03    ETX   ^C, End of text

1   004   004   04   37    EOT   ^D, End of transmission

0   005   005   05   2D    ENQ   ^E, Enquiry

0   006   006   06   2E    ACK   ^F, Acknowledge

1   007   007   07   2F    BEL   ^G, Bell, beep, or fleep

1   008   010   08   16    BS    ^H, Backspace

0   009   011   09   05    HT    ^I, Horizontal tab

0   010   012   0A   25    LF    ^J, Line feed

1   011   013   0B   0B    VT    ^K, Vertical tab

0   012   014   0C   0C    FF    ^L, Form feed (top of page)

1   013   015   0D   0D    CR    ^M, Carriage return

1   014   016   0E   0E    SO    ^N, Shift out

0   015   017   0F   0F    SI    ^O, Shift in

1   016   020   10   10    DLE   ^P, Data link escape

0   017   021   11   11    DC1   ^Q, Device control 1, XON

0   018   022   12   12    DC2   ^R, Device control 2

1   019   023   13   13    DC3   ^S, Device control 3, XOFF

0   020   024   14   3C    DC4   ^T, Device control 4

1   021   025   15   3D    NAK   ^U, Negative acknowledge

1   022   026   16   32    SYN   ^V, Synchronous idle

0   023   027   17   26    ETB   ^W, End of transmission block

0   024   030   18   18    CAN   ^X, Cancel

1   025   031   19   19    EM    ^Y, End of medium

1   026   032   1A   3F    SUB   ^Z, Substitute

0   027   033   1B   27    ESC   ^[, Escape, prefix, altmode

1   028   034   1C   1C    FS    ^\, File separator

0   029   035   1D   1D    GS    ^], Group separator

0   030   036   1E   1E    RS    ^^, Record separator

1   031   037   1F   1F    US    ^_, Unit separator

The last four are usually associated with the  control  version  of  backslash,

right  square  bracket,  uparrow (or circumflex), and underscore, respectively,

- 78 -

but some terminals do not transmit these control characters.

The following characters are printable:

First, some punctuation characters.

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

1   032   040   20   40    SP    Space, blank

0   033   041   21   5A    !     Exclamation mark

0   034   042   22   7F    "     Doublequote

1   035   043   23   7B    #     Number sign, pound sign

0   036   044   24   5B    $     Dollar sign

1   037   045   25   6C    %     Percent sign

1   038   046   26   50    &     Ampersand

0   039   047   27   7D    '     Apostrophe, accent acute

0   040   050   28   4D    (     Left parenthesis

1   041   051   29   5D    )     Right parenthesis

1   042   052   2A   5C    *     Asterisk, star

0   043   053   2B   4E    +     Plus sign

1   044   054   2C   6B    ,     Comma

0   045   055   2D   60    -     Dash, hyphen, minus sign

0   046   056   2E   4B    .     Period, dot

1   047   057   2F   61    /     Slash

Numeric characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   048   060   30   F0    0     Zero

1   049   061   31   F1    1     One

1   050   062   32   F2    2     Two

0   051   063   33   F3    3     Three

1   052   064   34   F4    4     Four

0   053   065   35   F5    5     Five

0   054   066   36   F6    6     Six

1   055   067   37   F7    7     Seven

1   056   070   38   F8    8     Eight

0   057   071   39   F9    9     Nine

More punctuation characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   058   072   3A   7A    :     Colon

1   059   073   3B   5E    ;     Semicolon

0   060   074   3C   4C    <     Left angle bracket

1   061   075   3D   7E    =     Equal sign

1   062   076   3E   6E    >     Right angle bracket

0   063   077   3F   6F    ?     Question mark

1   064   100   40   7C    @     "At" sign

- 79 -

Upper-case alphabetic characters (letters):

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   065   101   41   C1    A

0   066   102   42   C2    B

1   067   103   43   C3    C

0   068   104   44   C4    D

1   069   105   45   C5    E

1   070   106   46   C6    F

0   071   107   47   C7    G

0   072   110   48   C8    H

1   073   111   49   C9    I

1   074   112   4A   D1    J

0   075   113   4B   D2    K

1   076   114   4C   D3    L

0   077   115   4D   D4    M

0   078   116   4E   D5    N

1   079   117   4F   D6    O

0   080   120   50   D7    P

1   081   121   51   D8    Q

1   082   122   52   D9    R

0   083   123   53   E2    S

1   084   124   54   E3    T

0   085   125   55   E4    U

0   086   126   56   E5    V

1   087   127   57   E6    W

1   088   130   58   E7    X

0   089   131   59   E8    Y

0   090   132   5A   E9    Z

More punctuation characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

1   091   133   5B   AD    [     Left square bracket

0   092   134   5C   E0    \     Backslash

1   093   135   5D   BD    ]     Right square bracket

1   094   136   5E   5F    ^     Circumflex, up arrow

0   095   137   5F   6D    _     Underscore, left arrow

0   096   140   60   79    `     Accent grave

- 80 -

Lower-case alphabetic characters (letters):

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

1   097   141   61   81    a

1   098   142   62   82    b

0   099   143   63   83    c

1   100   144   64   84    d

0   101   145   65   85    e

0   102   146   66   86    f

1   103   147   67   87    g

1   104   150   68   88    h

0   105   151   69   89    i

0   106   152   6A   91    j

1   107   153   6B   92    k

0   108   154   6C   93    l

1   109   155   6D   94    m

1   110   156   6E   95    n

0   111   157   6F   96    o

1   112   160   70   97    p

0   113   161   71   98    q

0   114   162   72   99    r

1   115   163   73   A2    s

0   116   164   74   A3    t

1   117   165   75   A4    u

1   118   166   76   A5    v

0   119   167   77   A6    w

0   120   170   78   A7    x

1   121   171   79   A8    y

1   122   172   7A   A9    z

More punctuation characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   123   173   7B   C0     {    Left brace (curly bracket)

1   124   174   7C   4F     |    Vertical bar

0   125   175   7D   D0     }    Right brace (curly bracket)

0   126   176   7E   7E     ~    Tilde

Finally, one more nonprintable character:

0    127   177  7F   07    DEL   Delete, rubout

- 81 -

Index

8th Bit   4                               Raw Mode   7

Records   9

ACK   6                                   Remote   4, 7

ASCII   5, 77                             Reserved Fields   15

Baud   7                                  Send-Init   13

Binary Files   8, 9                       Sequential Files   2

Binary Mode   7                           Server   4

Bit Positions   4                         SOH   7

Block Check   11

Bootstrap   45                            Text Files   9

BREAK   42                                Timeout   22

Transaction   17

Capabilies   14                           Transaction Log   20

CAPAS   14                                TTY   4

Char(x)   5

Checksum   11                             Unchar(x)   5

Control Characters   11, 77               User   4

Ctl(x)   5

XON/XOFF   7, 21, 77

Duplex   7

EBCDIC   7, 77

Edit Number   45

End-Of-Line (EOL)   7, 12

Errors   18

Fatal Errors   18

File Names   19

Flow Control   7, 21

Full Duplex   7

Half Duplex   7

Host   4

Initial Connection   13

KERMIT   2

Language, Programming   44

Line Terminator (see End-Of-Line)

Local   4

Logical Records   9

NAK   6

Normal Form for File Names   19

Packet   6, 11

Padding   7

Parity   77

Printable Files   9

Program, Kermit   43, 51

Protocol   2

- i -

Table of Contents

1. Introduction                                                               2

1.1. Background                                                               2

1.2. Overview                                                                 2

2. Definitions                                                                4

2.1. General Terminology                                                      4

2.2. Numbers                                                                  4

2.3. Character Set                                                            5

2.4. Conversion Functions                                                     5

2.5. Protocol Jargon                                                          6

3. System Requirements                                                        7

4. Printable Text versus Binary Data                                          9

4.1. Printable Text Files                                                     9

4.2. Binary Files                                                             9

5. Packet Format                                                             11

6. Initial Connection                                                        13

7. File Transfer                                                             17

7.1. Timeouts, NAKs, and Retries                                             18

7.2. Errors                                                                  18

7.3. Heuristics                                                              19

7.4. File Names                                                              19

7.5. Robustness                                                              20

7.6. Flow Control                                                            21

8. Basic KERMIT Protocol State Table                                         22

9. Optional Features                                                         25

9.1. 8th-Bit and Repeat Count Prefixing                                      25

9.2. Server Operation                                                        26

9.2.1. The R Command                                                    27

9.2.2. Short and Long Replies                                           28

9.2.3. Additional Server Commands                                       28

9.2.4. Host Commands                                                    29

9.2.5. Exchanging Parameters Before Server Commands                     29

9.3. Alternate Block Check Types                                             30

9.4. Interrupting a File Transfer                                            31

9.5. Transmitting File Attributes                                            31

10. KERMIT Commands                                                          36

- ii -

10.1. Basic Commands                                                         36

10.2. Program Management Commands                                            36

10.3. Terminal Emulation Commands                                            37

10.4. Special User-Mode Commands                                             37

10.5. Commands Whose Object Should Be Specified                              37

10.6. The SET Command                                                        39

10.7. Macros                                                                 40

11. Terminal Emulation                                                       41

12. Writing a KERMIT Program                                                 43

12.1. Program Organization                                                   43

12.2. Programming Language                                                   44

12.3. Documentation                                                          45

12.4. Bootstrapping                                                          45

I. Packet Format and Types                                                   46

II. List of Features                                                         47

III. Unresolved Issues                                                       49

IV. A KERMIT Program                                                         51

V. The ASCII Character Set                                                   77

Index                                                                        81