The Kermit protocol and the PDP-11

Brian Nelson   4-OCT-1985 14:10

Abstract

This article will describe the  author's  implementation  of  the

Kermit  file transfer protocol for the PDP-11 series under RSTS/E,

RSX11M/M+, P/OS RT11 and TSX+.  This protocol allows many (if  not

most)  types of computer systems to effect, at minimum, error free

file  transfer  with  other  systems   and   microcomputers   over

asynchronous lines.

The first obvious use of  any  program  or  protocol  designed  to

accomplish  file  transfer is to be able to provide the ability to

support file uploads and downloads from superminis such as the VAX

and  PDP-11  to  remote  personal  computers,  such  as the PC and

Rainbow.  Since as of this date (03-Oct-1985) there are  available

over 160 versions of Kermit available for numerous micro, mini and

mainframe configurations, Kermit addresses this need quite well.

Other uses of Kermit are quite numerous.  I routinely  use  Kermit

for  transfering  software  developed  for the PRO/350 on a RSTS/E

11/23+ host as well as using the PDP-11/44 and VAX 11/785 I run at

the University of Toledo for dialing out to other systems, such as

the LCG  Tops  20  system  and  the  LDP  public  domain  library.

Considering  that  there  exists  a  Kermit  for  almost  any  DEC

configuration one can even use Kermit as a poor man's Decnet.   In

my  case,  I have a DMF modem port from the 11/785 and a DZ11 port

from the 11/44 connected to the Gandalf  PACX  front  end  switch,

which  allows  me  to  connect  either  system to any of the other

systems on the PACX, which includes an IBM 370  compatible  system

as well as connecting the VAX to the PDP-11.

With the knowledge that there are Kermit implementations for  most

personal  computers  in  use  it  becomes apparent that the Kermit

standard is well worth looking in to.  A list of versions  running

on Digital hardware follows the article.

The Kermit protocol

The  Kermit  protocol  is  designed   to   operate   over   normal

asynchronous   terminal   lines.    All   data  and  commands  are

transferred with a packet oriented protocol, basically  consisting

of a start of packet character (normally SOH), followed by length,

control, data and checksum fields.  Communications is half duplex,

in  that for every packet sent, the sender must wait for either an

acknowledgement packet (ACK) or a negative acknowledgement  packet

(NAK).   Transmission is in ascii, with no requirements needed for

the transmission of eight bit  characters  or  control  characters

other  than  the  choice  of  control-A for marking the start of a

packet.   All  'control'  characters  imbedded  in  the  data  are

prefixed  to  convert  them  to  printable  characters,  the  same

Page 2

applying  to   eight   bit   characters   if   required   by   the

characteristics  of  the  line.   Since  there  are many different

implementations of Kermit, the protocol provides  a  mechanism  by

which  the capabilities of two connected Kermits can be negotiated

to allow  for  differences  in  the  level  of  protocol  support.

Examples  of  protocol  features  that  not all Kermits understand

include data compression and transfer of file attributes.

Rather than to go into more detail about the the Kermit protocol,

the reader should consult the references listed at the end of this

article.

The PDP-11 Kermit-11 implementation

The author's version of Kermit-11 is written in Macro-11 and  can

run  on  RSTS/E, RSX11M, RSX11M Plus, P/OS and RT11.  The RSTS and

RSX file system interface is via RMS11 version 2, while  the  RT11

interface  attempts to emulate the RMS11 subsystem.  The choice of

Macro-11 for the implementation  language  was  made  for  several

reasons,  one  being  the  availability  of  the  assembler on all

systems and another being speed and compactness of the code.

RMS11 was used for  RSTS/E  and  RSX  to  provide  a  common  i/o

interface  to  the  host  file system.  Additionally, Bob Denny of

Alisa Systems further extended the RMS interface to support remote

file  access  over  DECNET  with Kermit, allowing commands such as

SEND  NODENAME::[BRIAN.FUBAR]FILE.TYPE  and  other   remote   file

accesses over DECNET.  RMS11 version 2 also provides a very simple

and powerfull means of doing wildcard searching, file renames  and

file deletion via the $PARSE, $SEARCH, $RENAME and $DELETE macros.

Points against RMS basically amount to it's  size,  RMS  is  quite

large  even  if  overlayed.  This is helped by using the segmented

RMSRES available on RSTS/E and RSX11M Plus,  though  there  is  no

remote file access for RMSRES in the current release of Kermit-11.

The other objection to RMS will come from RSTS/E  users,  who  are

used  to  using files that normally lack file attributes.  This is

overcome by the ability of RMS v2 to create stream ascii files.

The RSTS/E Kermit, while it does 'run' under RSX  emulation,  does

NOT use any RSX directives (apart from GTSK$S) to interface to the

executive, as (one) the RSX directive emulation  under  RSTS/E  is

only a small subset of 'real' RSX and (two) there is no need to go

though an additional layer of overhead  to  make  RSTS/E  map  RSX

calls  to native calls.  The 'multiple private delimiters' feature

is used to avoid losing  read  pass  all  (binary)  mode  on  read

timeouts,  as well as setting the link to '8-bit' mode to keep the

terminal driver from stripping the high bit from data received.

The RSX11M/M+ and P/OS versions of Kermit-11, like the RSTS/E  and

RT  versions,  receive  eight bit data assuming no parity is used.

Where parity is a must, Kermit-11 has to use  a  prefixing  scheme

for  eight bit binary data.  Like the RSTS/E version, binary files

are created as FIXED no carriage control files such  as  used  for

task  images.   Note that parity generation is done by software in

Kermit-11.  The P/OS version runs under control of DCL.  The  next

release of Kermit-11, which will be 2.37, will include support for

the PRO TMS (Telephone Management System) option.

Page 3

The RT11 and TSX+ version of Kermit-11  maintains  source  module

compatability  with  the RSTS/E and RSX versions.  Each version of

Kermit-11 has it's own source file  to  deal  with  the  operating

system,  for  RSX it is K11M41.MAC, for RSTS/E they are K11E80.MAC

and K1180S.MAC, and for RT11 they are  called  K11RT*.MAC.   Apart

from these specific files, all other source files are shared.  The

RT11 Kermit-11 can use either the version 5.x XL  and  XC  handler

for high throughput, or it can use multiple terminal service to do

all its terminal i/o.  This second option allows the  use  of  any

interface  supported,  including the PDT150 modem port, DL/DLV11's

and DZ/DZV11's.  The drawback is overhead,  the  RT11  MT  service

can't  sustain  a rate much past 1200 baud at most.  This is not a

problem for Kermit, however, due to it's half  duplex  nature  and

the  fact  that  no  packet  received is ever longer than the ring

buffer size.  The only problem is in when Kermit-11 is running  as

a  terminal  emulator  (the Kermit CONNECT command) where the data

coming from the remote host can  easily  overrun  the  executive's

buffer.   A SET RT11 [NO]FLOW command was added to force Kermit-11

to send its own flow control to the host via XON and  XOFF.   TSX+

users  can  connect  to  CL:   for dialing out, the exact means is

documented in the Kermit-11 users guide.  The  disk  i/o  emulates

the RSTS/E and RSX RMS11 version, and each executive directive has

its error codes mapped into an unique global error code, with  the

symbolic  names  corresponding  to  the  nearest RMS11 error name.

Wildcarding is handled, of course, by non  file-structured  access

to  the  directory  on  the desired volume, and supports full RT11

wildcard filenames.

Transmission of file attributes

One of the  optional  features  of  the  Kermit  protocol  is  the

ATTRIBUTE packet.  The attribute packets allow a Kermit program to

send  to  a  receiving  Kermit  information  regarding  the   file

organization,  size,  cluster/retrieval  size,  protection  and so

forth.  There is even a system  dependant  attribute  packet  type

that  can  be  used  to  transfer  things like the RMS11 IFAB (the

RMS/FCS attributes).  One of the things that two Kermits  exchange

before  any  file  transfer  is an information packet, this packet

tells the receiving Kermit about itself.  The last field  in  this

packet,  the CAPAS mask, tells Kermit if the other one can process

attribute packets.  If two  Kermit-11's  are  communicating,  they

will  find that each can do so, and the sender of a file will then

send over attribute packets indicating the need (or lack  of)  for

binary  transmission, based on the file organization, filetype and

protection code (for RSTS/E).  If the sending Kermit-11 is running

on  RSTS/E,  RSX11M/M+  or  P/OS  it  will also send a copy of the

RMS/FCS attributes so the received file will be identical (to  FCS

and/or  RMS)  to  the  copy  on  the sender's system.  Since other

implementations of Kermit may use this  special  system  attribute

packet,  Kermit-11  always  sends  an attribute packet telling the

receiver what hardware and operating system it is running on,  and

thus  will  only use such data if they are compatible.  Of course,

there will be times when a file may be binary and Kermit-11  can't

tell  so,  many Kermit's have a SET FILE BINARY and SET FILE ASCII

to allow the user to override defaults.  Kermit-11 also has a  SET

Page 4

FILE  AUTO/NOAUTO  to disable it from trying to determine a file's

binary status.

Future directions

With  the  advent  of  packet  switched  networks  and   satellite

communications  the  Kermit  protocol  will  likely be extended to

increase efficiency over such links.  The main problem is the half

duplex  nature  of Kermit, the packet acknowledgements can take up

to several  seconds  in  transit  thus  drastically  reducing  the

throughput.   There are several possibilities under discussion and

a standard should be appearing shortly.

Page 5

Summary

This article describes only the PDP-11 Kermit-11  implementation,

for further reading see:

Kermit: A File-transfer Protocol for Universities

Frank da Cruz and Bill Catchings

BYTE Magazine, June/July 1984

The Kermit Protocol Manual, version 5

Frank da Cruz   April 1984

Columbia University Center for Computing Activities

Information on obtaining Kermit:

KERMIT Distribution

Columbia University Center for Computing Activities

7th Floor, Watson Laboratory

612 West 115th Street

New York, N.Y.  10025

Kermit is also usually found on the Decus symposia SIG tapes.

Kermit-11 is available from DECUS as number 11-731

Digital hardware that Kermit is currently available for:

Operating  Program

Machine          System     Language     Contributor

DEC PDP-11       MUMPS-11   MUMPS-1982   Cornell U

DEC PDP-11       RSTS/E     Macro-11     U of Toledo

DEC PDP-11       RSX-11/M   Macro-11     U of Toledo

DEC PDP-11       RSX-11/M+  Macro-11     U of Toledo

DEC PDP-11       RT-11      Macro-11     U of Toledo

DEC PDP-11       RT-11      OMSI Pascal  U of Toronto

DEC PDP-11       TSX+       Macro-11     U of Toledo

DEC PDP-11       Unix 2xBSD C            Columbia U

DEC PDP-11, ...  Unix V7    C            Columbia U

DEC PDP-8        OS8, RTS8  PAL-8        R. Hippe

DEC Pro-3xx      P/OS       Bliss, Macro Stevens I.T.

DEC Pro-3xx      P/OS       Macro-11     U of Toledo

DEC Pro-3xx      Pro/RT     Macro-11     U of Toledo

DEC Pro-3xx      Venix V1   C            Columbia U

DEC Pro-3xx      Venix V2   C            Columbia U

DEC Rainbow      CPM86      ASM86        Columbia U

DEC Rainbow      MS-DOS     MASM         Columbia U

DEC Rainbow      QNX 1.x    C            Merrell-Dow

DEC VAX          Ultrix-32  C            Columbia U

DEC VAX          VMS        Bliss,Macro  Stevens I.T.

DEC VAX          VMS        C (VAX-11 C) Columbia U

DEC VAX          VMS        Pascal       U of Toronto

DEC VAX, ...     Unix 4xBSD C            Columbia U

DEC VT-180 Robin CPM80      Turbo Pascal Jeff Duncan

DEC VT-180 Robin CPM80 2.2  M80,LASM     ACC

Page 6

DECmate-II,III   CPM80 2.2  M80,LASM     ACC

DECsystem-10     TOPS-10    Bliss, Macro Stevens I.T.

DECSYSTEM-20     TOPS-20    MACRO-20     Columbia U