Terminal Security System

Product Description

Cargill Grain Lab

Randy Baldwin

November 7, 1985

SM:[1,100]SCLIBRIEF.DOC

SCLI Overview                                         Page 2

1.  Introduction

This document provides a brief product  description  of

the  security CLI - SCLI.  It contains a description of what

SCLI provides, how it is implemented and maintainance proce-

dures.

1.1  SCLI Overview

SCLI is a user-written command line interpreter,  which

provides  additional access protection to one or more serial

ports, beyond that provided by the RSX login procedure.  The

secure system consists of three 'components':  the SCLI task

itself, a modified RSX BYE program, and a patch to  the  RSX

TT  driver.   This system has been designed to operate fully

un-attended.  It will be run at system  startup,  and  never

shut  down.  In fact, the system is very difficult to elimi-

nate.

SCLI implements access protection by simply sitting  on

the  terminal(s), absorbing all input.  When SCLI receives a

valid password on a protected terminal, it gives control  of

the  terminal to MCR, and remains 'quiet' until the terminal

is logged off.  At that point, BYE  gives  control  back  to

SCLI, re-establishing protection.

When SCLI is first  started,  normally  just  following

system  boot,  it  takes control of the protected terminals.

As long as SCLI remains  active  (normally  'forever'),  the

protected  terminals are in one of three states:  protected,

logged on, or disabled.  A terminal is protected  when  SCLI

is  that  terminal's  CLI.  A logged on terminal has already

been 'admitted' to the system, and is  in  use  by  a  valid

user.   A terminal can become disabled in the unlikely event

that SCLI is shut down.  In this state, no CLI is  connected

to  the  terminal,  and it is effectively slaved.  Note that

this state occurs briefly during  system  shutdown,  between

the  time  that  SHUTUP  disables the CLI, and the time that

shutdown completes.

SCLI places terminals which it is protecting in one  of

two  internal  states:   normal, and mumble mode.  In normal

mode, SCLI will accept a valid password, and give  the  user

access  to  the  system.   In mumble mode, SCLI has detected

unauthorized attempts to access the  system;  all  input  is

treated as invalid, and the user will 'never' gain access to

the system.

SCLI Overview                                         Page 3

Mumble mode is triggered by a preset number of  sequen-

tial  invalid login attempts.  Once in mumble mode, the (po-

tential) user must wait a preset time interval before  again

attempting  to  log  in.  If the timer expires, and no input

has been received, SCLI assumes that the villian  has  left,

and  a  new user is attempting to gain access.  On the other

hand, if the user keeps  trying,  SCLI  simply  re-sets  the

timer, perpetuating mumble mode.

SCLI maintains a log of all activity on both  the  Con-

sole  (CO:  or COT), and the system error log file.  The log

to CO:  is more detailed, since it is assumed that the  con-

sole  log  will be purged more frequently than the error log

file.

SCLI is also sensitive to messages issued via  the  MCR

CLI  commands.  There are currently three messages:  two en-

able/disable dumping of all invalid  input  to  the  console

file.  These are used for observing activity during attempt-

ed break-ins to the system.  The third message provides  the

only  known  means  of  shutting SCLI down while keeping the

system running.

1.2  SCLI Implementation.

This chapter describes the process required  to  imple-

ment SCLI on a operating RSX system.

1.2.1  Programs and System modifications.  --

The SCLI product consists of  three  related  programs.

One,  SCLI,  is a user written program and the other two are

RSX11M (DEC) supplied programs that are modified to fit with

SCLI.

1.2.1.1  SCLI program --

The SCLI program must be  rebuilt  and  configured  for

each  site.   The reason for this is each site (usually) has

differant needs for secure terminals.  (i.e.  all modems are

not on the same port).  Also passwords and user messages are

unique for each installation.

Password modifications is is accomplished by rebuilding

several  modules of the SCLI program thus user password mod-

SCLI Overview                                         Page 4

ification is not possible.  This is also an  added  security

benefit as it places control over the passwords.

1.2.1.2  BYE program modifications.  --

The modifications to BYE, RSX's log  off  program,  are

required  to  inform BYE which terminals are to be protected

and thus connected to SCLI when they  are  logged  off.   If

terminals  are to be added or removed from the security pro-

gram BYE will have to be modified to reflect these  changes.

(This  may  not require rebuilding as it may be done on-site

via ZAP depending upon the situation).

1.2.1.3  TTDRV program modifications.  --

The modifications to TTDRV, RSX's terminal driver  pro-

gram,  are required to prevent TTDRV from automatically set-

ting a remote (modem) port's CLI to MCR when it is answered.

This  mod  may be done by rebuilding the TTDRV program or by

using ZAP or OPEN/SAVE to alter the tasks code.  This  is  a

one  time modification that must be done and is not affected

by adding or subtracting terminals from SCLI or by  password

modification.

1.3  Related Documents

There are several documents available to further expla-

in and aid in the configuration and modification of the SCLI

product.  These documents are available upon request and are

listed below.

o  SCLIMAN.DOC -- This is the technical support  manual

for  SCLI.   It  contains  module design information

along with overall  design  guidelines.   Additional

information  is  provided for the use of maintainers

of the program, including instructions for modifying

passwords,  response lines, protected terminals, and

so on.  A chapter describing  modifications  to  the

RSX  system  required  by the security system is in-

cluded, along with instructions for  installing  the

changes.

o  SCLIREV.DOC -- This document summarizes design  fea-

tures of the dial-in security project.

SCLI Overview                                         Page 5

o  SCLISUG.DOC -- This document presents user's  infor-

mation  for  the terminal security system.  Included

in this document are descriptions of the normal  op-

eration of the secure system, including instructions

for obtaining reports of SCLI  activity.   Also  in-

cluded  are  descriptions of the various log entries

and error messages produced by SCLI.

CONTENTS

1.  Introduction . . . . . . . . . . . . . . . . . . . . . 2

1.1  SCLI Overview . . . . . . . . . . . . . . . . . . 2

1.2  SCLI Implementation.  . . . . . . . . . . . . . . 3

1.2.1  Programs and System modifications. . . . . 3

1.2.1.1  SCLI program  . . . . . . . . . . 3

1.2.1.2  BYE program modifications.  . . . 4

1.2.1.3  TTDRV program modifications.  . . 4

1.3  Related Documents . . . . . . . . . . . . . . . . 4