TCP/IP Package

for

TSX-Plus

CHAPTER 1  SOFTWARE INSTALLATION                             1-1

1.1     INSTALLATION CONFIGURATIONS                        1-1

1.1.1     Install All Files on System Device               1-2

1.1.2     Install Only Command Files on System Device      1-2

1.1.3     Install All Files on a Logical Disk              1-2

1.2     SOFTWARE COMPONENTS                                1-2

1.3     INSTALLATION IN DETAIL                             1-6

CHAPTER 2  HARDWARE INSTALLATION                             2-1

2.1     Q-BUS SYSTEMS                                      2-1

2.2     UNIBUS SYSTEMS                                     2-2

2.3     SERIAL PORT INSTALLATION                           2-2

CHAPTER 3  TSX-PLUS SYSTEM GENERATION                        3-1

3.1     BUSTYPE                                            3-1

3.1.1     TSGEN Unibus Mapping Modification                3-2

3.2     NGR                                                3-3

3.3     DEVXMR                                             3-3

3.4     IOABT                                              3-4

3.5     DEVDEF                                             3-4

3.6     MESSAGE COMMUNICATION CHANNELS                     3-5

3.7     LINE DEFINITIONS                                   3-6

3.8     DETACH                                             3-7

3.9     RECORD LOCKING                                     3-7

3.10    USER COMMAND LINKAGE                               3-8

CHAPTER 4  PROGRAM OPTIONS                                   4-1

4.1     TCPIP                                              4-1

4.2     SKTMON                                             4-7

4.3     PASWRD                                             4-8

4.4     TELNET                                             4-9

4.5     RTELNT                                            4-10

4.6     CNCT                                              4-11

4.7     RCNCT                                             4-12

4.8     FTP                                               4-13

4.9     FTPL                                              4-15

4.10    RFTP                                              4-17

4.11    MAIL                                              4-18

4.12    MMAIL                                             4-20

4.13    RPOP3                                             4-22

4.14    MAILER                                            4-23

4.15    RSMTP                                             4-25

4.16    LLPQRM                                            4-27

4.16.1    LLPC                                            4-28

4.16.2    LLPQ                                            4-28

4.16.3    LLPR                                            4-28

4.16.4    LLPRM                                           4-28

4.17    RLPD                                              4-29

4.18    LPRINT                                            4-30

4.19    LPQRM                                             4-32

4.19.1    LPC                                             4-34

4.19.2    LPQ                                             4-34

4.19.3    LPR                                             4-34

4.19.4    LPRM                                            4-34

4.20    NTSND                                             4-34

4.21    RHTTP                                             4-35

Page ii

4.22    RGOPH                                             4-37

4.23    UTIL                                              4-42

4.23.1    LCD                                             4-43

4.23.2    LDEL                                            4-43

4.23.3    LDIR                                            4-43

4.23.4    LLS                                             4-43

4.23.5    LMKDIR                                          4-43

4.23.6    LPRO                                            4-44

4.23.7    LRENAM                                          4-44

4.23.8    LRMDIR                                          4-44

4.23.9    LUNP                                            4-44

4.24    LDISKS                                            4-44

4.25    ABRTCP                                            4-45

4.26    ABRTJB                                            4-45

CHAPTER 5  ETHERNET DEVICE HANDLERS                          5-1

5.1     DEVICE HANDLER SYSTEM SUMMARY                      5-3

5.2     DEVICE HANDLER FEATURE SUMMARY                     5-4

5.3     PROGRAMMING                                        5-5

5.4     USE OF SPECIAL FUNCTIONS (.SPFUN)                  5-5

5.4.1     Unit Specification (.SPFUN 200)                  5-6

5.4.1.1     Allocate Unit                                  5-7

5.4.1.2     Deallocate Unit                                5-8

5.4.2     Promiscuous Mode (.SPFUN 201)                    5-9

5.4.2.1     Enable Promiscuous Mode                        5-9

5.4.2.2     Disable Promiscuous Mode                      5-10

5.4.3     Protocol Type (.SPFUN 202)                      5-11

5.4.3.1     Enable Protocol Type                          5-11

5.4.3.2     Disable Protocol Type                         5-12

5.4.4     Multicast Address (.SPFUN 203)                  5-13

5.4.4.1     Enable Multicast Address                      5-13

5.4.4.2     Disable Multicast Address                     5-14

5.4.5     Transmit Ethernet Frame (.SPFUN 204)            5-15

5.4.6     Receive Ethernet Frame (.SPFUN 205)             5-16

5.4.7     Frame Queueing (.SPFUN 206)                     5-17

5.4.7.1     Enable Frame Queueing                         5-18

5.4.7.2     Disable Frame Queueing                        5-18

5.4.8     Get Ethernet Address (.SPFUN 207)               5-19

5.4.9     Get Ethernet Status Block (.SPFUN 210)          5-20

5.4.10    Abort Job I/O (.SPFUN 211)                      5-21

5.5     HANDLERS FOR TSX-PLUS V6.0 THROUGH V6.2           5-21

5.5.1     Get PLAS Allocation Size (.SPFUN 300)           5-22

5.5.2     Initialize Handler (.SPFUN 301)                 5-22

5.5.3     ENPLAS.SAV - Allocating the PLAS Region         5-23

CHAPTER 6  BUILDING THE TCP/IP PACKAGE                       6-1

6.1     C COMPILER                                         6-1

6.2     TCP/IP PACKAGE                                     6-2

6.3     COMPILING OPTIONS                                  6-2

CHAPTER 7  DISTRIBUTION DISKS                                7-1

Page iii

APPENDIX A  HIDDEN OPTIONS                                   A-1

A.1     DISABLE PASSWORD                                   A-1

A.2     SUBDIRECTORY UNIT SELECTION                        A-1

Page iv

TCP/IP Package for TXS-Plus

Submitted by Alan R.  Baldwin,

Kent State University, Kent, Ohio 44242

Operating System:  TSX-Plus(RT-11)

Source Langauge:  DECUS C and RT-11 Macro

Abstract:

The 'TCP/IP Package for TSX-Plus' provides TCP/IP networking

services for multiple users with TSX-Plus.  The package is an

assemblage of server and client programs designed to provide

Telnet, FTP, Mail, POP3, LPD, and HTTP services for local and

remote users of TSX-PLUS systems.  The programs are written in C

and RT-11 macro.  A security system allows usernames/passwords

and authorizations for each client/server component to be speci-

fied for each user.

The sources for the 'TCP/IP Package for TSX-Plus' include

header, command, C language, and macro files.  The DECUS C com-

piler (with some bug fixes) and AS assembler are included with

the software distribution to allow the package to be recompiled

from the supplied sources.

The current implementation uses 8 global regions to support

multiple client/server programs simultaneously.  The number of

regions used by each service is TELNET(1), RTELNT(1), FTP(2),

FTPL(2), RFTP(2), MAIL(1), RSMTP(1), RPOP3(1), MMAIL(1),

MAILER(1), CNCT(1), RCNCT(1), RHTTP(1), RGOPH(1), NTSND(1),

LPQRM(1), and RLPD(1).

The hardware requirements are a TSX-PLUS system having a

minimum of 512k bytes of memory, a DEQNA/DELQA (Q-Bus) or

DEUNA/DELUA (Unibus) ethernet card with associated thick or thin

wire adapter, and optionally a pair of dedicated serial lines

for each simultaneously active remote Telnet session.

Alan R. Baldwin

Physics Department

Kent State University

Kent, Ohio  44242

tel: (330) 672 2531

fax: (330) 672 2959

Internet:

baldwin@shop-pdp.kent.edu

baldwin@ksuvxd.kent.edu

CHAPTER 1

SOFTWARE INSTALLATION

The TCP/IP Package software components should be installed as

described  in  the  following  sections.   The  ethernet  device

handler  specific to your particular hardware configuration must

be copied to the TSX-PLUS system device.  The logical  name  as-

signments used by the package are:

1.  TCP:   Location of client/server programs

and assorted configuration, message,

and help files.

2.  PAS:   Location of password files.

3.  ENx:   Selected device driver units 0-3.

The  logical  name  assignments  MAI:, location of mailboxes,

HTP:, location of HTTP home directory, and GPH:, location of GO-

PHER files, may be used in various command files.

The TCP/IP Package for TSX-Plus executables are found on log-

ical disks TSDSK1.DSK, TSDSK2.DSK, and TSDSK3.DSK.  The required

device handlers are found on the logical disk DVRDSK.DSK.

1.1  INSTALLATION CONFIGURATIONS

The TSX-Plus package may be installed in one of the following

three configurations:

SOFTWARE INSTALLATION                                   PAGE 1-2

INSTALLATION CONFIGURATIONS

1.1.1  Install All Files on System Device

All  command  files  and  client/server  programs  are copied

directly to your system disk.  Assign SY:  to TCP:.  This confi-

guration  puts  everything  in  one  place but installs about 90

files (and 2100 blocks) on your system disk.

1.1.2  Install Only Command Files on System Device

Create  a  seperate  logical  disk (assigned as TCP:) for the

client/server programs and perhaps seperate  logical  disks  for

the  password  files (assigned as PAS:) and mail boxes (assigned

as MAI:).  Copy the command files to the system directory.  This

will  install  about  50  files  (and  50 blocks) on your system

device.

1.1.3  Install All Files on a Logical Disk

Create  a  seperate  logical  disk (assigned as TCP:) for the

package and perhaps seperate  logical  disks  for  the  password

files  (assigned  as  PAS:)  and  mail boxes (assigned as MAI:).

Copy all the files to the TCP:  disk.   This  configuration  re-

quires  the  user  to  specify  the TCP:  logical when using any

TCP/IP command file or program but removes all  files  from  the

system  disk.   CCL  commands may be defined to obviate the need

for using TCP:, see the file TSXUCL.INS for a  complete  set  of

definitions.   Edit the file to include only the CCL definitions

you wish defined.

1.2  SOFTWARE COMPONENTS

The   following   annotated   list  summarizes  the  software

components:

1.  TCP/IP Network Server

1.  DETACH.TSX       detached job startup command file

2.  TCPIP.COM        TCPIP startup command file

3.  TCPIP.SAV        TCPIP network server Program

4.  TCPIP.CFG        TCPIP configuration file

2.  Telnet Client

1.  TELNET.COM       TELNET startup command file

2.  TELNET.SAV       TELNET client program

SOFTWARE INSTALLATION                                   PAGE 1-3

SOFTWARE COMPONENTS

3.  Remote TELNET Server

1.  RTELNT.SRV       TELNET server detached job startup

2.  RTELNT.COM       TELNET server startup command file

3.  RTELNT.SAV       TELNET server program

4.  RTELNT.MSG       TELNET server message file

5.  RTELNT.HLP       TELNET server help file

4.  CNCT Client

1.  CNCT.COM         CNCT startup command file

2.  CNCT.SAV         CNCT client program

5.  Remote CNCT Server

1.  RCNCT.SRV        CNCT server detached job startup

2.  RCNCT.COM        CNCT server startup command file

3.  RCNCT.SAV        CNCT server program

4.  RCNCT.MSG        CNCT server message file

5.  RCNCT.HLP        CNCT server help file

6.  FTP Client

1.  FTP.COM          FTP startup command file

2.  FTP.SAV          FTP client program

3.  FTPCMD.TXT       FTP command help text

7.  FTPL Client

1.  FTPL.COM         FTPL startup command file

2.  FTPL.SAV         FTPL client program

8.  Remote FTP Server

1.  RFTP.SRV         FTP server detached job startup

2.  RFTP.COM         FTP server startup command file

3.  RFTP.SAV         FTP server program

4.  RFTP.MSG         FTP server message file

5.  RFTP.HLP         FTP server help file

9.  MAIL Client

1.  MAIL.COM         MAIL startup command file

2.  MAIL.SAV         MAIL client program

3.  MAICMD.TXT       MAIL command help text

10.  MMAIL Client

1.  MMAIL.COM        MMAIL startup command file

2.  MMAIL.SAV        MMAIL client program

3.  MMACMD.TXT       MMAIL command help text

SOFTWARE INSTALLATION                                   PAGE 1-4

SOFTWARE COMPONENTS

11.  POP3 Mail Server

1.  RPOP3.SRV        RPOP3 server detached job startup

2.  RPOP3.COM        RPOP3 server startup command file

3.  RPOP3.SAV        RPOP3 server program

12.  MAILER Task

1.  MAILER.SRV       MAILER task detached job startup

2.  MAILER.COM       MAILER task startup command file

3.  MAILER.SAV       MAILER task program

13.  Remote MAIL Server

1.  RSMTP.SRV        MAIL server detached job startup

2.  RSMTP.COM        MAIL server startup command file

3.  RSMTP.SAV        MAIL server program

4.  RSMTP.HLP        MAIL server help file

14.  Local Printer Utilities

1.  LLPC.COM         LLPC local start printer queue

2.  LLPQ.COM         LLPQ local printer queue status

3.  LLPR.COM         LLPR local print command

4.  LLPRM.COM        LLPRM local remove print job

5.  LLPQRM.SAV       LLPxx local printer control program

15.  Remote LPD Server

1.  RLPD.SRV         RLPD server detached job startup

2.  RLDP.COM         RLPD server startup command file

3.  RLPD.SAV         RLPD server program

16.  Printer Daemon

1.  LPRINT.SRV       LPRINT task detached job startup

2.  LPRINT.COM       LPRINT task startup command file

3.  LPRINT.SAV       LPRINT task program

4.  LPDQRM.CFG       LPRINT configuration file

17.  Remote Printer Utilities

1.  LPC.COM          LPC local start printer queue

2.  LPQ.COM          LPQ local printer queue status

3.  LPR.COM          LPR local print command

4.  LPRM.COM         LPRM local remove print job

5.  LPQRM.SAV        LPxx local printer control program

18.  Remote Binary File Printing

1.  NTSND.COM        NTSND client startup command file

2.  NTSND.SAV        NTSND client program

SOFTWARE INSTALLATION                                   PAGE 1-5

SOFTWARE COMPONENTS

19.  Remote HTTP Web Server

1.  RHTTP.SRV        RHTTP server detached job startup

2.  RHTTP.COM        RHTTP server startup command file

3.  RHTTP.SAV        RHTTP server program

20.  Remote Gopher Server

1.  RGOPH.SRV        RGOPH server detached job startup

2.  RGOPH.COM        RGOPH server startup command file

3.  RGOPH.SAV        RGOPH server program

4.  RGOPH.CFG        RGOPH server configuration list

21.  Password and Authorization Files

1.  PASWRD.COM       Password startup command file

2.  PASWRD.SAV       Password program

3.  PASWRD.FIL       Password and authorization data file

22.  Client/Server Monitor

1.  SKTMON.COM       Monitor startup command file

2.  SKTMON.SAV       Monitor program

23.  Subdirectory Utilities

1.  LCD.COM          Change directory command file

2.  LDEL.COM         Delete file command file

3.  LDIR.COM         List directory command file

4.  LLS.COM          List directory command file

5.  LMKDIR.COM       Make directory command file

6.  LPRO.COM         Protect file command file

7.  LRENAM.COM       Rename file command file

8.  LRMDIR.COM       Delete Subdirectory command file

9.  LUNP.COM         Unprotect file command file

10.  UTIL.SAV         Utilities program

24.  Subdirectory Summary Utility

1.  LDISKS.COM       Subdirectory summary command file

2.  RFTHLP.COM       Generate RFTP.HLP command file

3.  LDISKS.SAV       Subdirectory summary program file

25.  Miscellaneous Files

1.  ABRTCP.SAV       Abort all Client/Server programs

2.  ABRTJB.COM       Abort Client/Server Job command file

3.  ABRTJB.SAV       Abort Client/Server Job program

4.  KILLM.SAV        Kill all Client/Server Messages

5.  REMOVE.COM       Remove all Client/Server global regions

SOFTWARE INSTALLATION                                   PAGE 1-6

INSTALLATION IN DETAIL

1.3  INSTALLATION IN DETAIL

Complete  the  hardware  installation  as  described  in  the

chapter titled Hardware Installation.

Under TSX-Plus the most convenient installation is:

1.  Create TSXTCP.DSK[2400] as TCP:,

a logical disk for the TCP/IP files

2.  Create PASWRD.DSK[200] as PAS:,

a logical disk for the password files

3.  Optionally Create MAIL.DSK[nnn],

a logical disk for the MAIL directory

4.  Optionally Create HTTP.DSK[nnn],

a logical disk for the HTTP home directory

5.  Optionally Create GOPHER.DSK[nnn],

a logical disk for the GOPHER directory

NOTE

The  MAIL,  HTTP,  and GOPHER directories are

needed only if you are going to use the mail,

http,  or  gopher  servers.   The size of the

MAI:, HTP:, and GPH:  logical disks are what-

ever is needed.

6.  Copy  the  contents  of  the distribution disks TSDSK1,

TSDSK2, and TSDSK3 to TCP:.

7.  Copy  and  rename (as EQ.TSX or EU.TSX) the appropriate

device handler from the logical disk DVRDSK.DSK to SY:.

(See  the  chapter on device handlers for more informa-

tion on selecting the appropriate device handler.)

8.  Edit  the  file  TCP:TSXUCL.INS to include the CCL com-

mands you want.

9.  Create  a  command file which will mount and assign the

logical   disk   for   TCP:    and   add    the    line

'@TCP:TSXUCL.INS' at the end.  Include this file in the

LINEx.TSX startup files.

10.  Copy PASWRD.SAV and PASWRD.FIL to PAS:.

SOFTWARE INSTALLATION                                   PAGE 1-7

INSTALLATION IN DETAIL

11.  Edit  the  command file TCP:PASWRD.COM to mount and as-

sign the logical disk PAS:.

12.  Update  the  password file.  The password file contains

an anonymous account which you will need to  update  or

delete.   If  you delete the anonymous account then add

at least one valid account  for  initial  testing  with

remote FTP enabled.

13.  Edit  the  file  TCP:TCPIP.CFG  for your configuration.

Your machine's name and IP address are required and the

specification  of  a name server and gateway may be re-

quired for your network.

14.  Edit  TCP:TCPIP.COM to mount TCP:  and assign units 0-3

of the device handler to EN0-3.  Copy this file to SY:.

15.  Edit  or  create  the  TSX  detached  job  startup file

SY:DETACH.TSX      to      include       the       line

'detach sy:tcpip.com'.

16.  Edit  the files TCP:*.SRV to mount and assign the logi-

cal disks for TCP:, PAS:, MAI:, HTP:, and GPH:  as  re-

quired  by  the particular service.  (Since the servers

are run as detached jobs these need to be defined  when

the job starts.)

17.  Update  your TSX configuration by following the section

on TSX-Plus system generation.

18.  Reboot your TSX-Plus system.  Verify that the job TCPIP

is running.  (If the job did not start then check  that

SY:TCPIP.COM  has  TCP:   defined correctly, the confi-

guration file path is correct, the  device  assignments

are correct, and file TCPIP.CFG is correct.  Try start-

ing the  TCPIP  server  from  the  console:   ru/single

TCP:TCPIP,  enter  the  command  line  arguments at the

argv:  prompt and see what errors the program gives.)

19.  Verify  that another networked computer can communicate

with your machine by doing a 'ping'.  Don't know what a

ping is?  Skip this step.  (verifies that TCPIP is able

to respond to data sent over the Ethernet wire)

20.  Attempt to Telnet or FTP to a local computer using only

the IP number of that computer.  (verifies  connections

are possible)

21.  Attempt  to Telnet or FTP to a local computer using the

computer's name.  The computer's name  and  IP  address

must  not be in the TCPIP.CFG file.  (verifies that the

name server entry is correct)

SOFTWARE INSTALLATION                                   PAGE 1-8

INSTALLATION IN DETAIL

22.  Attempt to Telnet or FTP to a machine not on your local

network.  If you use the machine's name then  the  name

server  must  be  working  or the machine's name and IP

number   must   be   in   the    configurataion    file

TCP:TCPIP.CFG.   (verifies  that  the  gateway entry is

correct)

23.  Attempt  to  FTP  to  your  computer  from  some  other

networked machine using your machine's IP address.   If

your  machine's  name  has  not  been added to the name

server you must use the IP number.  (verifies that  the

RFTP.SRV  configuration  is  correct  and that password

checking is working)

Essentially your installation is now complete.  You only need

to update the password file to  specify  user/password  informa-

tion,  access rights, and default FTP and MAIL directories.  In-

dividual program configuration information is contained  in  the

chapter titled Program Options.

CHAPTER 2

HARDWARE INSTALLATION

2.1  Q-BUS SYSTEMS

A  Q-Bus based TSX-PLUS system requires the installation of a

DEQNA or DELQA communications controller.  A system memory of at

least  512k  is  required  when installing the Internet Package.

The DEQNA/DELQA EQ.TSX handler requires 36k bytes for  code  and

packet buffers and the network server program TCPIP.SAV will use

64k bytes for a maximum of approximately 100k bytes  before  ac-

tivating  any client/server programs.  Each active client/server

program will use an additional 64k bytes plus one or two  global

PLAS regions each 8k bytes in size.

NOTE

It is possible to install the TCP/IP Package on a 248K

machine using the DEQNA/DELQA 18-bit EQ18.TSX handler.

This  handler  requires 7.5k bytes for code and packet

buffers.   This  configuration  allows  at  most   one

networking application to be active.

Blockmode  memory  is recommended for use with the DEQNA com-

munications controller although not  necessary.   The  DEQNA  or

DELQA interface is connected via a bulkhead cable assembly (with

perhaps an extension ETHERNET  transceiver  cable)  to  a  tran-

sceiver (with either a thick or thin wire connection).

HARDWARE INSTALLATION                                   PAGE 2-2

UNIBUS SYSTEMS

2.2  UNIBUS SYSTEMS

A Unibus based TSX-PLUS system requires the installation of a

DEUNA or DELUA communications controller.  A system memory of at

least  384k is recommended when installing the Internet Package.

The DEUNA/DELUA 22-bit (Unibus  Mapping)  EU22.TSX  handler  re-

quires  22k  bytes  for  code and packet buffers and the network

server program TCPIP.SAV will use 64k bytes  for  a  maximum  of

approximately 86k bytes before activating any client/server pro-

grams.  Each active client/server program will use an additional

64k  bytes  plus one or two global PLAS regions each 8k bytes in

size.

NOTE

It is possible to install the TCP/IP Package on a 248K

machine using the DEUNA/DELUA 18-bit (Non Unibus  Map-

ping)  EU18.TSX  handler.   This handler requires 7.5k

bytes for code and packet buffers.  This configuration

allows  at  most  one networking application to be ac-

tive.

The  DEUNA or DELUA interface is connected via a bulkhead ca-

ble assembly (with perhaps an extension ETHERNET transceiver ca-

ble)  to  a  transceiver  (with  either  a  thick  or  thin wire

connection).

2.3  SERIAL PORT INSTALLATION

If  the system is to be configured for remote Telnet sessions

then a pair of dedicated  serial  lines  is  required  for  each

simultaneously  active session.  Each dedicated pair consists of

a TSX-PLUS timeshare line cross connected to a  CL  line  via  a

NULL-MODEM cable.

For  a  DLV11-J  four port serial board the cross connections

may be easily obtained by constructing a 4" cable  with  the  10

wire  flat cable flipped between the two ends (1->10, 2->9, ....

10->1).  Then cut wires 1,2,9,and 10 at both connectors  leaving

a   6-wire  NULL-MODEM  cable.   Two  such  cables  allow  PORT1

(TSX-timeshare line) to connect  to  PORT2(CL  line)  and  PORT3

(TSX-timeshare  line)  to connect to PORT4(CL line) allowing two

simultaneous remote Telnet  sessions.   The  DLV11-J  baud  rate

should  be  limited to 4800 as two serial ports are required for

each session and  is  a  relatively  heavy  burden  even  for  a

LSI-11/73.

HARDWARE INSTALLATION                                   PAGE 2-3

SERIAL PORT INSTALLATION

Cross  connected  DZ ports will provide faster and more effi-

cient communication.

CHAPTER 3

TSX-PLUS SYSTEM GENERATION

The TSX-Plus system must be regenerated to provide the neces-

sary device support, system services, and  options  required  by

the  TCP/IP  Package.  What follows is a description of each op-

tion that should be examined and updated  to  allow  the  proper

operation of the various hardware and software components of the

TCP/IP Package.

3.1  BUSTYPE

NOTE

The DEQNA/DELQA device used by the TCP/IP Package is a

QBUS device.   The  DEUNA/DELUA  device  used  by  the

TCP/IP Package is a Unibus device.

BUSTYP defines the machine bus structure for TSX-Plus.  Their

are two possible machine bus structures supported by TSX-Plus  -

the  QBUS  (LSI) and the UNIBUS.  Select one of these parameters

below to specify the bus support desired.  Use the following in-

formation for choosing the correct bus structure.

QBUS - 11/23, 11/23-Plus, 11/73, and Professional.

UNIBUS - 11/24, 11/34a, 11/44, and 11/60.

BUSTYP = QBUS   ;Specify machine bus structure (UNIBUS/QBUS)

TSX-PLUS SYSTEM GENERATION                              PAGE 3-2

BUSTYPE

3.1.1  TSGEN Unibus Mapping Modification

A  change to the TSX-Plus TSGEN.MAC file is required to allo-

cate 4 UNIBUS MAPPING REGISTERS for the 22-bit  EU22.TSX  driver

when used with systems having more than 248.  kbytes of address-

able memory.

The  mapping  registers are only required when more than 248.

Kbytes of memory are used by TSX-Plus on a Unibus system.

The following was excerpted from TSGEN.MAC:

******************************************************

;

;  Generate tables for Unibus map registers

;

.GLOBL  UMRBAS,UMREND,UMRWHD

UMRWHD: .WORD   0

.MACRO  UMRDEF  NUM

.BYTE   CURUMR          ;UM$UMR

.BYTE   NUM             ;UM$NMR

.WORD   NUM*4096.               ;UM$WDS

.WORD   0                       ;UM$IOQ

CURUMR  =       CURUMR+NUM

.ENDM   UMRDEF

;  Define UMR sets in order of size -- small to large.

CURUMR  =       5       ;Map regs 0-4 always mapped

UMRBAS:

.IF     EQ,<BUSTYP-UNIBUS>      ;Generate only

UMRDEF  1.                      ;for UNIBUS

UMRDEF  1.                      ;machines

UMRDEF  4.

UMRDEF  4.

UMRDEF  8.

UMRDEF  8.

.ENDC

UMREND:

******************************************************

Change  the  first  'UMRDEF 8.' to 'UMRDEF 4.' This will free

the last 4 UNIBUS  MAPPING  REGISTERS  for  use  by  the  22-bit

EU22.TSX driver.

TSX-PLUS SYSTEM GENERATION                              PAGE 3-3

BUSTYPE

3.2  NGR

NOTE

The  TCP/IP Package limits its use to 8.  PLAS regions

for passing data and parameters  between  the  network

server  and  the  various  client/server  tasks.   The

number of regions used by  each  active  client/server

program  is:   TELNET(1),  RTELNT(1), FTP(2), FTPL(2),

RFTP(2),  MAIL(1),   RSMTP(1),   RPOP3(1),   MMAIL(1),

MAILER(1),   CNCT(1),  RCNCT(1),  RHTTP(1),  RGOPH(1),

NTSND(1), LPQRM(1), and RLPD(1).

The  number of shared global PLAS regions that can be created

by all jobs.

NGR = 20.       ;Number of global PLAS regions

3.3  DEVXMR

NOTE

The DEQNA/DELQA/DEUNA/DELUA device handlers use 1 PLAS

region.

Some  device handlers allocate extended memory (PLAS) regions

for their use.  For example, the DU and MU handlers each require

one  PLAS region.  If you are using any other handlers which re-

quire extended memory regions, include the number of regions re-

quired.

DEVXMR = 4.     ;Number of XM regions for device handlers

TSX-PLUS SYSTEM GENERATION                              PAGE 3-4

IOABT

3.4  IOABT

NOTE

The  TCP/IP  Package  Network  Server  which  uses the

DEQNA/DELQA/DEUNA/DELUA handlers internally queue  the

network  requests.   If IOABT is not set to 1 then the

Network Server program cannot be aborted.   This  will

also  be  true  for the remote TELNET and CNCT servers

which must abort I/O through the CL device handler.

The  IOABT  parameter  controls  the action taken by TSX-Plus

when a job terminates execution.  If IOABT=0, TSX-Plus will wait

for  all  outstanding I/O pending for the job to complete before

the job is actually terminated.  If IOABT=1, TSX-Plus will  call

the  handler  abort  entry point for all outstanding I/O pending

for the job.  Note, the "SET IO NO ABORT" keyboard  command  may

be used to change the value of this parameter.

IOABT = 1       ;0==>I/O rundown, 1==>I/O abort

3.5  DEVDEF

NOTE

The  device handler, EQ.TSX or EU.TSX must be added to

the list of system handlers loaded by TSX-PLUS at sys-

tem startup.  The handler may be mapped, uses DMA, and

should not allow SET operations on the loaded handler.

DEVDEF  <EQ>,DMA,MAPH,NOSET

TSX-PLUS SYSTEM GENERATION                              PAGE 3-5

MESSAGE COMMUNICATION CHANNELS

3.6  MESSAGE COMMUNICATION CHANNELS

NOTE

Each  active client/server program of the TCP/IP Pack-

age uses 1 message channel.  Those programs using mes-

sage  channels are:  TCPIP, TELNET, RTELNT, FTP, FTPL,

RFTP, MAIL, RSMTP, RPOP3, MMAIL, MAILER, CNCT,  RCNCT,

RHTTP, RGOPH, NTSND, LPQRM, and RLPD.

Define  parameters  pertaining  to  the inter-program message

communication feature.  If this feature is not wanted,  set  all

four parameters to 0 (zero).

Maximum number of message communication channels which may be

simultaneously in use.

MAXMC   =       10.     ;Max message channels

Maximum message length (bytes).

MSCHRS  =       200.    ;Max message length (bytes)

Maximum  of messages which may be held in queue.

MAXMSG  =       20.     ;Max queued messages

Maximum  of requests for messages that may be held in queue.

MAXMRB  =       10.     ;Max  pending message requests

The message length used by TCPIP programs is only 16.  bytes.

Thus the MSCHRS term may be changed to '16.' if  no  other  user

programs use the message channels.

TSX-PLUS SYSTEM GENERATION                              PAGE 3-6

LINE DEFINITIONS

3.7  LINE DEFINITIONS

NOTE

If  the  remote  TELNET  or  CNCT  servers  are  to be

enabled, then a pair of cross connected  serial  lines

must  be  added  to the Line Definition table for each

active session.  The  following  Line Definitions  il-

lustrate  the  additions  required.   If more than one

pair of lines are added then  select  sequential  time

share  and  sequential  CL lines.  The SILO length de-

finition for the CL line should  be  greater  than  or

equal  to the maximum number of characters it will re-

ceive in .5 seconds (120 for 2400 baud or 240 for 4800

baud).   This  will reduce XON/XOFF sequences and max-

imize the serial transfer rate.

; The TBLDEF macro call requires four arguments:

;   1. The number of real time-sharing lines on machine.

;   2. The number of subprocess jobs.

;   3. The number of detached jobs.

;   4. The number of dedicated CL lines.

TBLDEF  3.,10.,6.,1.    ;# Real, # Subprocess,

;# Detached, # CL lines

; #3 time-sharing line is cross linked to CL0:

LINDEF  340,175600

CMDFIL  LINE3.TSX

TRMTYP  VT100

FLAGS   NRMFLG

LINEND

; CL0: is cross linked to #3 time-sharing line

CLDEF   0,350,175610

SILO    256.,12.,4.

CLEND

TSX-PLUS SYSTEM GENERATION                              PAGE 3-7

DETACH

3.8  DETACH

NOTE

The  command  file  DETACH.TSX  is used to startup the

Network Server.

Use  the  "DETACH" macro here to declare any start-up command

files and associated parameters (up to 80 characters) to be  run

as detached jobs:

DETACH  <SY:DETACH.TSX>         ;Start-up detached job

3.9  RECORD LOCKING

NOTE

If the MAIL, RSMTP, MMAIL, MAILER, RPOP3, or RLPD pro-

grams are to be used then record locking is  necessary

to properly update the mail/lprint sequence files.

Define  parameters  pertaining  to record (block) locking for

shared files.  If the shared file block locking facility is  not

wanted, set all of these parameters to 0 (zero).

Maximum  number  of  shared  files  which  may be open simul-

taneously.  Note that several  users  accessing  the  same  file

count as 1.

MAXSF   =       4.      ;Max number of shared files

Maximum  number  of  I/O  channels  which  all  users may simul-

taneously have open to shared files.  Note, this  is  the  total

number for all users not for each user.

MAXSFC  =       16.     ;Max  shared file channels

Maximum number of blocks which may be simultaneously held locked

by any channel.  That is, max blocks locked per channel.

MXLBLK  =       16.     ;Max blocks locked per channel

TSX-PLUS SYSTEM GENERATION                              PAGE 3-8

USER COMMAND LINKAGE

3.10  USER COMMAND LINKAGE

NOTE

The  file  TCP:TSXUCL.INS contains over 40 UCL defini-

tions, however a typical user may require only  a  few

of them.

U$CL is a flag that controls whether the User Command Linkage

is to be used to allow users to define their own  commands.   If

U$CL  is  non-zero the UCL facility is enabled and users may de-

fine their own system commands.  If U$CL is zero,  user  defined

commands will not be supported by the system.  Note:  if the UCL

facility is enabled, the TSXUCL.SAV file must be placed  on  the

system disk.

U$CL    =       1       ;0==>No UCL program

;1==>UCL program

Number of user-defined commands that can be stored by TSXUCL for

each job.  (The number of blocks required in  the  SY:TSXUCL.DAT

file  is  approximately  equal to the number of commands per job

times the total number of time-sharing lines divided by 5).

UCLMNC  =       12.     ;Maximum user-defined

;commands per job

CHAPTER 4

PROGRAM OPTIONS

The various client/server/utility program startup options are

outlined and typical startup command files listed.

4.1  TCPIP

The  network  server  program TCPIP.SAV provides the software

interface  between  the  Ethernet   device   handler   and   the

client/server  programs  in  the  TCP/IP  Package.   The network

server program handles the processing of various network packets

and  services  (TCP,  IP, UDP, BOOTP, ARP, RARP, and DOMAIN name

lookup).

The  TCPIP.SAV  program may be started interactively (usually

for diagnostic purposes) with the following options:

TCPIP [?] [-f filename]

?               List this Help Text and Exit TCPIP

f  filename     Configuration File

The  startup  command  file  TCPIP.COM deassigns all devices and

then assigns the network server device EN to be the  DEQNA/DELQA

device  EQ or the DEUNA/DELUA device EU.  Four EN units are used

by TCPIP, three are used to  receive  IP(EN0:),  ARP(EN1:),  and

RARP(EN2:)  packets and one is used to transmit(EN3:) all packet

types.

PROGRAM OPTIONS                                         PAGE 4-2

TCPIP

deassign

dismount ld0:

dismount ld1:

dismount ld2:

dismount ld3:

dismount ld4:

dismount ld5:

dismount ld6:

dismount ld7:

mount ld7: dl1:tsxtcp.dsk

assign ld7: TCP:

assign eq0: en0:

assign eq1: en1:

assign eq2: en2:

assign eq3: en3:

run/single TCP:tcpip

^1 ^2 ^3 ^4 ^5 ^6

The  TCPIP  program is usually started at TSX-Plus startup using

the file DETACH.TSX:

detach sy:tcpip.com

The   TCPIP.SAV   program  requires  the  configuration  file

TCP:TCPIP.CFG which defines all the parameters required to  con-

figure the network server.  Typical configuration parameters are

detailed in the following configuration file:

PROGRAM OPTIONS                                         PAGE 4-3

TCPIP

#

#  Example host file for TCP/IP server

#

#  This file is free form

#  Separators are any char <32, 'space' and '='

#  Comments are preceeded by #

#

#  The form is keyword=value for each parameter.

#  The first set of parameters refer to the whole program's defaults.

#  These parameter values can be in any order.

#  Following this are the individual machine specs.

#  If the first machine is name "default", then it contains default

#  values for the rest of the machines.

#

#       These options are specified for this host machine:

#

#       myname=____.____.____   # my internet name

#       myip=___.___.___.___    # host machines's IP number

#           =BOOTP              # to use BOOTP to get host info

#           =RARP               # to use RARP to get IP number

#       netmask=___.___.___.___ # subnetting mask

#       domain=____.____        # default domain for lookup

#       nndomto=____            # timeout for domain lookups

#       nnretry=____            # number of retries for lookup

#       nnarpto=____            # timeout for ARPs

#       nndto=____              # timeout for data layer

#       nnpkt=____              # TCPIP upper limit packets per transq

#                               # nnpkt=1, requires ACK for each packet

#       nnwin=____              # TCPIP upper limit window size

#       nnseg=____              # TCPIP upper limit input segment size

#       nnmtu=____              # TCPIP upper limit output transfer size

#       service=___,___,_____   # TCPIP services specification

#       task=___,___,_____      # TCPIP task specification

#       logsession=__________   # TCPIP session logging filespec

#

PROGRAM OPTIONS                                         PAGE 4-4

TCPIP

#

#       These are the options which may be specified for each

#       individual machine specification:

#

#       name=____               # name of session (required)

#       host=____               # full name of host computer

#       hostip=___.___.___.___  # IP number of host

#       port=____               # specify initial connection port

#       nameserver=____         # name server level

#       gateway=____            # gateway level

#       retrans=____            # initial retransmit timeout

#       contime=____            # opening connection timeout

#       mwin=____               # advertized window size for this host

#       mseg=____               # maximum advertized input segment size

#       mtu=____                # maximum output transfer size

#       delete=backspace        # value to use for character deletion

#       crmap=4.3BSDCRNUL       # for 4.3BSD null CR

#       duplex=half             # for half duplex TELNET connections

#       copyfrom=____           # copy parameters from this name

#

myname=machine.domain.name

myip=192.3.2.100        # required, this machine's IP number

netmask=255.255.255.0   # subnetting mask

domain="domain.name"    # domain name search path

# affects machines on your local network

nnwin=2920              # limit advertized window (nnwin == mwin)

nndomto=5               # domain timeout

nnretry=4               # number of retries

nnarpto=2               # arp timeout in seconds

nnpkt=2                 # send only 2 packet(s) per transq

PROGRAM OPTIONS                                         PAGE 4-5

TCPIP

#  The following line specifies the TCPIP logging file

#  which will contain information for each session

#  started:     TELNET / RTELNT / FTP / RFTP / CNCT / RCNCT /

#               MAIL / RSMTP

#               RPOP3 / MAILER / MMAIL

#               RLPD / LPRINT / LPQRM / LLPQRM

#               NTSND

#               RHTTP

#               RGOPH

# logsession=LOG:tcpip.log[100]

#  The following are services supported by TCPIP.

#  Each entry contains the listen port, maximum number of

#  sessions and the name of the command file to initiate

#  the service.

service=21,2,TCP:rftp.srv       # remote ftp server

service=23,2,TCP:rtelnt.srv     # remote telnet server

service=25,2,TCP:rsmtp.srv      # remote smtp server

service=27,2,TCP:rcnct.srv      # remote cnct server

service=70,2,TCP:rgoph.srv      # remote gopher server

service=80,4,TCP:rhttp.srv      # remote http server

service=110,2,TCP:rpop3.srv     # remote pop3 server (task: -t 1)

service=515,2,TCP:rlpd.srv      # remote lpd server  (task: -t 2)

#  The following specify the tasks that may be initiated by TCPIP.

#  Each entry contains the taskid number, timer interval in seconds

#  (zero means not an automatically started task), and the name

#  of the command file to initiate the service.  These tasks donot

#  have to be related to TCPIP programs, however a TCPIP program

#  can use the TASKCLASS to start a specific task. The selected

#  task will be scheduled by TCPIP and only one (1) copy of the

#  the task will be allowed to run.  Additional requests to start

#  the task will be queued as a single request to rerun the task

#  at the completion of the currently executing task.

task=1,0,TCP:mailer.srv # mmail / pop3      mail delivery agent

task=2,0,TCP:lprint.srv # rlpd / llpqrm     printing delivery agent

PROGRAM OPTIONS                                         PAGE 4-6

TCPIP

#

#  Following are individual machine specifications

#  Gateways are used in the order that they appear in the file

#  Nameservers rotate, #1, #2, #3, #1, #2 when a request fails

#

#  The machine named "default" contains the fields which are

#  automatically filled in for later hosts

#

name=default            # Session name, "default" is a reserved name

# Not a real machine, default parameters only

delete=delete           # Assume mostly non-UNIX connections

#  The following entries affect the tuning of

#       TCP connections to this host.

#  They should be set by the network administrator who is familiar

#       with the requirements of your specific network.

contime=12              # timeout in seconds to try connection

#   before returning error to user

retrans=5               # starting retransmit time out in seconds

mtu=1460                # maximum transmit unit in bytes

#   outgoing packet size, MAX=1460

mseg=1460               # largest segment we can receive

#   whatever the hardware can take, MAX=1460

mwin=2920               # most bytes we can receive without ACK

#   =TCP window size, MAX=3047

#   larger isn't always better

#

#  Below this line, most of the communication parameters are obtained

#   from the "default" host entry.

#  Machine names, IP addresses, and special communication parameters

#   are present when needed.

#

name=mygateway          hostip=192.3.2.2

gateway=1

name=nameserver         hostip=192.3.2.1

nameserver=1

name=alpha              host=alpha.domain.name

hostip=192.3.2.200

PROGRAM OPTIONS                                         PAGE 4-7

TCPIP

#

# If during TCPIP startup you get the error Sreadhosts() error 2,

# you probably have exceeded the memory limits of the machine.

#

#       (1)     reduce the number of machine definitions above

#       (2) or  donot use session logging

#

# Sesion logging and Debugging should not be simultaneously enabled.

#

The  network  server  needs to know the ip address(number) of

the machine to which it is connecting.  If  the  TCPIP.CFG  file

does not contain the named machines' ip number then it will make

a request to the specified name server to obtain the machines ip

number.   If  a name server is not specified then the connection

cannot be made using the machines name (place the machines  name

and ip address in the configuration file).  If the ip address is

given as the input to the client program rather  than  the  name

then a domain name lookup is not required.

All changes to the configuration file will take effect at the

next startup of TCPIP.SAV.

4.2  SKTMON

The  SKTMON  program provides a diagnostic tool for observing

the state of all TCPIP connections.  SKTMON displays the current

state  of  network  connections  by  showing  the global regions

through  which  the  TCPIP  network  server  program   and   the

client/server programs communicate.  The program shows in detail

the various network packets,  the  incoming  and  outgoing  data

streams, and the status of the network ethernet driver.

The  startup  command  file  contains  an assignment to allow

SKTMON to access the status of the device driver:

assign eq0: en0:

run/single TCP:sktmon 0 ^1

PROGRAM OPTIONS                                         PAGE 4-8

PASWRD

4.3  PASWRD

A  password  file  (eg  PASWRD.FIL)  must be built if the lo-

cal/remote MAIL services are to be used or if remote network ac-

cess  (ie remote FTP, TELNET, CNCT or printing services) must be

restricted.  It is suggested that a seperate password  directory

be  created  to  hold the files PASWRD.SAV and PASWRD.FIL.  This

directory need be no larger than about 200 blocks.  The  maximum

number of user/password specifications is limited by the program

memory size to about 50.  The PASWRD.FIL contains  1  block  for

each  user/password  specification.  The PASWRD.COM command file

should be updated to reflect the location of the password files.

A  seperate  directory  allows  the  exclusion  of its access by

remote FTP sessions, local users or remote telnet sessions  with

appropriate TSX-PLUS access rights may still access the password

files.

Copy  the  file PASWRD.SAV (and optionally the supplied pass-

word file PASWRD.FIL) to  the  created  directory.   Modify  the

PASWRD.COM file to access this subdirectory.

Initiating  PASWRD.SAV with a file specification will open an

existing file or create the file if no existing file  is  found.

The PASWRD program screens will indicate all the options and the

file USRBLK.H illustrates the  formats  for  the  directory/sub-

directory/file specifications required by the password program.

NOTE

The  password  for  any added user must also be speci-

fied,  any  user  not  requiring  a  password  (ie  an

'anonymous'  account)  must have this password entered

as a null string by typing only a carriage return  for

the password.

To  use the created password file be sure the program startup

command files have specified the correct path  to  the  password

file.

NOTE

The directory/subdirectory/file access rights are only

used by the remote FTP server.  Logins via  TELNET  or

CNCT  rely  upon  any  security  features  enabled  in

TSX-PLUS and not by the PASWRD.FIL specifications.

PROGRAM OPTIONS                                         PAGE 4-9

TELNET

4.4  TELNET

The  TELNET  program allows the user to initiate a connection

to a remote host for a terminal session.   The  startup  options

for TELNET are:

TELNET destination [?] [-hp]

?               List the Help Text and Exit TELNET

h               List the Help Text and Open Connection

p  filename     Specify the Password Filespec

The  destination host specification may be the host name or host

ip number.  If the connection is to be made to an alternate port

then follow the name/ip with the port number (decimal).

The  TELNET  program  supports the following command line op-

tions which are activated by the META character 'control A':

M->C    open capture file

M->D    close capture file

M->F    FTP [internet address]

M->H    this help screen

M->I    type my internet address

M->K    erase character

M->O    abort output

M->Q    are you there?

M->R    toggle <CR> follow character

M->S    skip to end of buffer

M->T    toggle backspace/delete

M->U    erase line

M->X    close connection

M->Y    interrupt process

M->@    send a true null character

^?      abort Telnet session

The startup command file TELNET.COM follows:

run/single TCP:telnet

-hbp PAS:/paswrd.dsk/paswrd.fil ^1 ^2 ^3 ^4 ^5 ^6

PROGRAM OPTIONS                                        PAGE 4-10

RTELNT

4.5  RTELNT

RTELNT  is  a  server program for remote TELNET sessions into

TSX-Plus.  The program startup options are:

RTELNT [?] [-hmp filespec] [-acelrtvyz] [parameters]

?       List the Help Text and Exit RTELNT

a       Attach to Service

c  i    CL line i is cross connected to TSX line j

e       Enable Monitoring of all Transactions

h       Specify the Help Filespec

l  n    Number of CL/TSX line pairs

m       Specify the Message Filespec

p       Specify the Password Filespec

r       Restart RTELNT after Disconnect

t  j    TSX line j is cross connected to CL line i

v       Verbose Mode

y  s    Login Timeout in seconds

z  s    Inactivity Timeout in seconds

The  program  may  be started interactively (for diagnostic pur-

poses) by the command file RTELNT.COM:

run/single TCP:rtelnt

-ctlpyz 0 3 1 PAS:/paswrd.dsk/paswrd.fil 15 600 ...

... ^1 ^2 ^3 ^4 ^5 ^6

The program is normally invoked by the TCPIP network server pro-

gram by the command file RTELNT.SRV:

deassign

dismount ld0:

dismount ld1:

dismount ld2:

dismount ld3:

dismount ld4:

dismount ld5:

dismount ld6:

dismount ld7:

assign sy: wf:

mount ld6: dl1:paswrd.dsk

assign ld6: PAS:

mount ld7: dl1:tsxtcp.dsk

assign ld7: TCP:

run/single TCP:rtelnt

-actlpyz 0 3 1 PAS:paswrd.fil 15 600

The  RTELNT help and message files are RTELNT.HLP and RTELNT.MSG

PROGRAM OPTIONS                                        PAGE 4-11

RTELNT

respectively.  The h or m options may be used if these files are

not  on the TCP:  device.  These files are created by the system

manager and are not required.

4.6  CNCT

The  CNCT program allows the user to initiate a connection to

a remote TSX-Plus host for a terminal session.  This program  is

the  TELNET program stripped of all TELNET negotiations and uses

a non standard TCP port (27).  The startup options for CNCT are:

CNCT destination [?] [-hp]

?               List the Help Text and Exit CNCT

h               List the Help Text and Open Connection

p   filename    Specify the Password Filespec

The  destination host specification may be the host name or host

ip number.  If the connection is to be made to an alternate port

then follow the name/ip with the port number (decimal).

The  CNCT program supports the following command line options

which are activated by the META character 'control A':

M->C    open capture file

M->D    close capture file

M->F    FTP [internet address]

M->H    this help screen

M->I    type my internet address

M->O    abort output

M->Q    are you there?

M->S    skip to end of buffer

M->X    close connection

M->Y    interrupt process

^?      abort CNCT session

The startup command file CNCT.COM follows:

run/single TCP:cnct

-hbp PAS:/paswrd.dsk/paswrd.fil ^1 ^2 ^3 ^4 ^5 ^6

PROGRAM OPTIONS                                        PAGE 4-12

RCNCT

4.7  RCNCT

RCNCT  is  a  server  program  for  remote CNCT sessions into

TSX-Plus.  This program is the RTELNT program  stripped  of  all

TELNET  negotiations and uses a non standard TCP port (27).  The

startup options for RCNCT are:

RCNCT [?] [-hmp filespec] [-acelrtvyz] [parameters]

?       List the Help Text and Exit RCNCT

a       Attach to Service

c  i    CL line i is cross connected to TSX line j

e       Enable Monitoring of all Transactions

h       Specify the Help Filespec

l  n    Number of CL/TSX line pairs

m       Specify the Message Filespec

p       Specify the Password Filespec

r       Restart RCNCT after Disconnect

t  j    TSX line j is cross connected to CL line i

v       Verbose Mode

y  s    Login Timeout in seconds

z  s    Inactivity Timeout in seconds

The  program  may  be started interactively (for diagnostic pur-

poses) by the command file RCNCT.COM:

run/single TCP:rcnct

-ctlpyz 0 3 1 PAS:/paswrd.dsk/paswrd.fil 15 600 ...

... ^1 ^2 ^3 ^4 ^5 ^6

The  program  is normally invoked by the TCPIP network server

program by the command file RCNCT.SRV:

deassign

dismount ld0:

dismount ld1:

dismount ld2:

dismount ld3:

dismount ld4:

dismount ld5:

dismount ld6:

dismount ld7:

assign sy: wf:

mount ld6: dl1:paswrd.dsk

assign ld6: PAS:

mount ld7: dl1:tsxtcp.dsk

assign ld7: TCP:

run/single TCP:rcnct

-actlpyz 0 3 1 PAS:paswrd.fil 15 600

PROGRAM OPTIONS                                        PAGE 4-13

RCNCT

The  RCNCT  help  and  message files are RCNCT.HLP and RCNCT.MSG

respectively.  The h or m options may be used if these files are

not  on the TCP:  device.  These files are created by the system

manager and are not required.

4.8  FTP

The  FTP  program  provides  reliable  data  transfer between

cooperating computer systems.  The FTP program  startup  options

are:

FTP [?] [-f filename] [-p filename]

[-ghinrv] [destination host]

?               List this Help Text and Exit FTP

f  filename     Command File

g               Wildcard Expansion Disabled

h               FTP Help List

i               Interactive Prompting Off

n               AutoLogin Disabled on Connect

p  filename     Specify the Password Filespec

r               Output Redirection Disabled

v               Verbose Mode Disabled

The  destination host specification may be the host name or host

ip number.  If the connection is to be made to an alternate port

then follow the name/ip with the port number (decimal).

If  the  work file device, wf:, is not assigned then the pro-

gram will fail to execute the commands mget, mdel, or mls.

The FTP startup command file is:

run/single TCP:ftp

-hbp PAS:/paswrd.dsk/paswrd.fil ^1 ^2 ^3 ^4 ^5 ^6

The FTP program supports the following command line options:

?               print local help information

ascii           set ascii transfer type

bell            toggle command completed beep

bget            get a file in binary mode

binary          set binary transfer type

bput            put a file in binary mode

bye             terminate ftp session and exit

cd              change remote working directory

close           terminate ftp session

debug           toggle/set debugging mode

delete          delete remote file

PROGRAM OPTIONS                                        PAGE 4-14

FTP

- inquires if prompting is on

dir             list contents of remote directory

get             receive file

hash            toggle printing `#' for each

1024 bytes transferred

help            print local help information

home            set default directory to home

interactive     turn on prompting for multiple commands

lcd             change local working directory

ldelete         delete local files

ldir            list contents of local directory

lls             list contents of local directory

lmkdir          make a local directory

lprotect        set protection flag on local file

lpwd            show local working directory

lrename         rename a local file

lrmdir          remove a local directory

ls              list contents of remote directory

ltype           print a local file to the screen

lunprotect      clear protection flag on a local file

mdelete         delete multiple files

mdir            list contents of multiple remote directories

mget            get multiple files

mkdir           make directory on the remote machine

mls             list contents of multiple remote directories

mode            set file transfer mode

mput            send multiple files

noninteractive  turn off prompting on multiple commands

open            connect to remote ftp

passive         enable or disable passive transfer mode

prompt          toggle interactive prompting

on multiple commands

protect         set protection flag on file

put             send one file

pwd             print working directory on remote machine

quit            terminate ftp session and exit

quote           send arbitrary ftp command

recv            receive file

remotehelp      get help from remote server

rename          rename remote file

rm              delete remote file

rmdir           remove directory on the remote machine

send            send one file

sendport        toggle use of PORT cmd for each data connection

status          show current status

struct          set file transfer structure

system          query remote system type

type            print a remote file to the screen

unprotect       clear protection flag on file

user            send new user information

verbose         toggle verbose mode

version         FTP-11 version displayed

PROGRAM OPTIONS                                        PAGE 4-15

FTP

wild            toggle wildcard expansion of local file names

>   <filespec>  redirect (m)dir or (m)ls response to a file

The commands bget, get, put, and send allow explicit renaming

of the files during the  transfer  by  specifying  the  optional

destination file name:

bget/get/put/send [source name] [destination name]

4.9  FTPL

The  FTPL  program  provides  reliable  data transfer between

cooperating computer systems.  FTPL is a stripped version of the

FTP  program  for  use  on  smaller  systems.   The FTPL program

startup options are:

FTPL [?] [-p filename] [-ghinrv] [destination host]

?               List this Help Text and Exit FTPL

g               Wildcard Expansion Disabled

h               FTPL Help List

i               Interactive Prompting Off

n               AutoLogin Disabled on Connect

v               Verbose Mode Disabled

The  destination host specification may be the host name or host

ip number.  If the connection is to be made to an alternate port

then follow the name/ip with the port number (decimal).

If  the  work file device, wf:, is not assigned then the pro-

gram will fail to execute the commands mget, mdel, or mls.

The FTPL startup command file is:

run/single TCP:ftpl

-hb -p PAS:/paswrd.dsk/paswrd.fil ^1 ^2 ^3 ^4 ^5 ^6

The FTPL program supports the following command line options:

?               print local help information

ascii           set ascii transfer type

bell            toggle command completed beep

bget            get a file in binary mode

binary          set binary transfer type

bput            put a file in binary mode

bye             terminate ftp session and exit

cd              change remote working directory

close           terminate ftp session

debug           toggle/set debugging mode

PROGRAM OPTIONS                                        PAGE 4-16

FTPL

delete          delete remote file

- inquires if prompting is on

dir             list contents of remote directory

get             receive file

hash            toggle printing `#' for each

1024 bytes transferred

help            print local help information

interactive     turn on prompting for multiple commands

ls              list contents of remote directory

mdelete         delete multiple files

mget            get multiple files

mkdir           make directory on the remote machine

mode            set file transfer mode

mput            send multiple files

noninteractive  turn off prompting on multiple commands

open            connect to remote ftp

passive         enable or disable passive transfer mode

prompt          toggle interactive prompting

on multiple commands

put             send one file

pwd             print working directory on remote machine

quit            terminate ftp session and exit

quote           send arbitrary ftp command

recv            receive file

remotehelp      get help from remote server

rename          rename remote file

rm              delete remote file

rmdir           remove directory on the remote machine

send            send one file

sendport        toggle use of PORT cmd for each data connection

status          show current status

struct          set file transfer structure

system          query remote system type

type            print a remote file to the screen

user            send new user information

verbose         toggle verbose mode

version         FTP-11 version displayed

wild            toggle wildcard expansion of local file names

The commands bget, get, put, and send allow explicit renaming

of the files during the  transfer  by  specifying  the  optional

destination file name:

bget/get/put/send [source name] [destination name]

PROGRAM OPTIONS                                        PAGE 4-17

RFTP

4.10  RFTP

RFTP  is  a  server  program  for  remote  FTP  sessions into

TSX-Plus.  The program startup options are:

RFTP [?] [-hmp filespec] [-aeruvyz]

?               List the Help Text and Exit RFTP

a               Attach to Service

e               Monitor all RFTP Transactions

h               Specify the Help Filespec

m               Specify the Message Filespec

p               Specify the Password Filespec

r               Restart RFTP after Disconnect

u               UNIX Style DIR Command

v               Verbose Mode

y  s            Login Timeout in seconds

z  s            Inactivity Timeout in seconds

The  program  may  be started interactively (for diagnostic pur-

poses) by the command file RFTP.COM:

run/single TCP:rftp

-rpyz PAS:/paswrd.dsk/paswrd.fil 15 600 ...

... ^1 ^2 ^3 ^4 ^5 ^6

The program is normally invoked by the TCPIP network server pro-

gram by the command file RFTP.SRV:

deassign

dismount ld0:

dismount ld1:

dismount ld2:

dismount ld3:

dismount ld4:

dismount ld5:

dismount ld6:

dismount ld7:

assign sy: wf:

mount ld6: dl1:paswrd.dsk

assign ld6: PAS:

mount ld7: dl1:tsxtcp.dsk

assign ld7: TCP:

run/single TCP:rftp

-aspyz 0 4 PAS:paswrd.fil 15 600

The  RFTP  help  and  message  files  are  RFTP.HLP and RFTP.MSG

respectively.  The h or m options may be used if these files are

not  on the TCP:  device.  These files are created by the system

PROGRAM OPTIONS                                        PAGE 4-18

RFTP

manager and are not required.  The command file RFTHLP.COM  uses

the  subdirectories  utility LDISKS to create a logical disk map

for inclusion in the RFTP.HLP file.

The  -u  option  specifies  that the 'LIST' command returns a

UNIX style directory listing rather than the default RT-11 style

directory  format.   This format is useful if the remote systems

are PC's using one of the window oriented FTP utilities such  as

PROCOMM  V3.0 (DataStorm Technologies).  These utilities use the

unix access-rights string to determine whether the  entries  are

files  or  directories.   The  general  form  for the UNIX style

directory is:

-rwxrwxrwx   1 system  user       1536 12 Jan 96 file.ext

drwxrwxrwx   1 system  user     512000 01 Mar 96 subdir.dsk

4.11  MAIL

The  MAIL  program provides local and internet mail services.

This program is recommended for systems  not  requiring  a  POP3

server.  The startup options for MAIL are:

MAIL [?] [-fp filespec] [-i IP Address] [-z s]

?               List the Help Text and Exit MAIL

f  filename     Command File

h               Help list

i  IP Address   Intermediate Host Name / IP Address

p  filename     Specify the Password Filespec

z  s            Connection Timeout in Seconds

The  Intermediate Host IP option provides a means of routing the

internet mail through another host machine.  The  TSX-Plus  MAIL

program  does not support retry operations if the mail cannot be

delivered, using an intermediate host  supporting  retries  will

provide this feature to the TSX-Plus MAIL system.  Permission to

use a particular host as a MAIL relay should  be  obtained  from

the  hosts'  system manager.  The option must be specified using

the complete name (an IP address may also be used).   A  typical

invocation follows:

-i mail.domain.name

The MAIL program is invoked from the command file MAIL.COM:

run/single TCP:mail

-hpz PAS:/paswrd.dsk/paswrd.fil 60 ^1 ^2 ^3 ^4 ^5 ^6

PROGRAM OPTIONS                                        PAGE 4-19

MAIL

The MAIL program supports the following command line options:

?               print local help information

bye             terminate the MAIL program

copy            copy a message to a file

debug           toggle/set debugging mode

delete          delete a message

dir             list of unread mail messages

exit            terminate the MAIL program

hash            toggle printing `#' during transfers

help            print local help information

home            go to home (initial) directory

index           an index of received mail

interactive     turn on prompting on multiple commands

last            read most recent mail message

lcd             change local working directory

ldelete         delete local files

ldir            list contents of local directory

listmail        list of unread mail messages

lls             list contents of local directory

lmkdir          make a local directory

lprotect        set protection flag on local file

lpwd            show local working directory

lrename         rename a local file

lrmdir          remove a local directory

ltype           print a local file to the screen

lunprotect      clear protection flag on a local file

mail            go to mail directory

newmail         count number of new mail messages

noninteractive  turn off prompting on multiple commands

prompt          toggle interactive prompting

on multiple commands

quit            terminate the MAIL program

read            read a mail message

scan            scan directory for all mail messages

send [filespec] initiate the sending of a message

status          show current status

user            enter a new user/password access key

verbose         toggle verbose mode

version         MAIL-11 version displayed

wild            toggle wildcard expansion of local file names

Mail  is sent to a local user if only a name is specified and to

a remote machine if the user and IP address are specified.

Local:          username

Internet:       username@machine.domain.name

PROGRAM OPTIONS                                        PAGE 4-20

MAIL

The MAIL programs MAIL.SAV, MMAIL.SAV, MAILER.SAV, RPOP3.SAV,

and RSMTP.SAV make use of the password file to  restrict  access

to  the  mail  system.  A user may have local and/or remote MAIL

access (or no access) depending on the password file parameters.

Incoming  mail  may be directed to a users' default directory

or to a specific  mail  directory  (specified  in  the  password

file).   A  typical configuration is to create a mail directory,

SY:MAIL.DSK, and have mail subdirectories for each  user.   Thus

the path SY:/MAIL/USER1/ specifies the users' unique mail direc-

tory and should be specified in the password configuration.  The

local  mail programs MAIL.SAV, MMAIL.SAV, and MAILER.SAV and the

remote mail programs RSMTP.SAV and RPOP3.SAV check the  password

file  to  verify  that the user has the privilege to send or re-

ceive mail via the network and places any received mail  in  the

specified mail directory.  These programs will use/create a mail

index file in the mail directory which is used to keep an  index

of all received mail.

4.12  MMAIL

The MMAIL program is a front end interface to the MAILER ser-

vice.  This program is recommended for systems requiring a  POP3

server.  The startup options for MMAIL are:

MMAIL [?] [-fp filespec] [-t n] [-h]

?               List the Help Text and Exit MMAIL

f  filename     Command File

h               Help list

p  filename     Specify the Password Filespec

t  n            MAILER Delivery Task Number

The  MAILER  delivery  task  number must be specified.  The task

number must be that specified in the TCPIP.CFG  for  the  MAILER

task.  The MAILER task is started by MMAIL when mail is sent.

The MMAIL program is invoked from the command file MMAIL.COM:

run/single TCP:mmail

-hpt PAS:/paswrd.dsk/paswrd.fil 1 ^1 ^2 ^3 ^4 ^5 ^6

The MMAIL program supports the following command line options:

?               print local help information

bye             terminate the MMAIL program

PROGRAM OPTIONS                                        PAGE 4-21

MMAIL

copy            copy a message to a file

debug           toggle/set debugging mode

delete          delete a message

dir             list of unread mail messages

exit            terminate the SMAIL program

help            print local help information

home            go to home (initial) directory

index           an index of received mail

interactive     turn on prompting for multiple commands

last            read most recent mail message

lcd             change local working directory

ldelete         delete local files

ldir            list contents of local directory

listmail        list of unread mail messages

lls             list contents of local directory

lmkdir          make a local directory

lprotect        set protection flag on local file

lpwd            show local working directory

lrename         rename a local file

lrmdir          remove a local directory

ltype           print a local file to the screen

lunprotect      clear protection flag on a local file

mail            go to mail directory

newmail         count number of new mail messages

noninteractive  turn off prompting for multiple commands

prompt          toggle interactive prompting

on multiple commands

quit            terminate the MAIL program

read            read a mail message

scan            scan directory for all mail messages

send [filespec] initiate the sending of a message

status          show current status

user            enter a new user/password access key

version         SMAIL-11 version displayed

wild            toggle wildcard expansion of local file names

Mail  is sent to a local user if only a name is specified and to

a remote machine if the user and IP address are specified.

Local:          username

Internet:       username@machine.domain.name

The MAIL programs MAIL.SAV, MMAIL.SAV, MAILER.SAV, RPOP3.SAV,

and RSMTP.SAV make use of the password file to  restrict  access

to  the  mail  system.  A user may have local and/or remote MAIL

access (or no access) depending on the password file parameters.

PROGRAM OPTIONS                                        PAGE 4-22

MMAIL

Incoming  mail  may be directed to a users' default directory

or to a specific  mail  directory  (specified  in  the  password

file).   A  typical configuration is to create a mail directory,

SY:MAIL.DSK, and have mail subdirectories for each  user.   Thus

the path SY:/MAIL/USER1/ specifies the users' unique mail direc-

tory and should be specified in the password configuration.  The

local  mail programs MAIL.SAV, MMAIL.SAV, and MAILER.SAV and the

remote mail programs RSMTP.SAV and RPOP3.SAV check the  password

file  to  verify  that the user has the privilege to send or re-

ceive mail via the network and places any received mail  in  the

specified mail directory.  These programs will use/create a mail

index file in the mail directory which is used to keep an  index

of all received mail.

4.13  RPOP3

RPOP3  is  a server program for remote access to the TSX-Plus

mail system via the extended POP3 protocol.   This  server  sup-

ports  the  PC  shareware  packages PC Eudora and WinQVT/Net for

Windows 3.x and NT in particular and others using the POP3  pro-

tocol.  The program startup options are:

RPOP3 [?] [-p filespec] [-t n] [-z s] [-y s] [-aerv]

?               List the Help Text and Exit RPOP3

a               Attach to Service

e               Enable Monitoring of

RPOP3 Transactions

p  filespec     Specify the Password Filespec

r               Restart RPOP3 after Disconnect

t  n            Task Number for Mail Delivery Agent

v               Verbose Mode

y  s            Login Timeout in seconds

z  s            Inactivity Timeout in seconds

The  MAILER  delivery  task  number must be specified.  The task

number must be that specified in the TCPIP.CFG  for  the  MAILER

task.  The MAILER task is started by RPOP3 when mail is sent.

The program may be started interactively (for diagnostic pur-

poses) by the command file RPOP3.COM:

run/single TCP:rpop3

-rptyz PAS:/paswrd.dsk/paswrd.fil 1 15 600 ...

... ^1 ^2 ^3 ^4 ^5 ^6

The program is normally invoked by the TCPIP network server pro-

gram by the command file RPOP3.SRV:

PROGRAM OPTIONS                                        PAGE 4-23

RPOP3

deassign

dismount ld0:

dismount ld1:

dismount ld2:

dismount ld3:

dismount ld4:

dismount ld5:

dismount ld6:

dismount ld7:

assign sy: wf:

mount ld6: dl1:paswrd.dsk

assign ld6: PAS:

mount ld7: dl1:tsxtcp.dsk

assign ld7: TCP:

run/single TCP:rpop3

-aptyz PAS:paswrd.fil 1 15 600

The MAIL programs MAIL.SAV, MMAIL.SAV, MAILER.SAV, RPOP3.SAV,

and RSMTP.SAV make use of the password file to  restrict  access

to  the  mail  system.  A user may have local and/or remote MAIL

access (or no access) depending on the password file parameters.

Incoming  mail  may be directed to a users' default directory

or to a specific  mail  directory  (specified  in  the  password

file).   A  typical configuration is to create a mail directory,

SY:MAIL.DSK, and have mail subdirectories for each  user.   Thus

the path SY:/MAIL/USER1/ specifies the users' unique mail direc-

tory and should be specified in the password configuration.  The

local  mail programs MAIL.SAV, MMAIL.SAV, and MAILER.SAV and the

remote mail programs RSMTP.SAV and RPOP3.SAV check the  password

file  to  verify  that the user has the privilege to send or re-

ceive mail via the network and places any received mail  in  the

specified mail directory.  These programs will use/create a mail

index file in the mail directory which is used to keep an  index

of all received mail.

4.14  MAILER

MAILER  is  a  backend  program  for  MMAIL  and  RPOP3 which

provides mail delivery to local users and  remote  destinations.

When  MAILER is activated it scans all user mail directories for

files with names similiar to  ml____.nnn  created  by  MMAIL  or

RPOP3.   These files are routed to their respective destinations

and then deleted.  If the mail is not deliverable then the  mes-

sage  is  directed  back to the sender.  The program startup op-

tions are:

PROGRAM OPTIONS                                        PAGE 4-24

MAILER

MAILER [?] [-p filespec] [-i IP-address] [-t n] [-z s] [-ev]

?               List the Help Text and Exit MAILER

e               Enable Monitoring of MAILER Transactions

i  IP-address   Intermediate Host Name / IP Address

p  filespec     Specify the Password Filespec

t  n (minutes)  Continuous scanning time interval

v               Verbose Mode

z  s            Conection Timeout in Seconds

The  Intermediate Host IP option provides a means of routing the

internet mail through another host machine.  The  TSX-Plus  MAIL

program  does not support retry operations if the mail cannot be

delivered, using an intermediate host  supporting  retries  will

provide this feature to the TSX-Plus MAIL system.  Permission to

use a particular host as a MAIL relay should  be  obtained  from

the  hosts'  system manager.  The option must be specified using

the complete name (an IP address may also be used).   A  typical

invocation follows:

-i mail.domain.name

The  program  may  be started interactively (for diagnostic pur-

poses) by the command file MAILER.COM:

run/single TCP:mailer

-p PAS:/paswrd.dsk/paswrd.fil ^1 ^2 ^3 ^4 ^5 ^6

The program is normally invoked by the TCPIP network server pro-

gram by the command file MAILER.SRV:

deassign

dismount ld0:

dismount ld1:

dismount ld2:

dismount ld3:

dismount ld4:

dismount ld5:

dismount ld6:

dismount ld7:

assign sy: wf:

mount ld6: dl1:paswrd.dsk

assign ld6: PAS:

mount ld7: dl1:tsxtcp.dsk

assign ld7: TCP:

run/single TCP:mailer

-piz PAS:paswrd.fil mail.domain.name 60

The  specification  of  device  wf:   is  required  by MAILER to

PROGRAM OPTIONS                                        PAGE 4-25

MAILER

temporarily store local mail messages before writing the mail to

the users mail directories.  If wf:  is undefined, inaccessable,

or not initialized then mail will not be delivered.

The MAIL programs MAIL.SAV, MMAIL.SAV, MAILER.SAV, RPOP3.SAV,

and RSMTP.SAV make use of the password file to  restrict  access

to  the  mail  system.  A user may have local and/or remote MAIL

access (or no access) depending on the password file parameters.

Incoming  mail  may be directed to a users' default directory

or to a specific  mail  directory  (specified  in  the  password

file).   A  typical configuration is to create a mail directory,

SY:MAIL.DSK, and have mail subdirectories for each  user.   Thus

the path SY:/MAIL/USER1/ specifies the users' unique mail direc-

tory and should be specified in the password configuration.  The

local  mail programs MAIL.SAV, MMAIL.SAV, and MAILER.SAV and the

remote mail programs RSMTP.SAV and RPOP3.SAV check the  password

file  to  verify  that the user has the privilege to send or re-

ceive mail via the network and places any received mail  in  the

specified mail directory.  These programs will use/create a mail

index file in the mail directory which is used to keep an  index

of all received mail.

4.15  RSMTP

RSMTP is a server program for remote MAIL into TSX-Plus.  The

program startup options are:

RSMTP [?] [-hp filespec] [-z s] [-aerv]

?               List the Help Text and Exit RSMTP

a               Attach to Service

e               Monitor all RSMTP Transactions

h               Specify the Help Filespec

p               Specify the Password Filespec

r               Restart RSMTP after Disconnect

t  n            Mail Delivery Task Number

v               Verbose Mode

z  s            Inactivity Timeout in Seconds

The RSMTP help file RSMTP.HLP is of diagnostic use only.

The program may be started interactively (for diagnostic pur-

poses) by the command file RSMTP.COM:

run/single TCP:rsmtp

-rptz PAS:/paswrd.dsk/paswrd.fil 1 600 ...

... ^1 ^2 ^3 ^4 ^5 ^6

PROGRAM OPTIONS                                        PAGE 4-26

RSMTP

The program is normally invoked by the TCPIP network server pro-

gram by the command file RSMTP.SRV:

deassign

dismount ld0:

dismount ld1:

dismount ld2:

dismount ld3:

dismount ld4:

dismount ld5:

dismount ld6:

dismount ld7:

assign sy: wf:

mount ld6: dl1:paswrd.dsk

assign ld6: PAS:

mount ld7: dl1:tsxtcp.dsk

assign ld7: TCP:

run/single TCP:rsmtp

-aptz PAS:paswrd.fil 1 600

The  specification  of  device wf:  is required by RSMTP to tem-

porarily store incoming mail messages before writing the mail to

the users mail directories.  If wf:  is undefined, inaccessable,

or not initialized then incoming mail will not be delivered.

The MAIL programs MAIL.SAV, MMAIL.SAV, MAILER.SAV, RPOP3.SAV,

and RSMTP.SAV make use of the password file to  restrict  access

to  the  mail  system.  A user may have local and/or remote MAIL

access (or no access) depending on the password file parameters.

Incoming  mail  may be directed to a users' default directory

or to a specific  mail  directory  (specified  in  the  password

file).   A  typical configuration is to create a mail directory,

SY:MAIL.DSK, and have mail subdirectories for each  user.   Thus

the path SY:/MAIL/USER1/ specifies the users' unique mail direc-

tory and should be specified in the password configuration.  The

local  mail programs MAIL.SAV, MMAIL.SAV, and MAILER.SAV and the

remote mail programs RSMTP.SAV and RPOP3.SAV check the  password

file  to  verify  that the user has the privilege to send or re-

ceive mail via the network and places any received mail  in  the

specified mail directory.  These programs will use/create a mail

index file in the mail directory which is used to keep an  index

of all received mail.

PROGRAM OPTIONS                                        PAGE 4-27

LLPQRM

4.16  LLPQRM

LLPQRM is the program which submits, controls and reports the

status of the local LPRINT daemon.   The  program  performs  the

functions  of  LPC, LPQ, LPR, and LPRM.  The program startup op-

tions are:

LLPQRM [?] [-p filespec] [-t n] -o opt [opt arguments]

?               List the Help Text and Exit LLPQRM

p  filespec     LPD configuration file

t  n            TCPIP Task Number for LPD Delivery Agent

o  lpc  [?]     Start printer queue

lpq  [?]     Get printer queue status

lpr  [?]     Submit a job to a printer queue

lprm [?]     Remove a job from a printer queue

The -o opt argument must preceed the opt specific op-

tions.

The  LPRINT  delivery  task  number must be specified.  The task

number must be that specified in the TCPIP.CFG  for  the  LPRINT

task.   The LPRINT task is started by LLPQRM when a print job is

queued by LPR option or the print queue is started  by  the  LPC

option.

Program sub options are:

LPC [?]

?               List the LPC Help Text and Exit LLPQRM

LPQ [?] [-#P argument]

?               List the LPQ Help Text and Exit LLPQRM

#  n            Repeat query time in seconds

P  Queue Name   Printer Queue

LPR [?] [-#IPW argument] [-flr] [file to print]

?               List the LPR Help Text and Exit LLPQRM

#  n            # of copies to print (1-5)

I  n            Indentation

P  Queue Name   Printer Queue (default = lp)

W  n            Page Width

f               Plain Text Printing (default)

l               No Control Character Processing

r               Fortran Carriage Control Printing

LPRM [?] [-P argument] [file to remove from queue]

?               List the LPRM Help Text and Exit LLPQRM

P  Queue Name   Printer Queue

PROGRAM OPTIONS                                        PAGE 4-28

LLPQRM

Each of the options is invoked by a command file more indicative

of its intended function.  (The "" around the  TSX-Plus  parame-

ters  ^1 - ^6  are required to insure that the case of the argu-

ments are not changed.)

4.16.1  LLPC

LLPC.COM is the local start printer daemon command file.

run/single TCP:llpqrm

-pto TCP:lpdqrm.cfg 2 lpc "^1" "^2" "^3" "^4" "^5" "^6"

4.16.2  LLPQ

LLPQ.COM  is  the  local  check  printer queue status command

file.

run/single TCP:llpqrm

-pto TCP:lpdqrm.cfg 2 lpq "^1" "^2" "^3" "^4" "^5" "^6"

4.16.3  LLPR

LLPR.COM is the local print command file.

run/single TCP:llpqrm

-pto TCP:lpdqrm.cfg 2 lpr "^1" "^2" "^3" "^4" "^5" "^6"

4.16.4  LLPRM

LLPRM.COM is the local remove queued print file command file.

run/single TCP:llpqrm

-pto TCP:lpdqrm.cfg 2 lprm "^1" "^2" "^3" "^4" "^5" "^6"

For a normal user:

(1) if the printer queue is not specified then all of the queues are

checked.

(2) If  a  file name is not specified then all queued files for this

user are checked and the  user  queried  for  each  file  to  be

deleted.

For the special user 'system':

(1) if the printer queue is not specified then all of the queues are

PROGRAM OPTIONS                                        PAGE 4-29

LLPQRM

checked.

(2) If  a  file  name is not specified then all queued files for all

users are checked and the 'system' queried for each file  to  be

deleted.

(3) If  a  username  is  entered,  then  all files for this user are

checked and 'system' queried for each file deletion.

(4) If  a filename is entered, then the specific file is checked and

'system' queried for file deletion.

4.17  RLPD

RLPD  is  a  server  program providing remote printing on the

TSX-Plus system printer.  The program startup options are:

RLPD [?] [-p filespec] [-t n] [-aer]

?               List the Help Text and Exit RLPD

a               Attach to Service

e               Enable Monitoring of RLPD Transactions

p  filespec     Specify the Configuration Filespec

r               Restart RLPD after Disconnect

t  n            TCPIP Task Number for

RLPD Delivery Agent

The  LPRINT  delivery  task  number must be specified.  The task

number must be that specified in the TCPIP.CFG  for  the  LPRINT

task.   The  LPRINT  task is started by RLPD when a print job is

received.

The program may be started interactively (for diagnostic pur-

poses) by the command file RLPD.COM:

run/single TCP:rlpd

-rpt TCP:lpdqrm.cfg 2 ^1 ^2 ^3 ^4 ^5 ^6

The program is normally invoked by the TCPIP network server pro-

gram by the command file RLPD.SRV:

PROGRAM OPTIONS                                        PAGE 4-30

RLPD

deassign

dismount ld0:

dismount ld1:

dismount ld2:

dismount ld3:

dismount ld4:

dismount ld5:

dismount ld6:

dismount ld7:

assign sy: wf:

mount ld6: dl1:paswrd.dsk

assign ld6: PAS:

mount ld7: dl1:tsxtcp.dsk

assign ld7: TCP:

run/single TCP:rlpd

-apt TCP:lpdqrm.cfg 2

4.18  LPRINT

The  program  LPRINT is the backend printing daemon servicing

printing on the TSX-Plus system.  Print jobs submitted by remote

systems  through  the RLPD server and local print jobs submitted

through LLPQRM are processed  by  the  LPRINT  task.   RLPD  and

LLPQRM  write  two  files (cfa.nnn and dfa.nnn) into a temporary

file directory, cfa.nnn is the print control file and dfa.nnn is

the  print  data  file.   At  the completion of the printing job

these files are deleted.

The command options for LPRINT are:

LPRINT [?] [-p filespec] [-t n] [-v]

?               List the Help Text and Exit LPRINT

p  filespec     Specify the Configuration Filespec

t  n (minutes)  Continuous Scanning Time Interval

v               Verbose Mode

The LLPQRM.CFG configuration file is of the following form:

001                                     ; three digit sequence

sy2:\lpdtmp\                            ; temporary directory

sy0:\paswrd\paswrd.fil                  ; pass word file

lp      *       name    ls:             ; line printer

dspcgc  *       name    hf:             ; cgc graphics device

dspqbs  *       name    hg:             ; qbs graphics device

file    *       name    sy3:\tmpwrk\    ; write to a file

lp      pdp     *       ls:             ; line printer

dspcgc  pdp     *       hf:             ; cgc graphics device

dspqbs  pdp     *       hg:             ; qbs graphics device

PROGRAM OPTIONS                                        PAGE 4-31

LPRINT

The  first line is a sequence number maintained by RLPD, LLPQRM,

and LPRINT.  The second line is the directory used  for  holding

the  queued  files  until  processed  by LPRINT.  The third line

references the password file which may be used to check  if  the

user exists.

The  fourth  and  succeeding  lines  contain printing queues and

their access permissions.  The four entries are:

(1)     queue name

(2)     machine name (* is any)

(3)     user name (* is any)

(4)     device

The  data  is  copied to the specified printing device or random

access  device  using  the  name  in  the  print  control   file

(cfa.nnn).

The program may be started interactively (for diagnostic pur-

poses) by the command file LPRINT.COM:

run/single TCP:lprint

-p TCP:lpdqrm.cfg ^1 ^2 ^3 ^4 ^5 ^6

The program is normally invoked by the TCPIP network server pro-

gram by the command file LPRINT.SRV:

deassign

dismount ld0:

dismount ld1:

dismount ld2:

dismount ld3:

dismount ld4:

dismount ld5:

dismount ld6:

dismount ld7:

assign sy: wf:

mount ld6: dl1:paswrd.dsk

assign ld6: PAS:

mount ld7: dl1:tsxtcp.dsk

assign ld7: TCP:

run/single TCP:lprint

-p TCP:lpdqrm.cfg

The  specification  of device wf:  is required by LPRINT to tem-

porarily store data files before writing the data  to  subdirec-

tories.   If wf:  is undefined, inaccessable, or not initialized

then data will not be copied.

PROGRAM OPTIONS                                        PAGE 4-32

LPQRM

4.19  LPQRM

LPQRM  is the program which submits, controls and reports the

status of remote printer queues.  The program performs the func-

tions  of  LPC, LPQ, LPR, and LPRM.  The program startup options

are:

LPQRM [?] [-ipq argument] [-e] -o opt [opt arguments]

?               List the Help Text and Exit LPQRM

e               Enable Monitoring of LPQRM Transactions

i  IP Address   Remote Server Name / IP Address

n  Port Number  Remote Server Port Number

q  Queue Name   Remote Server Printer Queue

p  filespec     Configuration File

o  lpc  [?]     Start printer queue

lpq  [?]     Get printer queue status

lpr  [?]     Submit a job to a printer queue

lprm [?]     Remove a job from a printer queue

The -o opt argument must preceed the opt specific op-

tions.

Program sub options are:

PROGRAM OPTIONS                                        PAGE 4-33

LPQRM

LPC [?] [-SP argument]

?               List the LPC Help Text and Exit LPQRM

P  Queue Name   Remote Host Printer Queue

S  IP Address   Remote Host Name / IP Address

LPQ [?] [-SP# argument] [-l]

?               List the LPQ Help Text and Exit LPQRM

#  n            Repeat query time in seconds

l               Long form of queue status

P  Queue Name   Remote Host Printer Queue

S  IP Address   Remote Host Name / IP Address

LPR [?] [-SPIW# argument] [-flr] [filespec]

?               List the LPR Help Text and Exit LPQRM

#  n            # of copies to print (1-5)

P  Queue Name   Remote Host Printer Queue

S  IP Address   Remote Host Name / IP Address

f               ASCII Plain Text (default)

l               ASCII With Controls

r               Fortran Carriage Control

I  n            Indentation (f option only)

W  n            Page Width (flr options only)

Other Options:

1234cdghnoptvz  UNIX(tm) Options

M               Mail Option

C       Class           J       Job

L       Banner          T       Title

LPRM [?] [-SP argument] job [job [...]]

?               List the LPRM Help Text and Exit LPQRM

P  Queue Name   Remote Host Printer Queue

S  IP Address   Remote Host Name / IP Address

Each of the options is invoked by a command file more indicative

of its intended function.  (The "" around the  TSX-Plus  parame-

ters  ^1 - ^6  are required to insure that the case of the argu-

ments are not changed.)

PROGRAM OPTIONS                                        PAGE 4-34

LPQRM

4.19.1  LPC

LPC.COM is the remote start printer daemon command file.

run/single TCP:lpqrm

-po TCP:lpdqrm.cfg lpc "^1" "^2" "^3" "^4" "^5" "^6"

4.19.2  LPQ

LPQ.COM  is  the  remote  check  printer queue status command

file.

run/single TCP:lpqrm

-po TCP:lpdqrm.cfg lpq "^1" "^2" "^3" "^4" "^5" "^6"

4.19.3  LPR

LPR.COM is the remote print command file.

run/single TCP:lpqrm

-po TCP:lpdqrm.cfg lpr "^1" "^2" "^3" "^4" "^5" "^6"

4.19.4  LPRM

LPRM.COM is the remote remove queued print file command file.

run/single TCP:lpqrm

-po TCP:lpdqrm.cfg lprm "^1" "^2" "^3" "^4" "^5" "^6"

4.20  NTSND

NTSND  is a program for transferring data to a remote printer

without any translation (good for binary files).   This  program

supports  products  such  as the Emulex NETJET for the HP 4Si/Mx

printer.  The command options are:

NTSND [?] [-inpq argument] file [file [...]]

?               List the Help Text and Exit NTSND

i  IP Address   Remote Server Name / IP Address

n  Port Number  Remote Server Port Number

p  filespec     Password File

q               Query for File Name

PROGRAM OPTIONS                                        PAGE 4-35

NTSND

The NTSND program is invoked from the command file NTSND.COM:

run/single TCP:ntsnd

-bp PAS:/paswrd/paswrd.fil ^1 ^2 ^3 ^4 ^5 ^6

4.21  RHTTP

RHTTP  is  an  HTTP/0.9 server supporting only simple request

and response formats for GET, HEAD, and POST methods.  The  POST

method  always  responds  as  unimplemented.  HTTP transfers are

always in BINARY format except for files of type HTM, DOC,  TXT,

and LST which are in ASCII format.  The command options are:

RHTTP [?] [-p filespec] [-z s] [-aerv]

?               List the Help Text and Exit RHTTP

a               Attach to Service

e               Enable Monitoring of all

RHTTP Transactions

p               Specify the HTTP Directory and Home File

r               Restart RHTTP after Disconnect

v               Verbose Mode

z  s            Inactivity Timeout in seconds

The HTTP directory and home file must be specified, using the

-p option.  The directory path is used as the base directory for

all HTTP accesses.

The program may be started interactively (for diagnostic pur-

poses) by the command file RHTTP.COM:

mount ld0: dl1:/http.dsk

assign ld0: HTP:

run/single TCP:rhttp

-rspz 1 2 HTP:/home.htm 30 ^1 ^2 ^3 ^4 ^5 ^6

The program is normally invoked by the TCPIP network server pro-

gram by the command file RHTTP.SRV:

PROGRAM OPTIONS                                        PAGE 4-36

RHTTP

deassign

dismount ld0:

dismount ld1:

dismount ld2:

dismount ld3:

dismount ld4:

dismount ld5:

dismount ld6:

dismount ld7:

assign sy: wf:

mount ld0: dl1:http.dsk

assign ld0: HTP:

mount ld7: dl1:tsxtcp.dsk

assign ld7: TCP:

run/single TCP:rhttp

-aspz 1 2 HTP:/file.ext 30

NOTE

The  slashs,  /.../,  are required to delimit the HTTP

base directory.   If  a  device  or  (logical)  device

specification  is  used,  HTP:,  then the slash is re-

quired after the colon.

All accesses to files outside the http directory are not per-

mitted and are reported as  unauthorized.   Absolute  references

are  not allowed (e.g.  dl1:/http.dsk/file.ext) even if they are

in the http directory, these are also reported as  unauthorized.

Use  only  relative  paths  in  file  specifications to the http

directory and subdirectories  (e.g.   /subdir/file.ext).   These

requirements  provide  the most security for http requests which

require no password authentication.

The  home  file  is an HTML file and should be written to re-

flect the functionality of the browsers you wish to support with

this http server.

PROGRAM OPTIONS                                        PAGE 4-37

RGOPH

4.22  RGOPH

RGOPH is a simple Gopher server supporting transfers of text,

binary, and image files and linking to other  network  resources

using basic ASCII text constructs.  The command options are:

RGOPH [?] [-p filespec] [-z s] [-aerv]

?               List the Help Text and Exit RGOPH

a               Attach to Service

e               Enable Monitoring of all

RGOPH Transactions

p               Specify the Master GOPHER File

r               Restart RGOPH after Disconnect

v               Verbose Mode

z  s            Inactivity Timeout in seconds

The  Gopher  master file must be specified, using the -p option.

The master file contains the access specifications for  the  go-

pher  server.  The format of the master file is described in the

file RGOPH.CFG and included here for reference:

PROGRAM OPTIONS                                        PAGE 4-38

RGOPH

#

# This is the Master Gopher List

#

# Each entry has the following format:

#<code><TAB>description

#      <TAB>file or list specification

#      <TAB>machine name or IP address

#      <TAB>port number

#

# End list with '.' and <CRLF>

#

# Common item types specified by <code> are:

#

#       0       item is a file

#       1       item is a directory

#       2       item is a CSO phone-book server

#       3       Error

#       4       item is a NINHEXed MacIntosh file

#       5       item is DOS archive of some sort

#       6       item is a UNIX uuencoded file

#       7       item is an Index-Search server

#       8       item points to a text-based telnet server

#       9       item is a binary file

#       +       item is a redundant server

#       T       item points to a text-based TN3270 session

#       g       item is a GIF format graphics file

#       I       item is some kind of image file

#

#       All references are described in RFC1436

#

The master list contains file and list references which denote a

tree structure of accessible resources.  A typical  master  list

is shown here to clarify the specification.

The top level follows:

PROGRAM OPTIONS                                        PAGE 4-39

RGOPH

0       About This GOPHER Site

sy2:/gopher.dsk/rgoph.doc

shop-pdp.kent.edu

70

1       TCP/IP Package For TSX-Plus, ...

List1/

shop-pdp.kent.edu

70

1       ASxxxx Assemblers and Linker

List2/

shop-pdp.kent.edu

70

.

The  'List1/'  (<code>  =  1)  specification indicates this is a

directory of subitems:

PROGRAM OPTIONS                                        PAGE 4-40

RGOPH

#

# Elements of List1/

#

List1/

0       Abstract [2K]

sy3:/tcpip.pkg/abstra.doc

shop-pdp.kent.edu

70

0       Documentation [158K]

sy3:/tcpip.pkg/manual.doc

shop-pdp.kent.edu

70

9       Minimal  Package, Logical Disk Image, [300K]

sy3:/tcpipm.dsk

shop-pdp.kent.edu

70

9       Complete Package, Multilevel Logical ...

sy3:/tcpip.pkg/tcpip.dsk

shop-pdp.kent.edu

70

9       Disk1 of Package, RX02 Image [494K]

sy3:/tcpip.pkg/tcpip.dsk/disk1.dsk

shop-pdp.kent.edu

70

9       Disk2 of Package, RXO2 Image [494k]

sy3:/tcpip.pkg/tcpip.dsk/disk2.dsk

shop-pdp.kent.edu

70

9       Disk3 of Package, RXO2 Image [494k]

sy3:/tcpip.pkg/tcpip.dsk/disk3.dsk

shop-pdp.kent.edu

70

9       Disk4 of Package, RXO2 Image [494k]

sy3:/tcpip.pkg/tcpip.dsk/disk4.dsk

shop-pdp.kent.edu

70

9       Disk5 of Package, RXO2 Image [494k]

sy3:/tcpip.pkg/tcpip.dsk/disk5.dsk

shop-pdp.kent.edu

70

9       Disk6 of Package, RXO2 Image [494k]

sy3:/tcpip.pkg/tcpip.dsk/disk6.dsk

shop-pdp.kent.edu

70

9       Disk7 of Package, RXO2 Image [494k]

sy3:/tcpip.pkg/tcpip.dsk/disk7.dsk

shop-pdp.kent.edu

70

PROGRAM OPTIONS                                        PAGE 4-41

RGOPH

9       Documentation Source, RX02 Image [494K]

sy3:/tcpip.pkg/tcpip.dsk/tcpdoc.dsk

shop-pdp.kent.edu

70

9       Current Updates, Logical Disk Image [988K]

sy3:update.dsk

shop-pdp.kent.edu

70

.

The  'List2/'  (<code> = 1) specification indicates the presence

of a second directory of available resources:

#

# Elements of List2/

#

List2/

1       ASxxxx for PDP-11 and LSI-11 Computers

List2.1/

shop-pdp.kent.edu

70

1       ASxxxx for PC Compatible Computers

List2.2/

shop-pdp.kent.edu

70

.

These  elements  also contain references to additional subdirec-

tories of resources.  Nesting to any level may  be  accomplished

using this syntax.  Ensure that all subdirectory references have

unique names.  The use of 'xxxxx/' is not required,  any  unique

character string may be used to reference a subdirectory entry.

The  program  may  be started interactively (for diagnostic pur-

poses) by the command file RGOPH.COM:

run/single TCP:rgoph

-rsp 0 4 TCP:rgoph.cfg ^1 ^2 ^3 ^4 ^5 ^6

The program is normally invoked by the TCPIP network server pro-

gram by the command file RGOPH.SRV:

PROGRAM OPTIONS                                        PAGE 4-42

RGOPH

deassign

dismount ld0:

dismount ld1:

dismount ld2:

dismount ld3:

dismount ld4:

dismount ld5:

dismount ld6:

dismount ld7:

assign sy: wf:

mount ld6: dl1:paswrd.dsk

assign ld6: PAS:

mount ld7: dl1:tsxtcp.dsk

assign ld7: TCP:

run/single TCP:rgoph

-aspz 0 4 TCP:rgoph.cfg 600

4.23  UTIL

The program UTIL is a compendium of various functions used to

perform useful operations on TSX-Plus subdirectories.  The  com-

mand options for UTIL are:

UTIL [?] [-cdlmnpqru] [file-spec(s)]

?       List this Help Text and Exit UTIL

c       change directory

d       delete a file

l       list a directory

m       make a directory

n       delete a directory

p       set protection flag on a file

q       disable query on delete

r       rename a file

u       clr protection flag on afile

Each of the options is invoked by a command file more indicative

of its intended function.

PROGRAM OPTIONS                                        PAGE 4-43

UTIL

4.23.1  LCD

Change directory command file LCD.COM:

run/single TCP:util

-sc 0 2 ^1

@wf:cmdfil

4.23.2  LDEL

Delete file command file LDEL.COM:

run/single TCP:util

-d ^1

4.23.3  LDIR

List directory command file LDIR.COM:

run/single TCP:util

-l ^1

4.23.4  LLS

List directory command file LLS.COM:

run/single TCP:util

-l ^1

4.23.5  LMKDIR

Make a subdirectory command file LMKDIR.COM:

run/single TCP:util

-m ^1

PROGRAM OPTIONS                                        PAGE 4-44

UTIL

4.23.6  LPRO

Set protection flag command file LPRO.COM:

run/single TCP:util

-p ^1

4.23.7  LRENAM

Rename a file command file LRENAM.COM:

run/single TCP:util

-r ^1 ^2

4.23.8  LRMDIR

Remove a subdirectory command file LRMDIR.COM:

run/single TCP:util

-n ^1

4.23.9  LUNP

Clear protection flag command file LUNP.COM:

run/single TCP:util

-u ^1

4.24  LDISKS

The  LDISKS  program  generates  a  subdirectory map for each

device specified and optionally outputs the result to a file.

LDISKS [?] [-f filename] [dev1] [dev2] ...

?               List this Help Text and Exit LDISKS

f  filename     Output File Name

The command file LDISKS.COM allows arbitrary input parameters:

run TCP:ldisks

^1 ^2 ^3 ^4 ^5 ^6

PROGRAM OPTIONS                                        PAGE 4-45

LDISKS

An example of its use is:

LDISKS -f disk.map du2:

The output file disk.map would look like:

Logical Subdirectories for du2:

du2:

`--> maclib[ 2000]

`--> pictur[ 6000]

`-->    phd[14000]

`--> paswrd[  500]

`-->   mail[ 1000]

`--> baldwi[  500]

`-->  guest[  200]

`--> anonym[  200]

4.25  ABRTCP

ABRTCP  is  a  program which attempts to gracefully abort the

TCPIP  system.   All  TCP/IP   connections   are   closed,   all

client(s)/server(s) are requested to abort, and the TCPIP server

closes.   The  program  is  invoked  from   the   command   file

ABRTCP.COM:  as:

run/single TCP:abrtcp

4.26  ABRTJB

ABRTJB  is  a  program  which  attempts to gracefully abort a

specific TCP/IP client/server program.  The program  is  invoked

from the command file ABRTJB.COM:

run/single TCP:abrtjb

^1 ^2 ^3 ^4 ^5 ^6

and may request from 1 to 6 jobs be aborted.  The job aborts are

sent to the TCPIP server program which closes any  open  connec-

tions  associated with the job and sends an abort request to the

specified job.  This program will not  abort  the  TCPIP  server

program.

CHAPTER 5

ETHERNET DEVICE HANDLERS

The   device  handlers  provide  support  for  the  following

hardware controllers:

1.  DEQNA/DELQA (EQ and NQ Drivers)

2.  DEUNA/DELUA (EU and NU Drivers)

3.  DECNA (NC Driver)

The  Ethernet handlers provide support for one controller and

a maximum of 8 units numbered 0 through 7.  These  unit  numbers

are  used  as a logical connection between a user program and an

address/protocol pair to be recognized by the Ethernet hardware.

The NC/NQ/NU device handlers are the DEC handlers modified to

include special functions 206 (enable/disable frame bufferring),

207  (get  physical address) and 210 (get handler status block).

These handlers support only RT11XM V5.6 (and later), NU is coded

to use the UB.SYS pseudo handler for UNIBUS systems with greater

than 256K bytes.  The NC/NQ/NU handlers  perform  all  interrupt

processing  at  IRQ  level (FORK level is never entered), and do

not internally support device timeout.

The  EQ/EU  device  handlers  are logically equivalent to the

NQ/NU device handlers but were designed with  special  functions

206  (enable/disable  frame  bufferring),  207 (get physical ad-

dress), 210 (get handler status block), and 211 (abort job  i/o,

for  SJ monitors only).  These handlers may be conditionally as-

sembled to support the RT11 SJ, SB, FB, and  XM  monitors  (ver-

sions 5.3 and later) not supported by the NQ/NU device handlers.

The  EQ  and  EU  handlers may be conditionally assembled for

Version 6 of the  TSX-Plus  operating  system.   The  22-bit  EU

handler  requires  a  change in the TSX sysgen file TSGEN.MAC to

allocate 4 UMR registers to the EU handler.   TSX-Plus  versions

6.0  through  6.2  require a unique version of the EQ/EU handler

ETHERNET DEVICE HANDLERS                                Page 5-2

with special functions 300 (get PLAS region size), 301 (initial-

ize handler with PLAS region base address), and the support pro-

gram ENPLAS.SAV.  The program ENPLAS.SAV creates the PLAS memory

region required by the handler.

ETHERNET DEVICE HANDLERS                                Page 5-3

5.1  DEVICE HANDLER SYSTEM SUMMARY

Summary of prebuilt EQ/EU/NC/NQ/NU

device drivers and supported systems:

RT-11 VERSION   [5.3]   [5.4]   [5.5]      [5.6 ...]    TSX

-----   -----   -----      ---------    ---

eqsjms.sys      SJ/FB   SJ/FB   SJ/FB          -         -

eusjms.sys      SJ/FB   SJ/FB   SJ/FB          -         -

eqsjps.sys      SJ/FB   SJ/FB   SJ/FB          -         -

eusjps.sys      SJ/FB   SJ/FB   SJ/FB          -         -

eqsbms.sys        -       -       -          SB/FB       -

eusbms.sys        -       -       -          SB/FB       -

eqsbps.sys        -       -       -          SB/FB       -

eusbps.sys        -       -       -          SB/FB       -

eq18xm.sys        XM      XM      XM      XB/XM/ZB/ZM    -

eq22xm.sys        XM      XM      XM      XB/XM/ZB/ZM    -

eu18xm.sys        XM      XM      XM      XB/XM/ZB/ZM    -

ncxv56.sys        -       ?       ?       XB/XM/ZB/ZM    -

nqxv56.sys        -       ?       ?       XB/XM/ZB/ZM    -

nuxv56.sys        -       ?       ?       XB/XM/ZB/ZM    -

eq18.v62          -       -       -            -        <= V6.2

eq22.v62          -       -       -            -        <= V6.2

eu18.v62          -       -       -            -        <= V6.2

eu22.v62          -       -       -            -        <= V6.2

eq18.tsx          -       -       -            -        >= V6.4

eq22.tsx          -       -       -            -        >= V6.4

eu18.tsx          -       -       -            -        >= V6.4

eu22.tsx          -       -       -            -        >= V6.4

____ms.sys      Drivers are for LSI/11 processors not having

a processor status word (PSW == @#177776).

____ps.sys      Drivers are for all processors with a

processor status word (PSW == @#177776).

__sj__.sys      Drivers contain special function 211 (abort

job i/o) for RT11SJ monitors.

ETHERNET DEVICE HANDLERS                                PAGE 5-4

DEVICE HANDLER SYSTEM SUMMARY

5.2  DEVICE HANDLER FEATURE SUMMARY

Summary of prebuilt EQ/EU/NC/NQ/NU

device driver features:

SPFUNS                               DEVICE

206  207  210  211    EIS  PSW  UMR  FORK  TIMEOUT

---  ---  ---  ---    ---  ---  ---  ----  -------

eqsjms.sys    X    -    -    X      -    -    -     X      X

eusjms.sys    X    -    -    X      -    -    -     X      X

eqsjps.sys    X    -    -    X      -    X    -     X      X

eusjps.sys    X    -    -    X      -    X    -     X      X

eqsbms.sys    X    -    -    -      -    -    -     X      X

eusbms.sys    X    -    -    -      -    -    -     X      X

eqsbps.sys    X    -    -    -      -    X    -     X      X

eusbps.sys    X    -    -    -      -    X    -     X      X

eq18xm.sys    X    X    X    -      X    X    -     X      X

eq22xm.sys    X    X    X    -      X    X    -     X      X

eu18xm.sys    X    X    X    -      X    X    -     X      X

ncxv56.sys    X    X    X    -      X    X    -     -      -

nqxv56.sys    X    X    X    -      X    X    -     -      -

nuxv56.sys    X    X    X    -      X    X    X     -      -

eq18.v62      X    X    X    -      X    X    -     X      X

eq22.v62      X    X    X    -      X    X    -     X      X

eu18.v62      X    X    X    -      X    X    -     X      X

eu22.v62      X    X    X    -      X    X    X     X      X

eq18.tsx      X    X    X    -      X    X    -     X      X

eq22.tsx      X    X    X    -      X    X    -     X      X

eu18.tsx      X    X    X    -      X    X    -     X      X

eu22.tsx      X    X    X    -      X    X    X     X      X

____ms.sys      Drivers are for LSI/11 processors not having

a processor status word (PSW == @#177776).

____ps.sys      Drivers are for all processors with a

processor status word (PSW == @#177776).

__sj__.sys      Drivers contain special function 211 (abort

job i/o) for RT11SJ monitors.

ETHERNET DEVICE HANDLERS                                PAGE 5-5

DEVICE HANDLER FEATURE SUMMARY

5.3  PROGRAMMING

Programs that call the Ethernet handler must follow a certain

order.  Write such programs with the following elements  in  the

order indicated.

1.  Use the .LOOKUP programmed request to open a channel to

the device unit.

2.  Allocate the unit using SPFUN 200.

3.  Perform the Ethernet operation(s).

4.  Deallocate the unit using SPFUN 200.

5.  Use  the .CLOSE programmed request to close the channel

to the specified device unit.

NOTE

Special functions 207 (get physical address), 210 (get

handler status block), and 211 (abort job i/o) do  not

require device allocation.

5.4  USE OF SPECIAL FUNCTIONS (.SPFUN)

The  EQ/EU/NC/NQ/NU  Ethernet  handlers  operate the Ethernet

controllers using the .SPFUN programmed request with the follow-

ing function codes:

ETHERNET DEVICE HANDLERS                                PAGE 5-6

USE OF SPECIAL FUNCTIONS (.SPFUN)

Function                        Code

--------                        ----

Allocate unit                   200

Deallocate unit                 200

Enable Promiscuous Mode         201

Disable Promiscuous Mode        201

Enable protocol type            202

Disable protocol type           202

Enable multicast address        203

Disable multicast address       203

Transmit Ethernet frame         204

Receive Ethernet frame          205

Enable frame bufferring         206

Disable frame bufferring        206

Get physical address            207

Get handler status block        210

Abort Job I/O                   211

Successful  completion of a .SPFUN request clears the carry bit.

Completion with an error sets the carry bit, and the status word

in the buffer contains an error code.

5.4.1  Unit Specification (.SPFUN 200)

The  allocate  unit  special  function allocates a unit for a

job's exclusive use.

The deallocate unit special function deallocates a unit so it

can be used by another job.

ETHERNET DEVICE HANDLERS                                PAGE 5-7

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.4.1.1  Allocate Unit  -

The  following  is  the form of the special function allocate

unit (.SPFUN 200):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 200

buf     is the address of a 4-word buffer

containing the status word and space

for the station's physical address.

The buffer contents are returned by

the allocate unit special function.

The high byte of the status word contains a 0.

Allocate unit returns one of the following

octal status codes in the low byte of the

status word:

0  Success

2  Controller error while attempting to

initialize the network interface

3  No resources (unit in use)

11 Reserved

wcnt    is 0

blkn    is 1

crtn    is the entry point of a completion routine

ETHERNET DEVICE HANDLERS                                PAGE 5-8

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.4.1.2  Deallocate Unit  -

The  following is the form of the special function deallocate

unit (.SPFUN 200):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 200

buf     is the address of a 1-word buffer

containing the status word.

The high byte of the status word contains a 0.

Deallocate unit returns one of the following

octal status codes in the low byte of the

status word:

0  Success

1  Unknown unit. The specified unit was not

opened by the job issueing the request.

2  Controller error while attempting to

initialize the network interface

11 Unit still active

wcnt    is 0

blkn    is 0

crtn    is the entry point of a completion routine

ETHERNET DEVICE HANDLERS                                PAGE 5-9

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.4.2  Promiscuous Mode (.SPFUN 201)

The  enable  promiscuous  mode  special  function enables the

Ethernet hardware to receive all network traffic  regardless  of

address  or  protocol.  Any read request to the handler will re-

turn the first available packet received by the  controller  in-

terface.

The disable promiscuous mode special function enables the ad-

dress/protocol filtering provided by the Ethernet  hardware  and

enables the specified protocol.

5.4.2.1  Enable Promiscuous Mode  -

The  following  is  the  form  of the special function enable

promiscuous mode (.SPFUN 201):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 201

buf     is the address of a 1-word buffer

containing the status word.

The high byte of the status word contains a 0.

Enable promiscuous mode returns one of the

following octal status codes in the low byte

of the status word:

0  Success

6  address/protocol active

wcnt    is 0

blkn    is 1

crtn    is the entry point of a completion routine

ETHERNET DEVICE HANDLERS                               PAGE 5-10

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.4.2.2  Disable Promiscuous Mode  -

The  following  is  the  form of the special function disable

promiscuous  mode  (which  performs  an  enable  protocol  type)

(.SPFUN 201):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 201

buf     is the address of a 2-word buffer that

contains the status word, followed by

the 1-word protocol type that is to be

enabled when the promiscuous mode is

disabled.

The high byte of the status word contains a 0.

The disable promiscuous mode returns one of the

following octal status codes in the low byte

of the status word:

0  Success

1  Unknown unit. The specified unit was not

opened by the job issueing the request.

2  Controller error while attempting to

initialize the network interface

3  Reserved

6  Reserved

10 Protocol type in use

wcnt    is 0

blkn    is 0

crtn    is the entry point of a completion routine

ETHERNET DEVICE HANDLERS                               PAGE 5-11

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.4.3  Protocol Type (.SPFUN 202)

The  enable  protocol  type  special function adds a protocol

type to those to be recognized by the Ethernet controller.  Only

one  protocol type can be specified for each unit.  At least one

protocol type must be enabled to receive Ethernet frames.

The  disable  protocol type special function removes the pro-

tocol type from the list of those  recognized  by  the  Ethernet

controller.

5.4.3.1  Enable Protocol Type  -

The following is the form of the special function enable pro-

tocol type (.SPFUN 202):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 202

buf     is the address of a 2-word buffer

that contains the status word followed

by the protocol type word.

The high byte of the status word contains a 0.

Enable protocol type returns one of the

following octal status codes in the low byte

of the status word:

0  Success

1  Unknown unit. The specified unit was

not opened by the job issuing the

request

2  Controller error while attempting to

initialize the network interface

3  No resources (unit's protocol table full)

6  Reserved

ETHERNET DEVICE HANDLERS                               PAGE 5-12

USE OF SPECIAL FUNCTIONS (.SPFUN)

10 Protocol type in use

The protocol type is specified by the user

wcnt    is 0

blkn    is 1

crtn    is the entry point of a completion routine

5.4.3.2  Disable Protocol Type  -

The  following  is  the  form of the special function disable

protocol type (.SPFUN 202):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 202

buf     is the address of a 2-word buffer

that contains the status word followed

by the protocol type word.

The high byte of the status word contains a 0.

Disable protocol type returns one of the

following octal status codes in the low byte

of the status word:

0  Success

1  Unknown unit. The specified unit was

not opened by the job issuing the

request

2  Controller error while attempting to

initialize the network interface

If the disabled protocol type is not the

same as the enabled protocol type for this

unit then 'success' is returned.

wcnt    is 0

ETHERNET DEVICE HANDLERS                               PAGE 5-13

USE OF SPECIAL FUNCTIONS (.SPFUN)

blkn    is 0

crtn    is the entry point of a completion routine

5.4.4  Multicast Address (.SPFUN 203)

The enable multicast address special function adds the multi-

cast address to those recognized  by  the  Ethernet  controller.

Ypu  need  not specify the unit's physical or broadcast address.

Only one multicast address is allowed per handler unit.

The  disable  multicast  address special function removes the

multicast address from those to be recognized  by  the  Ethernet

controller.

5.4.4.1  Enable Multicast Address  -

The following is the form of the special function enable mul-

ticast address (.SPFUN 203):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 203

buf     is the address of a 4-word buffer

that contains the status word, followed

by the 3-word multicast address. The

low order bit of the first address

word should be a 1.

The high byte of the status word contains a 0.

Enable multicast address returns one of the

following octal status codes in the low byte

of the status word:

0  Success

1  Unknown unit. The specified unit was not

opened by the job issueing the request.

2  Controller error while attempting to

ETHERNET DEVICE HANDLERS                               PAGE 5-14

USE OF SPECIAL FUNCTIONS (.SPFUN)

initialize the network interface

3  No resources (unit's address table is full,

or hardware address table full)

wcnt    is 0

blkn    is 1

crtn    is the entry point of a completion routine

5.4.4.2  Disable Multicast Address  -

The  following  is  the  form of the special function disable

multicast address (.SPFUN 203):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 203

buf     is the address of a 4-word buffer

that contains the status word, followed

by the 3-word multicast address. The

low order bit of the first address

word should be a 1.

The high byte of the status word contains a 0.

Disable multicast address returns one of the

following octal status codes in the low byte

of the status word:

0  Success

1  Unknown unit. The specified unit was not

opened by the job issueing the request.

2  Controller error while attempting to

initialize the network interface

If the disabled multicast address is not the

same as the enabled multicast address for this

unit then 'success' is returned.

ETHERNET DEVICE HANDLERS                               PAGE 5-15

USE OF SPECIAL FUNCTIONS (.SPFUN)

wcnt    is 0

blkn    is 0

crtn    is the entry point of a completion routine

5.4.5  Transmit Ethernet Frame (.SPFUN 204)

The  special  function  transmit Ethernet frame transmits the

Ethernet frame pointed to in the buf argument.   If  the  source

field  of  the  frame  is not zero, it is kept and used.  If the

source field of the frame is zero, the controllers physical  ad-

dress is inserted in the source field before transmission.

The  following  is  the form of the special function transmit

Ethernet frame (.SPFUN 204):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 204

buf     is the address of a variable size buffer

containing the status word, followed by

the Ethernet packet to be transmitted.

The first word following the status word

is reserved. Next is the 3-word destination

address field followed by the 3-word

source address field. The next word is the

packet protocol type. Following this is the

variable length packet containing from 23 to

750 words.

The transmit Ethernet frame returns one of the

following octal status codes in the low byte

of the status word:

0  Success

1  Unknown unit. The specified unit was not

opened by the job issueing the request.

2  Controller error while attempting to

ETHERNET DEVICE HANDLERS                               PAGE 5-16

USE OF SPECIAL FUNCTIONS (.SPFUN)

initialize the network interface

13 Transmit failed

When status code 13 is returned in the low byte

of the status word, transmit Ethernet frame

returns one of the following octal status codes

in the high byte of the status word:

1  Invalid frame length

2  Excessive collisions

3  Carrier check failed

wcnt    is determined by the variable size of the user

buffer (including the status word and reserved

word). The packet size (including the status

word and reserved word) can vary between 32

and 759 words.

blkn    is 0

crtn    is the entry point of a completion routine

5.4.6  Receive Ethernet Frame (.SPFUN 205)

The  receive Ethernet frame special function returns the next

Ethernet packet with the desired unit address and protocol  type

to  the  buffer.   The  function does not return Ethernet frames

that are received with errors.

The  following  is  the  form of the special function receive

Ethernet frame (.SPFUN 205):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 205

buf     is the address of a variable size buffer

containing the status word, a word

containing the frame size, and followed by

ETHERNET DEVICE HANDLERS                               PAGE 5-17

USE OF SPECIAL FUNCTIONS (.SPFUN)

upto 757 words to receive the Ethernet

frame. The buffer contents are returned

by the receive Ethernet frame function.

The high byte of the status word contains a 0.

The receive Ethernet frame special function

returns one of the following octal status

codes in the low byte of the status word:

0  Success

1  Unknown unit. The specified unit was not

opened by the job issueing the request.

2  Controller error while attempting to

initialize the network interface

wcnt    is the size of the user buffer including

the status and frame size words

blkn    is 0

crtn    is the entry point of a completion routine

5.4.7  Frame Queueing (.SPFUN 206)

The   Enable  frame  queueing  special  function  allows  the

handlers to internally queue received  packets  that  match  any

enabled  address/protocol  even though a receive request has not

been issued for that address/protocol or a previous  packet  has

not been processed.  The EQ handler normally allocates space for

internally queueing upto 20 packets (this may be changed by mod-

ifying  'rbcnt'  in  EQ.MAC and rebuilding the EQ handler).  The

22-bit EU handler normally allocates space for internally queue-

ing  10  packets and the 18-bit EU handler allocates space for 2

packets (this may be changed by modifying 'rbcnt' in EU.MAC  and

rebuilding the EU handlers).

The  disable frame queueing special function disables the in-

ternal queueing of packets.  This function may only be issued if

all address/protocol pairs have been disabled.

ETHERNET DEVICE HANDLERS                               PAGE 5-18

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.4.7.1  Enable Frame Queueing  -

The  following  is  the  form  of the special function enable

frame queueing (.SPFUN 206):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 206

buf     is the address of a 1-word buffer

for the returned status word which

is always 0

wcnt    is 0

blkn    is 1

crtn    is the entry point of a completion routine

5.4.7.2  Disable Frame Queueing  -

The  following  is  the  form of the special function disable

frame queueing (.SPFUN 206):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 206

buf     is the address of a 1-word buffer

containing the status word.

The high byte of the status word contains a 0.

Disable frame queueing returns one of the

following octal status codes in the low byte

of the status word:

ETHERNET DEVICE HANDLERS                               PAGE 5-19

USE OF SPECIAL FUNCTIONS (.SPFUN)

0  Success

6  address/protocol active

wcnt    is 0

blkn    is 0

crtn    is the entry point of a completion routine

5.4.8  Get Ethernet Address (.SPFUN 207)

The  get Ethernet address special function returns the physi-

cal address of the Ethernet controller card.

The  following  is  the  form  of  the  special  function get

Ethernet address (.SPFUN 207):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 207

buf     is the address of a 4-word buffer

in which the status word, which is

always 0, and the six byte physical

Ethernet address are returned.

wcnt    is 0

blkn    is 0

crtn    is the entry point of a completion routine

ETHERNET DEVICE HANDLERS                               PAGE 5-20

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.4.9  Get Ethernet Status Block (.SPFUN 210)

The  get  Ethernet  status block special function returns the

handler's internal parameter block.  The  information  contained

in  the  parameter block includes the number of packets received

and transmitted for each EQ/EU unit as well as timeout  informa-

tion.

The  following  is  the  form  of  the  special  function get

Ethernet status block (.SPFUN 210):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 210

buf     is the address of a buffer into which

wcnt words are written from the

EQ/EU handlers status block.

wcnt    is the number of words to return

to the buffer.  The status word

must be included in the word count.

blkn    is 0

crtn    is the entry point of a completion routine

The complete list of parameters in the status block may be found

in the EQ/EU Ethernet handler files EQ.MAC, EU.MAC, and  EI.MAC.

The first word is always a status word of value 0, the following

4-byte (long integer) parameters are:

eiipkt          total receiver interrupts

eiopkt          total transmitter interrupts

eirtmo          total receive timeouts

eixtmo          total transmit timeouts

eirun[8]        received packets for each unit

eixun[8]        transmitted packets for each unit

These  are  the  only  parameters  used by any program in the

'TCP/IP Package for TSX-Plus' or 'TCP/IP Package for RT-11'.

ETHERNET DEVICE HANDLERS                               PAGE 5-21

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.4.10  Abort Job I/O (.SPFUN 211)

The abort job i/o special function is conditionally assembled

into drivers for the RT11SJ  monitors.   The  SJ  monitor  never

calls the abort entry point in the device handler (either from a

.ABTIO request or at job termination) this means that the inter-

nally  queued  i/o  requests will never complete and the monitor

will lock waiting for the i/o to complete.  This  special  func-

tion  calls the abort entry point clearing all internally queued

elements associated with the current job.

The  following  is the form of the special function Abort Job

I/O (.SPFUN 211):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 211

buf     is not used

wcnt    is not used

blkn    is not used

crtn    is the entry point of a completion routine

5.5  HANDLERS FOR TSX-PLUS V6.0 THROUGH V6.2

The  EQ and EU Ethernet handlers configured for TSX-Plus ver-

sions 6.0 through 6.2 have these  additional  .SPFUN  programmed

requests:

Function                        Code

--------                        ----

Get PLAS Allocation Size        300

Initialize Handler              301

Successful  completion of a .SPFUN request clears the carry bit.

Completion with an error sets the carry bit  without  any  error

ETHERNET DEVICE HANDLERS                               PAGE 5-22

HANDLERS FOR TSX-PLUS V6.0 THROUGH V6.2

code.

5.5.1  Get PLAS Allocation Size (.SPFUN 300)

The  following  is  the form of the special function Get PLAS

Alocation Size (.SPFUN 300):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 300

buf     handler returns the requested allocation

size, in 32. word increments, in the

first word of the buffer

wcnt    is not used

blkn    is not used

crtn    is the entry point of a completion routine

5.5.2  Initialize Handler (.SPFUN 301)

The  following is the form of the special function Initialize

Handler (.SPFUN 301):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 301

buf     the address of buf is the PLAS base address

ETHERNET DEVICE HANDLERS                               PAGE 5-23

HANDLERS FOR TSX-PLUS V6.0 THROUGH V6.2

wcnt    is not used

blkn    is not used

crtn    is the entry point of a completion routine

Attempting  to  reinitialize the handler or to initialize the

handler with a PLAS base address of 0 will be reported as an er-

ror by setting the carry bit.

5.5.3  ENPLAS.SAV - Allocating the PLAS Region

The program ENPLAS.SAV is a support program which creates the

PLAS buffer required by the TSX-Plus V6.0 through V6.2 EQ and EU

device  handlers.  The program uses .spfun 300 to get the buffer

size, creates the PLAS region, uses .spfun 301  to  handoff  the

PLAS region's Base Address to the device handler and to initial-

ize the handler.  This program must run  before  the  EQ  or  EU

device handler can be used.

The device EN0:  must be assigned before running ENPLAS.SAV.

ASSIGN EU0: EN0:        or

ASSIGN EQ0: EN0:

R ENPLAS.SAV

The  device  handler requires only a single initialization to

be used by the TCPIP.SAV server or the small system client  pro-

grams.

CHAPTER 6

BUILDING THE TCP/IP PACKAGE

6.1  C COMPILER

The DECUS C Compiler and Assembler and the RT-11 Macro Assem-

bler and Linker are required to build the the TCP/IP Package for

TSX-Plus  from  the  source  files included in the distribution.

The DECUS C Compiler is from  the  DECUS  11-SP-18  distribution

with  bug  fixes made to the compiler and the library.  (The up-

dates are on logical disk CCDSK2.DSK).  EIS and non-EIS versions

of  the  compiler  along  with  the  assembler, library, and the

necesary header files are included with the distribution to  al-

low  the  package to be recompiled.  The CC.SAV files were modi-

fied by the TSX-Plus SETSIZ  program  to  allow  it  to  use  an

extended program space of 64Kb.

1.  Create CDISK.DSK[1500] as C:,

a logical disk for the C compiler

2.  Copy the contents of the logical disk CCDSK1.DSK to C:.

The  device  containing  the  compiler,  assembler  and

header  files  should  be  assigned  as device C:.  The

build procedures assume that the  compiler  and  header

files are on the logical device 'C:'.

3.  Copy  the  compiler  helper  files  from  logical  disk

CCXDSK.DSK to C:.

4.  Use  the  command  file  C:CCSET.EIS  to  copy  the EIS

specific  files  into  the  C:   directory   with   the

appropriate .SAV and .OBJ file types.

5.  Copy the command files C:CC.TSX and C:ASM.TSX as CC.COM

and ASM.COM to the system device.  These TSX-Plus  com-

mand files are used to invoke the C compiler and AS as-

sembler.

BUILDING THE TCP/IP PACKAGE                             PAGE 6-2

TCP/IP PACKAGE

6.2  TCP/IP PACKAGE

The  TCP/IP Package is supplied with the complete source code

as an assemblage of C files (*.c), macro files  (*.mac),  header

files (*.h), and an assortment of command files (*.com).

1.  Copy   the   logical   disks   SRDSK1.DSK,  SRDSK2.DSK,

SRDSK3.DSK, and SRDSK4.DSK to a disk  or  logical  disk

having at least 10000 blocks of free space.

2.  Use  the command file BLDTSX.SET to create the TSX-Plus

conditional files.

3.  The  command  file BLDALL.COM will compile and link the

complete TCP/IP Package.  Inspect BLDALL.COM to see how

each  individual component is built.  An unloaded 11/73

processor will take from 40 to 50  minutes  to  compile

and link the complete package.

4.  Rename all .SAV files to .STV files.

5.  The  file  TSX.INS  may  be  used to copy all the files

necessary to configure a complete implementation of the

TCP/IP  Package  for  TSX-Plus  to the TCP:  directory.

(This includes all the files  found  on  logical  disks

TSDSK1.DSK, TSDSK2.DSK, and TSDSK3.DSK).

6.3  COMPILING OPTIONS

The TCP/IP Package components may be compiled with or without

the diagnostic options (the default is  no  diagnostics).   Each

component  has  an  individual  header  file, e.g.  VTCPIP.H for

TCPIP.SAV, which contains define statements that have been  com-

mented  out  to exclude the diagnostics.  Several other programs

have optional sections that may be disabled  by  commenting  out

selected define statements.

CHAPTER 7

DISTRIBUTION DISKS

The  "TCP/IP  Package  for TSX-Plus", the "TCP/IP Package for

RT-11", and the "FTP and Telnet Package for Small  Systems"  are

distributed  as  a  combined  package  of 21 RX02 logical floppy

disks:

1.  TSDSK1    TCP/IP Package for TSX-Plus

2.  TSDSK2      Executables for TSX-Plus Systems.

3.  TSDSK3      (EQ/EU Device Driver Required)

4.  RTDSK1    TCP/IP Package for RT-11

5.  RTDSK2      Executables for RT-11 Mapped Systems.

6.  RTDSK3      (EQ/EU/NC/NQ/NU Device Driver Required)

7.  SRDSK1    TCP/IP Package Sources

8.  SRDSK2      All source code files for:

9.  SRDSK3            TCP/IP Package for TSX-Plus

10.  SRDSK4            TCP/IP Package for RT-11

11.  SMLTCP    Small System Package Executables

(EQ/EU/NC/NQ/NU Device Driver Required)

12.  SSDSK1    Small System Package Sources

13.  DVRDSK    EQ/EU/NC/NQ/NU Device Drivers

14.  CCDSK1    DECUS C Compiler, Assembler, and Libraries.

15.  CCDSK2    DECUS C Source Changes

16.  CCXDSK    Compiler helper files for mapped

RT-11 systems.

17.  DOCDSK    Package Documentation

18.  RNODSK    Package Documentation in RNO Format

19.  RTHTML    RT-11 Docs in HTML Format

20.  TSHTML    TSX-Plus Docs in HTML Format

DISTRIBUTION DISKS                                      Page 7-2

21.  SSHTML    Small System Docs in HTML Format

APPENDIX A

HIDDEN OPTIONS

A.1  DISABLE PASSWORD

The  -b  option  will disable the user/password query in pro-

grams TELNET, RTELNT, FTP, RFTP, CNCT, RCNCT, MAIL,  and  MMAIL.

Local  users normally are not required to use passwords when ac-

tivating TELNET, FTP, or CNCT.  The MAIL and MMAIL programs  and

the  network  server programs RTELNT, RFTP, and RCNCT should not

specify the -b option for security reasons.

A.2  SUBDIRECTORY UNIT SELECTION

The  default  LD units used by the client/server/utility pro-

grams  for  subdirectory  access  are  LD2:   and  LD3:.   These

defaults may be changed with the hidden -s option:

-s arg1 arg2

where arg1 is the first LD unit number to use and arg2 is the

maximum number of nested subdirectories.  The value of arg1+arg2

must  be  less than or equal to 8, the total number of LD units.

The default is arg1=2 and arg2=2.

###############################################################################################################