This applies to the ES-300C, too.

=======

Quick Note: Set SW2-8 to OFF for parallel scanning.

If you have problems setting up the scanner for parallel or SCSI

(just a HINT, not verified!):

My settings are (1=ON, 0=OFF):

SW1: 1 1 1 1 0 0 0 0

SW2: 1 0 0 0 0 1 1 0

Using the parallel Port.

SCSI interface currently not installed.

The switches according to the manual:

(; how archaic: typed in instead of a scanned picture ;)

1 2 3 4 5 6 7 8

SW1: b b b s p p l l	b=Baud rate   s=stop bit  p=parity   l=line printer

1 0 0 1 0 0 0 0	Factory default

0 0 0		19200 bps

1 0 0		 9600 bps (factory)

0 1 0		 4800 bps

1 1 0		 2400 bps

0 0 1		 1200 bps

1 0 1		  600 bps

0 1 1		  300 bps

1 1 1		    -

0		1 stopbit

1		2 stopbits (factory)

x 0	parity disabled (factory)

0 1	parity odd

1 1	parity even

0 0	EPSON 24 bit color printer (80 col.) (factory)

1 0	HP PaintJet color printer (parallel)

0 1	undefined

1 1	EPSON 24 bit color printer (136 col.) (factory)

1 2 3 4 5 6 7 8

SW2: h h g g g c c i	h=halftoning   g=gamma   c=color   i=par. interface

1 0 0 0 0 1 1 0	Factory default

0 0		disabled

1 0		halftoning mode A (factory)

0 1		halftoning mode B

1 1		halftoning mode C

0 0 0		CRT Display A (factory)

1 0 0		CRT Display B

0 1 0		Printer Output A

1 1 0		Printer Output B

0 0 1		Printer Output C

1 x 1		undefined

x 1 1		undefined

0 0	Impact dot matrix printer

1 0	Thermal transfer printer

0 1	Ink jet printer

1 1	CRT display (factory)

0	Bi-Directional parallel interface (factory)

1	Unidirectional parallel interface

Only set SW2-8 to ON if the parallel printing

function is used.